“Talon Talisman”

Published as a Reference by the 43rd FTS.*

T-38 Dash-1 Pocket Reference

AIRSPEEDS

½	ADD HALF THE GUST

1	PER 100LBS >1000LBS

5	WINGMAN UPWIND

10	NOSEWHEEL SCRUBS W/FULL DEFLECTION

10	STANDING H20 T/O or LAND

15	MAX X-WIND FORM LAND

15/10Wet	MAX X-WIND FOR SOLO STUDENT

15	LAND UPWIND ON RWY; X-WIND

+15	ADD TO NORMAL FOR NO-FLAP

20	WET RWY X-WIND LIMIT

25	T&G X-WIND LIMIT

30	MAX DRY X-WIND

35	MAX SURFACE WIND (Bailout)

50	CANOPY LIMIT

60-120	MA-1A ENGAGE ENVELOPE

65	NWS LIMIT

100	FIRST AIRSPEED MARKING

100	GO FOR BLOWN TIRE

<100	OPTIMUM BRAKING

100-120	USE CAUTION WHEN BRAKING

130+GAS	TOUCHDOWN

- ROTATE

150	MIN FORM. MANEUVERING SPEED

155+GAS	FINAL

- LIFTOFF

174	RATED NOSE-TIRE GROUND SPEED

175+GAS	FINAL TURN

>180	FLAP ASSYMETRY

190	RETRACT FLAPS; S/E-GO

190	BEST S/E CLIMB, GEAR & 60 FLAPS

210±30	GEAR WARNING SWITCH SPEEDS

220	>60% FLAPS LIMIT SPEED

220	BEST S/E CLIMB, GEAR & NO-FLAP

220	RATED MAIN-TIRE GROUND SPEED

225	MIN DRAG, NO CANOPY

230+GAS	L/D MAX

240	GEAR and DOORS

240	46-60% FLAPS LIMIT SPEED

240	MAX FULL-DEFL. AILERON, WSSP

250-265	HOLDING

280	TURB AIR PENETRATION

300	1-45% FLAPS LIMIT SPEED

300	RESTRICTED CLIMB <10,000

- PATTERN SPEED

300	MIN LOW-LEVEL SPEED

350	TRAVEL POD MAX, BOARDS/TURB

360	PLANNED LOW-LEVEL SPEED

400	TRAVEL POD MAX, SMOOTH AIR

420	MAX LOW-LEVEL SPEED

500	EJECTION INJURIES LIKELY

500+	EGT DROOP @ LOW ALTITUDE

0.9M	MINIMUM CONTROL FORCES

0.9M	MIN CRUISE MACH > FL360

800/1.4M	HIGHEST A/S IND MARKING

FUEL FLOW

400-600	APPROX IDLE

1000 pph	APPROX CRUISE (each)

<360pph	MAX BEFORE LIGHTOFF

7300pph   	FULL AB (each); STATIC

11,400pph  	FULL AB (each) @ MACH 1.0

500 lbs	WILL TAKE YOU 100NM (hi alt)

WEIGHTS

<140lb 	SOME JETS SOLO RESTRICTED

140lb	TRAVEL POD CAPACITY

140-210	NORMAL CREW WT

12,500	FULLY-FUELED T-38A

PROHIBITED MANEUVERS

-VERTICAL STALLS

-INTENTIONAL SPINS

-CONT. AILERON ROLLS>360

w/MORE THAN ¾ DEFLECTION

OIL LIMITS

ZERO “g”		10 SECONDS

NEGATIVE “g”	30 SECONDS

>55 PSI		6 MINUTES (GROUND)

1 PINT/HR		MAX CONSUMPTION

10 secs		RECOVERY TIME

10 PSI		MAX JUMP

6 MINUTES	COLD WX > 55PSI

RPMs

12%	START BUTTON

12	MIN AIRSTART (200KIAS)

14	THROTTLE OVER HUMP

18-20	HEART OF AIRSTART (280KIAS)

24	AIRSTART MAX (310KIAS)

43-48	GEN COMES ON-LINE

46.5-49.5	GROUND IDLE

65-70	GENERATOR SHIFT RANGE

80	MIN FOR EFF. ANTI-ICE

83-98.5	CONTINUOUS RANGE

<96	GEAR WARNING

<97	NOZZLE POSITION  HYDROMECH.

99-100.5	MIL or MAX—GROUND

99-104	MIL or MAX--FLIGHT

107	MAX TRANSIENT (accel)

# EGT

15C	EGT RISE w/ANTI-ICE

140	MIN IDLE (flight)

140-630	CONTINUOUS RANGE

630-645	MIL/MAX RANGE

845	ABORT START

925	MAX TRANSIENT

TIME

* This publication is NOT a substitute for an in-depth knowledge of the Dash-1, 11-201, 11-202V3, 11-2T-AT-38, and 11-238.  It is intended as a study-aid or reference when these pubs are not available.  Cited references are accurate as of date of publication in the footer.

0.25 sec	CHUTE DELAY < BAROSTAT ALT

1 sec	DEFINITION OF THROTTLE-BURST

1 sec	FUEL SHUTOFF-VALVE

3.42 sec	EJECT HANDLE  FULL CHUTE

5 sec	AB LIGHT ON TAKEOFF

6 sec	AIR COND. CONTROL VALVE

6 sec	GEAR CYCLE TIME

7.5 sec	FUEL-LOW WARNING DELAY

12 sec	LIGHTOFF FROM FUEL FLOW (start)

15-35 sec	ALTERNATE GEAR EXTENSION

30 sec	IGNITION CYCLE

30 sec	FAST-SLAVE CYCLE

1 min	WINDMILL GR UP, SETOS+10190

2 mins	TIME BETWEEN FAST-SLAVE

2 mins	MOTOR AFTER WET-START

2 mins	WAKE SEPARATION: HELO/LARGE

3.5 mins	ADI/HSI ERECTION TIME

4 mins	WAKE SEPARATION: HEAVY

6 mins	COLD WX OIL > 55PSI

9 mins	STBY ADI GOOD > DC LOST; 6

35 mins	POSS. HYDRAULIC TRANSFER

FUELS

3982lb	TOTAL USABLE (JP-8, single-pt)

3921lb	TOTAL FUEL (JP-8, over-top)

2500lb	MAX FOR SOLO PATTERN (Student)

2500lb	MAX FOR NO-FLAP (practice)

800lb	NORMAL RECOVERY

600lb	MIN FUEL

400lb	EMERGENCY FUEL

25025lb	FUEL-LOW (after 7.5 seconds)

190lb	START, TAXI, T/O (typical)

47 (LEFT); 32 (RIGHT): FCP TOLERANCE

25	RCP TOLERANCE FROM FCP

SERVICING++

APPROVED FUELS:

|  | PRIMARY |  | ALTERNATE |  |
| --- | --- | --- | --- | --- |
| MILITARY | JP-8 | JP-4 | JP-5 |  |
| CIVILIAN | JET-A1 |  | JET-A | JET-B |

* CIVILIAN ALTERNATE FUELS MUST USE ”PRIST” ANTI-ICE/FUNGAL INHIBITOR

OIL:	MIL-7808

DO NOT MIX WITH OTHER TYPES!!!

1 PINT/HR MAX CONSUMPTION

SYSTEM CAPACITY: 8 PINTS

TIRES:	NOSE: 75 +5/-0 psi

MAIN: 250  5 psi

Note: Mx Manual says Mains 2655 psi

OXYGEN:	MIL O-27210

50-120 PSI

10 LITERS MAX CAPACITY

1 LTR/HR NORMAL CONSUMPTION

HYDRAULICS:  MIL 5606 or 83282

1.25 GAL RESERVOIR CAPACITY

BRAKES:	0.2 GAL CAPACITY

++ SERVICING DATA FROM TO 1T-38A-2-1,

### 1T-38A-2-2, AND 1T-38A-6WC-1

START

12 SEC	FUEL FLOW UNTIL LIGHTOFF

30 SEC 	HOLDING RELAY (start button)

360pph	MAX FUEL FLOW < LIGHTOFF

845°C	ABORT START

925°C	ABORT AIRCRAFT IF EXCEEDED

AIRSTART

0-25K	     DASH-1 ENVELOPE

2000AGL-25K  RECOMMENDED ENVELOPE

200-310KIAS   AIRSPEED ENVELOPE

12%	     MIN RPM (200KIAS)

18-20%	     HEART OF ENVELOPE

(280-310 KIAS)

24%	     MAX RPM (310 KIAS)

TEMPERATURES

<-15°F	T5 LIMITS MAX FUEL FLOW

<-2°F	T5 BEGINS TO OPEN NOZZLE

140°C	MIN IDLE (flight)

400-600°C	NORMAL IDLE (ground)

140-630°C	CONTINUOUS RANGE

630-645°C	MIL/MAX RANGE

845°C	ABORT START

925°C	MAXIMUM ALLOWABLE

ALTITUDES

0	MIN EJECT (at 50 KIAS; with lanyard)

50 AGL	MIN CHASE, IP

100 AGL	MIN PRACTICE S/E GO, 60% FLAP

100 AGL	MIN FORM LOW APPROACH

300 AGL	MIN CHASE

300 AGL	MIN PRACTICE S/E GO, FULL-FLAP

400 AGL	MIN TURN > T/O (Except Closed)

500 AGL	MIN LEAD CHANGE, OVER LAND

1000 AGL	MIN LEAD CHANGE, OVER WATER

1000 AGL	MIN IFR ALT TO TURN > DEPT.

<1000 AGL	WINGMAN MUST BE ABOVE LEAD

1000 AGL	MIN ROUTE ABORT  5NM

1500 AGL	MIN LEAD CHANGE, NIGHT/IFR

2000 AGL	MIN BAILOUT (CONTROLLED)

2000 AGL	MIN OVER NAT’L PARKS

5000 AGL	NO ROLLING MANEUVERS BELOW

5000 AGL	MIN FOR AEROBATICS

6000 MSL	GRAVITY-FEED GUARANTEED

8000 AGL	MIN ALT FOR STALLS

>10K	MUST USE OXYGEN (cabin altitude)

15000 AGL	BAILOUT (OUT OF CONTROL)

15000 MSL	FLAMEOUTS HAVE OCCURED

20000 MSL	MAX ALTITUDE FOR STALLS

25K MSL	GRAVITY FEED SHOULD WORK

25K	USAF/FAA MAX IF NON-PRESS

36K	USE >0.9 MACH FOR CRUISE

ELECTRICAL

10 volts	MIN TO HOLD BATTERY RELAY

18 volts	MIN TO CLOSE BATTERY RELAY

24V/13AHr:	    BATTERY RATING

115/200VAC:  AC SYSTEM VOLTAGE

320/480 CPS:  AC SYSTEM RATING

BAILOUT

15000 AGL	OUT OF CONTROL

2000 AGL	CONTROLLED

>500 KIAS	EJECTION INJURIES LIKELY

11,500-15,000MSL: BAROSTAT

FUNCTIONS

20, 200 KIAS, NO-SINK:

OPT LOW-ALTITUDE

3.42 sec	EJECT  HANDLE  FULL CHUTE

- CHUTE WILL OPEN IMMEDIATELY IF D-RING IS PULLED, REGARDLESS OF ALTITUDE OR AIRSPEED

# CAUTION LIGHTS

| LEFT GEN | RIGHT GEN |
| --- | --- |
| UTIL HYD | FLT HYD |
| FUEL PRESS | FUEL PRESS |
| ANTI-ICE | OXYGEN |
| FUEL LOW | XFMR RECT |

LEGEND:

GENERATOR: BAD GENERATOR, PHASE,

GEARBOX, PROTECTION

PANEL, OR BUS ON THAT SIDE

HYDRAULIC:  PRESS. < 1500 PSI, OR TEMP HI;

GOES OUT > 1800 PSI

FUEL PRESS:  NORMAL PRESS 10 PSI;

WARNING @ 6 PSI

OXYGEN:	      LOX < 1 LITER; POSSIBLE

DUE TO SLOSHING < 3 LITERS

XFMR REC:    BATT POWERING DC BUS

ANTI-ICE:	      SYSTEM ON; COMES ON WITH

RT A-C FAIL

FUEL LOW:    FUEL < 25025 FOR 7.5 SECONDS

##### GEAR SQUAT SWITCHES

|  | NOSE GEAR: 1. ENABLES NOSE-GEAR STEERING. |  |  |
| --- | --- | --- | --- |
| LEFT MAIN: 1. CONTROLS LANDING LIGHT ANGLE & FILAMENTS. 2. UNLOCKS GEAR HANDLE FOR RETRACTION. |  | RIGHT MAIN: 1. TURNS ON/OFF NOSE-COMPARTMENT AVIONICS BLOWER. |  |

PATTERN SPACING

INITIAL:	FROM  APP. END  3000’ DOWN

135 ASPECT 3000’ SPACING

ABEAM 	6000’ SPACING

FINAL TURN: FOLLOW 2/3  3000’ SPACING

FOLLOW ABEAM  6000’

# COMPATIBLE ARRESTING GEAR

MA-1*; MA-1A*

BAK-15; 61QSII

* ENGAGE BETWEEN 60 AND 120 KNOTS,

### CENTERED, BRAKES OFF, BOARDS CLOSED

CROSS-COUNTRY PROCEDURES

1.  MAXIMUM OF 12-HOUR DAY FOR STUDENTS or IPs.

2.  IPs & STUDENTS MAY HAVE MAXIMUM

OF 4.5 HOURS FLYING PER DAY.  THIRD SORTIE CANNOT BE AT NIGHT.

3.  IPs MAY FLY 6.5 HOURS/DAY NON-INSTRUCTIONAL (C-T).

4.  AT CIVILIAN FIELDS, GET WX BRIEF BY CALLING FSS @ 1-800 WXB-RIEF, OR CALL

COLUMBUS COMMAND POST 1-800-982-4257, AND GET PHONE PATCH TO WX (x2970).

5.  SAVE/TURN IN ALL FUEL RECEIPTS ASAP.

6.  TRAVEL VOUCHER MUST BE FILED

WITHIN 5 DAYS OF RETURN.

7.  COORDINATE CHANGES IN ITINERARY THROUGH THE TOP-3 OR COMMAND POST .  USE 1-800 NUMBER ABOVE.

AETC T-38 PILOT CURRENCIES+++

LANDING:

31-90 DAYS: REGAIN CURRENCY WITH AN IP

91-135 DAYS:  SAME AS ABOVE PLUS EP/INSTRUMENT TABLE-TOP REVIEW (IP-conducted)

136-210 DAYS:  ABOVE PLUS RECUR FLIGHT AND OPEN/CLOSED-BOOK TESTING

RCP LANDINGS:  60 DAYS NON-EXP; 90 EXP

APPROACHES:

PRECISION:  45 DAYS NON-EXP; 60 EXP

NON-PREC:  NONE

FORMATION:

FORM T/O OR LAND:  60 DAYS

F-M: 90 DAYS

OTHER:

INSTRUCTIONAL SORTIE:  60 DAYS

STALL RIDE:  180 DAYS

LOW-LEVEL:  120 DAYS

+++CURRENCIES ARE FROM 11-2T/AT-38V1

CAUTIONS, NOTES, AND WARNINGS

* NEVER ATTEMPT GEN. RESET MORE THAN ONCE IF FIRST CYCLE UNSUCCESSFUL

* XFMR RECT OUT BLINK IS NORMAL FOR A/B OR FLAP/GEAR OPERATION

11-2T/AT-38V1 REQUIREMENTS

12 	PREC APPROACHES A YEAR

12 	NON-PRECISION A YEAR

6	PENETRATIONS

4	NO-FLAPS

4	SINGLE-ENGINE

4	TP-STALLS

4	CIRCLING APPROACHES

4	FORM T/O & LAND

4	S/E-GO AROUND

2	EP SIMS

2 	LOW-LEVELS

# PHONE NUMBERS

COLUMBUS PREFIXES:

- COMM:	(601) 434-XXXX*

- DSN:	742-XXXX

COMMAND POST	7020; 1-800-982-4257

50th DUTY DESK	7734

BASE OPS		2998/2861

WEATHER		2970

MAINTENANCE	2896

WING SAFETY	2522

43rd		3075; 1-877-281-1482

BILLETING		2548

COLLECT:		1-800 COLLECT

FSS		1-800-WXB-RIEF

RAVEN		7722

HAWK		7716

EAGLE		7712

FALCON		7720

T-38 FLT CC	7739

MISCELLANEOUS

2050lb	ENGINE THRUST (Mil)

2900lb	ENGINE THRUST (AB)

9.0%	ANTI-ICE THRUST LOSS (Mil)

6.5%	ANTI-ICE THRUST LOSS (AB)

>90	DEFINITION OF AEROBATICS

GEAR-DOWN CALL REQ’D BEFORE CROSSING RWY THRESHOLD

CABIN PRESS SCHEDULE:

< 8000		AMBIENT

>8000, < FL230	80001000

>FL230		(ALT-6000)2,

2000

NOSE GEAR UP TO ¾ DOWN: 6 RUDDER

>3/4 DOWN: 30

60 SECONDS: VOR STATION IDENTS

35 SECONDS: TACAN STATION IDENTS

15 SECONDS: TACAN SELF-TEST 180

7.5 SECONDS :

-TACAN SELF-TEST DME/270 DISPLAY

7.5 SECONDS: DELAY FOR FUEL LOW LIGHT

10 INCHES: HOW FAR TO PULL ALT GEAR RELEASE

1 SECOND: ENGINE RUNS AFTER FUEL SHUT-

OFF SWITCH IS CLOSED

15° PER MINUTE: ADI FAST-ERECT RATES

# FORMATION

3 FEET	NORMAL WINGTIP SEPARATION

10 FT	MIN LATERAL  (form land)

150 FT	TAXI SPACING STAGERRED

150 FT	MIN RWY WIDTH, FORM T/O

300 FT	TAXI SPACING ON CENTERLINE

5 SECS	MIN BREAK INTERVAL

10 SECS	MIN INTERVAL (VMC)

20 SECS	MIN IMC TRAIL (except San Antonio)

100AGL	MIN FORM APP LOW-APPROACH

500 AGL	MIN DAY LEAD CH.  (over land)

1000 AGL	MIN DAY LEAD CH. (over H2O)

1000 AGL	WINGMAN NO LOWER THAN

LEAD

1000 AGL	NO TURNS BELOW IF WX<1500&3

1500 AGL	MIN NIGHT/IMC LEAD CHANGE

(or radar downwind altitude)

30 AFT	MAX AFT LINE FOR LEAD CH.

5 KNOTS	PUT WINGMAN UPWIND

15 KNOTS	MAX X-WIND, FTO or LAND

150 KIAS	MIN MANEUVERING SPEED

500 & 1 ½  OR PWC:  MIN FORM APP WX

1500&3SM	MIN VFR REJOIN BELOW WX

70% RL	MAX T/O DIST FOR FORM T/O

80% RL	MAX T/O DIST FOR INTVL T/O

*RL MUST EXCEED T/O DIST BY MIN. 1000 FT FOR FORM T/O; 150’ WIDE (min)

*NO STANDING WATER, ICE, SLUSH, OR SNOW FOR FORM TAKEOFF

*NO WET RWY, ICE, SLUSH, OR SNOW FOR FORM LAND

*NIGHT FORM T/O REQUIRES RADIO CALLS

FOR BRAKES AND GEAR

*NIGHT/IMC LEAD CHANGE REQUIRES A

RADIO CALL

*NO NIGHT REJOINS

* MAX 4 JETS IN IMC

*NO ROLLING MANEUVERS FOR POSITION

BELOW 5000 AGL

# TOLD

10 RCR	MIN FOR T/O

70% RL	MAX T/O DIST FOR FORM T/O

80% RL	MAX T/O DIST FOR INTVL T/O

* NO ROLLING T/O UNLESS RL>CFL BY

AT LEAST 1000 FT

*IF LAND DIST >70% RL, SELECT DIFF. RWY

*BUG > OF ARS OR CEFS IF NO BAK-15

ROLLING T/O ADDS 150-300’ TO TOD & CFL

LAND DIST = 2500+FUEL

60% FLAP ADDS 500’

NO FLAP = DOUBLE

NORMAL TECHNIQUE IS 150% OF DASH-1

# FLIGHT TIME LIMITATIONS

*CREW REST IS 8 HOURS IN 12 HOURS

*12 HOURS BOTTLE TO THROTTLE

*24 HOURS NO-FLY AFTER SCUBA-DIVING

*NO DELAY REQ’D FOR ALT CHAMBER 25K

*7 DAY MAX: 30 HOURS

*30 DAY MAX: 125 HOURS

*90 DAY MAX: 330 HOURS

*YEARLY MAX: 1000 HOURS (FAA)

| AEROBATICS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MANEUVER | #### KNOTS (PROFIC) | POWER (PROFIC) | KNOTS (POST-PRO) |  | POWER (POST-PRO) | ALTITUDE | DISTANCE | FUEL |
| ##### LAZY-8 | 350 | 95% | 300-400 |  | 90%-MIL | 4-6K | 6NM | 100 lb |
| ##### BARREL | 400 | 95% | 300-500 |  | 90%-MIL | 4-6K | 7-8NM | 100 |
| CHANDELLE | 400 | 95% | 350-450 |  | 90%-MIL | 6-7K | 3NM | 100 |
| SPLIT-S | 200 | MIL | 200 |  | 90%-MIL | 8-10K | 2-3NM | 50 |
| CLOVER | 450 | MIL | 400-500 |  | 550-MIL | 8-10K | 5NM | 200 |
| LOOP | 500 | MIL | 450-500 |  | 550-MIL | 8-10K | 5NM | 150 |
| IMMELMAN | 500 | MIL | 450-500 |  | 550-MIL | 8-10K | 2NM | 100 |
| CUBAN-8 | 500 | MIL | 450-500 |  | 550-MIL | 8-10K | 7NM | 200 |
| AHC |  |  |  |  |  |  |  |  |
| PITCHBACK | 450-500 KIAS |  |  | 550-MIL |  | 4-6K | 2NM | 100 lb |
| SLICEBACK | 200-300 |  |  | 90%-MIL |  | 6-8K | 2-3NM | 100 lb |
| STAB DEMO | 60 NOSE-HIGH |  |  | 85% MINIMUM |  | 6K | 5-6NM | 100 lb |
| AFT STICK STALL | 6000 FPM DOWN |  |  | 80% MIN |  | 6K, < FL200 | 4NM | 100 lb |
| SIM TRIM FAIL | <FL 200 |  |  | 300 KIAS |  |  |  | 100 lb |
| RUDDER EFF. |  |  |  | 85%-MIL |  | 2K | 3NM | 100 lb |
| AIL. EFF. |  |  |  | 85%-MIL |  |  | 3NM | 100 lb |
| ACCEL X |  |  |  | MIL-MAX |  | 1K | 6-7NM | 100 lb |
| TURN REV. | 300-400 KIAS; 4G |  |  | MIL MIN |  |  | 4-5NM | 100 lb |
| #### INSTRUMENT EXERCISES |  |  |  |  |  |  |  |  |
| STEEP TURNS | 45-60 |  |  | 300 or 350 KIAS |  |  | 4-5NM DIAM. | 100 |
| WINGOVER | #### 350 KIAS |  |  | 95% |  | 1-2K | 4NM | 50 |
| AIL. ROLL | 300 KIAS |  |  | 85% |  | 1K | 3NM | 50 |
| #### MISCELLANEOUS |  |  |  |  |  |  |  |  |
| G-WARMUP | 400-420 KIAS |  |  | 4G THEN 5G |  | 1K | 4NM | 100 |
| TP STALLS | 8000 AGL – FL200 |  |  | 80% MIN |  | 500-1K | 3NM | 200 |
| SLOW FLT | BUG MINUS 10 |  |  | #### VARIOUS CONFIGS |  |  |  | 100 |

##### “G” LIMITS

| FUEL REMAINING | SYMMETRICAL | ASSYMM |
| --- | --- | --- |
| 3500 | -2.3/+5.8 | 0/+4.1 |
| 3000 | -2.5/+6.0 | 0/+4.3 |
| 2000 | -2.7/+6.6 | 0/+4.7 |
| 1000 | -2.9/+7.2 | 0/+5.1 |

ENGINE OP LIMITS

## See checklist page EE-1

WEATHER, IFR FILING, AND 11-202V3

| DESTINATION WX | #### GO / NO-GO? | ALTERNATE REQUIRED? | #### ALTERNATE WX REQ’D |
| --- | --- | --- | --- |
| 3000&3 or 2SM ABOVE PUBLISHED, WIG* ###### See Notes 13 & 14 | IFR: YES | USUALLY NO: ###### See Note 12 | 1000&2, OR 500 AND 1 PLUS MINS, WIG ###### See Notes 3 & 15 |
|  | VFR: YES | N/A |  |
| FORECAST < 3000&3 OR 2 ABOVE, BUT >1500&3 ###### See Notes 13 & 14 | IFR: YES | YES | 1000&2, OR 500 AND 1 PLUS MINS, WIG ###### See Notes 3 & 15 |
|  | VFR: YES | N/A |  |
| FORECAST < 1500&3, BUT >PWC/MINS ###### See Notes 13 & 14 | IFR: YES | YES | 1000&2, OR 500 AND 1 PLUS MINS, WIG ###### See Notes 3 & 15 |
|  | VFR: NO | N/A |  |

*WIG=WHICHEVER IS GREATER

1.  FLIGHT INTO FORECAST ICING (other than light rime) IS PROHIBITED.

2.  FLIGHT INTO ANY ACTUAL/REPORTED ICING IS PROHIBITED.

3.  AETC REQUIRES FORECAST WINDS 1 HOUR OF ETA TO BE WITHIN LIMITS FOR DESTINATION

and ALTERNATE.

4.  MIN AETC WX FOR LOW-LEVEL IS 3000 & 5.

5. BASIC USAF VFR IS 1500&3.

6. FUEL RESERVE IS 10% OF TOTAL PLANNED ETE, OR 20 MINUTES, WHICHEVER IS GREATER

-20 MINUTES BASED ON MAX ENDURANCE @ 10K.

7. OK TO DESIGNATE CIVIL (Non Joint-Use) AIRPORT OR MILITARY “OBO/PPR” AS AN ALTERNATE WITHOUT PPR OR PRIOR COORDINATION IF NO OTHER SUITABLE AIRPORT IS AVAILABLE.

8. ALTERNATE CANNOT BE      "              N/A "

9. OFF-AIRWAY REQUIRES 1000 FT OBSTACLE CLEARANCE   5 NM (NON-MOUNTAINOUS)

-2000 FT (MOUNTAINOUS)

10.  MIN RUNWAY LENGTH IS 8000’ (without waiver).

11.  CAT-D REQUIRES (Note: now OK without an emergency; IC 99-11, 5 Aug 99 to 11-2T/AT-38):

-FINAL 165 KIAS MAX

-MISSED 260 TAS MAX

- ALWAYS DESIGNATE AN ALTERNATE IN AETC IF:

- ONLY APPROACH REQUIRES RADAR

- NAVAID IS UNMONITORED

- AIRFIELD HAS NO WX REPORTING CAPABILITY

- WINDS FORECAST OUT OF LIMITS (See Note 3 above)

13.  IF APPROACH LIGHTING IS INOP, PUBLISHED VIS MINS INCREASE BY 1/2 SM OR PUBLISHED INCREASE AT BOTTOM OF APPROACH PLATE

14. TEMPO CONDITION FOR DESTINATION < 3000&3/2 ABOVE OR EVEN MINS/PWC IS OK; JUST REQUIRES AN ALTERNATE

15. TEMPO CONDITION FOR ALTERNATE BELOW REQ’D IS NOT OK, UNLESS DUE TO RAIN SHOWER

OR THUNDERSTORM (AETC SUPP)

16. IF ONLY APPROACH IS RADAR-ONLY, MUST BE ABLE TO NAVIGATE TO WITHIN 25NM OF DEST.

# DC INSTRUMENTS

STBY ADI

FLAP INDICATOR (reads Right Flap only)

NOZZLES

TURN NEEDLE

NON-REPEATING INSTRUMENTS (NOT-H)

NOZZLE

OIL

TACHOMETERS

HYDRAULICS

# TOLERANCES

RPM 1%; FOR OTHER THAN IDLE OR MIL ON

GROUND

OIL  2 PSI

NOZZLES 3%

FUEL FLOW 25 IDLE, 50 MIL/AB

EGT  7.5; 15 TOTAL, AVG 630-645

ALTIMETER  150 STBY/RESET BELOW 10K

 250 STBY/RESET ABOVE 10K

* *10K IS NOT SPECIFIED**

75 BETWEEN COCKPITS AND

FIELD ELEVATION

HSI: 8 to MAG COMPASS; 5 to KNOWN HEADING

TACAN: @ GND √-PT; CSW4 of BEARING

POINTER

60 to 1 STUFF

MACH-2	TURN RADIUS

5 descent	300KIAS, 80%

7	300KIAS, IDLE

10	300KIAS, 80%, BOARDS

13	300KIAS, IDLE, BOARDS

7+(TAS/10)=BANK ANGLE FOR STD RATE TURN

BANK TO MAINTAIN ARC:

= 30 x (TURN RADIUS)/ARC

TAS  IAS + 5% OF IAS per 1000 ft ALTITIDE

LOW-LEVEL TIMING CORRECTIONS:

WIND:    1 OF CRAB FOR 6 KNOTS CROSS

INCREMENT:  (EXAMPLE)

10 SECONDS LATE, PUSH UP 60 KNOTS FOR 1.0 MINUTE

-OR-

PUSH UP 30 KNOTS FOR 2.0 MINUTES

NEXT LEG METHOD:

NEXT LEG MINUTES/ERROR (SECONDS) TIMES 6 = AMOUNT TO ADD/SUBTRACT OVER NEXT LEG

Talisman (tal’ is man, n) An object or engraving which averts evil or brings good luck through

magical powers.

# RIGHT AC

FLAPS

ADI

TACAN

HSI

ILS

TRIM

# LEFT AC

LIGHTS (external)

IFF

STAB AUG

AOA

3-PHASE AC

BOOST PUMPS

ADI

RECTIFIERS (TRs)

FUEL PUMPS

# AIRFRAME-MOUNTED GEARBOX

GENERATORS

HYDRAULIC PUMPS

# RESERVE PAY CODES

AT/MAN-DAYS	1

UTA		2

TPs		3

# LOST AIRCRAFT

1.  CONFESS

2.  CLIMB........................ABOVE 5000 AGL

3.  COMMUNICATE.......243.0 Mhz or As REQD

4.  CONSERVE………….0.3 AOA or 230+GAS

- COMPLY

*SUBMIT RECOMMENDATIONS FOR

CHANGES AND ADDITIONS/DELETIONS

TO MAJ BENYSHEK, RAVEN, x7722

| STATUS | TYPE OF DUTY | PAID TRAVEL DAY? | PAID LODGING? | PAID IF NO FLYING? | TRNG PERIODS PER DAY: | RETIREMENT POINTS | REMARKS | PAY DOCUMENT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inactive-Duty Training “IDT” | UTA | NO | YES | YES | 2 (4 HOURS EACH) | 1 PER PERIOD | 48 periods/year | 40A |
|  | TP | NO | YES | SEE NOTES BELOW | 2 (4 HOURS EACH) | 1 PER PERIOD | 18 MAX TPs per QTR MAX 27 PTs/YR (TPs); PLUS 48 UTA PTs = 75 MAX | 781 |
| Active-Duty Training “ADT” | ANNUAL TOUR | YES, 4 MAX/YR | REIMBURSED | YES | 1 (8 HOURS) | 1 | 14 DAYS MIN; 15 MAX; 4 TRVL DAYS MAX/YR | ORDERS & TRAVEL VOUCHER |
|  | MAN-DAYS | YES | REIMBURSED | YES | 1 (8 HOURS) | 1 | MUST FINISH ANNUAL TOUR BEFORE MAN-DAYS ALLOWED | ORDERS & TRAVEL VOUCHER |

Cancelled Flying During AFTP (WX, MX, etc):

- Must get approval from your flight’s AGR to accomplish any TP.

- Can still get paid if you do STUDENT or IP-related Training.

- Examples:

- Brief Student/conduct P-Missions/other training support

- Simulator mission

- Self-study ( checkride prep, PME, etc)

- Annual Flight Physical, altitude chamber, getting off DNIF, etc.

NOTE: GTP – LIMIT ONE PER DAY / 12 PER YEAR  (except Sim-TPs can get another 12 for 24 total)

Bundling:

- For authorization of paid travel, must have AD days at beginning of trip.

- AD days must equal or exceed IDT days, up to 3 AD-days minimum.