WAVEOFF

Lowest IP initiated waveoff alt 100’AGL.

Reasons to waveoff:

1. Excessive overshoot/ >30AOB on appr turn.

2. Land clearance not received by short final

3. IP, wheels watch, Tower, or RDO issue verbal or visual waveoff signal.

4. Three green gear lights not visible after rolling out on final.

5. P feels unsafe

6. Consider waveoff if can’t land on first 1/3 of rwy.

Once decision to waveoff is made:

1. Make ‘waveoff’ call to Tower.

2. Reduce pwr to level at 500’AGL.

3. Don’t exceed 130K or overtake other aircraft.

4. Add pwr as needed to maintain + rate of climb.

5.  Offset slightly from rwy to keep rwy traffic in sight.

6. Report waveoff and reason to Tower.