THE BREAK

[1] 1000’MSL, 170K.

[2] Offset from runway to pattern side.

[3] Over the numbers, CH 4 (TWR) “(Navy Corpus/Cabaniss) Tower, Montana (063), over the numbers for the break.” When tower clears break:

[4] Roll towards pattern (45AOB).  During turn:

[A] Power – 400 ft-lbs

[B] Flaps – APPR

[C] Gear Down Landing Checklist

[D] Descend to pattern alt (800’MSL)

[E] Slow to 120K

[5] At 180 position:

[A] Report “(Navy Corpus/Cabaniss) Tower,

Montana (063), left/right 180, 3 down & locked,

touch & go/full stop.”

[B] Power – 300-375ft-lbs

[C] Roll – 15-30AOB

[D] Descend – IVSI 500fpm

[6] At 90 position:

[A] Perpendicular/500’MSL/110K

[7] Final:

[A] Roll out extended centerline, 1000-1200’ from

threshold at 250’MSL & 110K

[B] Aim for numbers

[C] Props full forward.

[D] Verify “3 down & locked.  Review me

complete.”

[E] Over numbers, Power – IDLE

[F] Flare – Electric trim off all pressure.  Land.