PREFLIGHT

__1. Check current and forecast weather

__2. Check local area NOTAMS

__3. Note runway in use

__4. Check with SDO for aircraft assigned

__5. Bring ATJ to brief (if required)

Note: Return to STUCON before flight

__6. If solo, obtain brief

__7. Aircraft Inspection and Acceptance Record (OPNAV 4790/141) AIA ‘A’ sheet

Check for: [1] Pilot accectance [2] Maintenance certification- check fuel, oil, & oxygen.

__8. Read aircraft logbook.  Print crewmember names/ squadron on back of ‘A’ sheet.  If pax embarked, list name/rank/SSN/duty station/activity/ and debarkation point.

__9. PIC reviews all outstanding descrepancies and corrective actions for last 10 flights.  Verify no downing descrepancies.  Note Daily/ Turnaround Inspection Record.  Unless ‘hotseat,’ turnaround must be signed after every flight.  Turnaround good for 24 hours if aircraft not flown.  Daily completed at end of flying day – good for 72 hours if aircraft not flown, 24 hours if flown.

__10. PIC signs for aircraft and places logbook back in slot.

__11. Ensure NATOPS, local enroute chart (L-15), Course Rules Manual, and FTI are onboard aircraft.

__12. Compute takeoff wt. to ensure below max 9,650 lbs for takeoff.  (Subtract basic wt, baggage, crew/pax from max landing wt (9,168lbs) to compute max total fuel required prior to first landing.

__13. Calculate accelerate-stop distance (26-7), one-engine inop max climb rate (27-3) for first takeoff.  Compute stall speeds for flaps up, flaps full down, and flaps approach at 30 AOB (25-14).  [Get density alt from squadron homepage]

# AIRCRAFT INSPECTION

__1. Student start preflighting interior

__2. IP inspect from airstair door aft

__3. OBS inspect from airstair forward

__4. After brakes pumped firm, remove chocks during preflight.

__5. Don’t leave any panels open or caps off

__6. If ‘hotseating,’ review logbook, take an ‘A’ sheet to aircraft (w/ crewmembers names on back).  Get a discrepancy brief and sign for aircraft after original PIC releases it.  Relieved PIC return new ‘A’ sheet to Maintenance.  Engines remain running.

__7. Hearing protection worn on line!

__8. During warm wx, open cockpit windows and leave cabin door open.

__9. During cockpit preflight, set up audio panel for “Before Start Checklist.”

__10. If no OBS, last crewmember in locks cabin door.