SSE PATTERN (02 May 02)

CASE 1 (UPWIND)

Power-UP, Rudder-UP, Clean-UP, Speed-UP (continue climb min 102K, prefer 110K), Checklist

[1] __ Power -IDLE (concur)

[2] __ Prop – FEATHER (concur)

[3] __ Condition Lever – FUEL CUTOFF (concur)

[4] “Is it fire or fuel related?”  Simultaneously Props—1900, reset Max Power (2nd red line)

If fuel or fire related:

[5b] __ Firewall valve – CLOSED (concur)

[6] __ Fire extingisher – EXTING (concur)

[7] __ Bleed Air Valve – CLOSED (concur)

[5a or 8] “Give me the first 2 items of the dead engine checklist, you have the comms, declare an emergency, this will be a full stop landing.”

Continue turn (start 300 – 500), climb to 800 downwind.

SSE Full Stop Brief: “This will be a simulated single engine full stop landing.  Once safely on the deck, I will pull both power levers over the detent, smoothly apply reverse on the operating engine, countering any swerve with rudder and aileron as necessary to maintain directional control,  As rudder effectiveness is lost, I will return both power levers toward flight idle until directional control is regained.”

CASE 2 (CROSSWIND)

Wings level, Power-UP, Rudder-UP, Clean-UP (Flaps-UP), Speed-UP, Turn, Checklist

[1] __ Power -IDLE (concur)

[2] __ Prop – FEATHER (concur)

[3] __ Condition Lever – FUEL CUTOFF (concur)

[4] “Is it fire or fuel related?”  If ‘yes,’ go to [5].  If ‘no,’ go to [8].

If fuel or fire related:

[5] __ Firewall valve – CLOSED (concur)

[6] __ Fire extingisher – EXTING (concur)

[7] __ Bleed Air Valve – CLOSED (concur)

[8] “Give me the first 2 items of the dead engine checklist, you have the comms, declare an emergency, this will be a full stop landing.”

SSE Full Stop Brief: “This will be a simulated single engine full stop landing.  Once safely on the deck, I will pull both power levers over the detent, smoothly apply reverse on the operating engine, countering any swerve with rudder and aileron as necessary to maintain directional control,  As rudder effectiveness is lost, I will return both power levers toward flight idle until directional control is regained.”

CASE 3 (DOWNWIND)

Power-UP (Max if Dirty,1000 if Clean), Rudder-UP, Clean-UP(Gear-Up, Flaps-Up), Speed-Up (120K), Checklist

[1] __ Power -IDLE (concur)

[2] __ Prop – FEATHER (concur)

[3] __ Condition Lever – FUEL CUTOFF (concur)

[4] “Is it fire or fuel related?”  If ‘yes,’ go to [5].  If ‘no,’ go to [8].

If fuel or fire related:

[5] __ Firewall valve – CLOSED (concur)

[6] __ Fire extingisher – EXTING (concur)

[7] __ Bleed Air Valve – CLOSED (concur)

[8] “Give me the first 2 items of the dead engine checklist, you have the comms, declare an emergency, this will be a full stop landing.”

[9] Just prior to 180: Flaps-APPR, “Gear Down Landing Checklist.”

[10] Through 180: Power-700ft-lbs, Speed - (120K Max, 110K Min)

CASE 4 (APPR TURN)

Power-UP (700ft-lbs), Rudder-UP, Clean-UP (N/A), Speed-UP (115K), Checklist (Just 1st 3 Items)

[1] __ Power -IDLE (concur)

[2] __ Prop – FEATHER (concur)

[3] __ Condition Lever – FUEL CUTOFF (concur)

[4] “Give me the first 2 items of the dead engine checklist, you have the comms, declare an emergency, this will be a full stop landing.”

Note: Fire or fuel prob after 180, only do 1st 3 items of emerg shutdown checklist.

NOTES:

[1] If Geardown Landing Checklist not complete by 90, SSE waveoff.  Power-Max, Rudder-Up, Gear-Up, Flaps-Up, Speed-Up (120K), Dead Engine Checklist.

[2] If fuel leak, before starting Emergency Shutdown Checklist – “__Condition Lever – FUEL CUTOFF (concur)”

CASE 5 (AFTER 90)

Power-UP (700 on op eng), Rudder-UP, Clean-UP (N/A), Speed-UP (min 110K), Checklist (1st 3 items optional/recommended)

[1] __ Power -IDLE (concur)

[2] __ Prop – FEATHER (concur)

[3] __ Condition Lever – FUEL CUTOFF (concur)

[4] “Props full forward, 3 down and locked, review me complete, you have the comms, declare emergency once safely on deck”

[5] Land using SSE Full Stop Landing Procedures

# SSE FULL STOP LANDING

Upon touchdown:

[1] Power – IDLE

[2] Reverse – AS REQ

[3] Rudder & Braking – AS REQ (counter act yaw)

[4] Scan to end of runway

[5] If yaw excessive, reduce reversing and stop with brakes (don’t lock brakes)

[6] Complete when at slow taxi on rwy

[7] Clear rwy if practicable, then shutdown

[8] Don’t taxi on one engine!

[9] On rollout, dead engine into wind.

[10] In air, good engine into wind.

[11] Must complete a SSE Full Stop brief to perform SSE Full Stop Landing.  Initiate from a Case One or Two scenario and only when required in syllabus.