Fam 1 Study Guide (16 Apr 02)

# POWER DEFINTIONS

1. Max allowable power: Max power available for takeoff (limited to 5 min)

2. Max continuous power: Max power available, not limited by time.  (Intended for emergency use at pilot’s discretion)

3. Cruise climb power: Max power approved for normal climb.

4. Cruise power: Power needed to maintain level flight at desired alt while traveling at airspeed within normal op range of aircraft (varies with alt and airspeed).

# ITT

1. Normal (green arc) op range: 400-790C

2. Max ITT for start is 1090.  Time limited to 2 seconds.

3. The right engine is limited to 790C degrees during the start procedure for the left engine.

4. Always report the amount and duration of start exceeding 925C.

5. Always report the amount of operating ITT exceeding >790C.

6. Max ITT low idle 685C.

7. Max ITT in reverse 790C.

8. Max ITT for acceleration 850C (for 2 sec).

9. Max ITT for cruise climb 765C.

10. Max ITT for cruise 740C.

# TORQUE

1. Normal (green arc) operating range: 400-1315ft-lbs.

2. Max at 2200RPM is 1520 ft-lbs.

3. Max at 1900RPM is 1315 ft-lbs.

4. Acceleration limit is 2100 ft-lbs.

PROP RPM (N2)

1. Normal (green arc) op range 1800-2200RPM.

2. Max RPM for takeoff 2,200 RPM. (NATOPS 10-12)

3. Min RPM for takeoff 2,200 RPM. (NATOPS 10-12)

4. Max RPM for acceleration 2420RPM (for 2 seconds).

5. Max RPM in reverse 2100RPM (limited by pneumatic section of primary governor)

6. Overspeed governor limited to 2,288(+/- 40)RPM.

7. Fuel topping governor limited to 2,332RPM.

N1 LIMITS (%)

1. Normal (green arc) operating range 50 – 101.5%

2. Low Idle range 50-53%

3. High idle range 64-67%

4. Max N1 acceleration 102.6%

5. Max N1 reverse 86%

6. Min N1 in reverse 80%

7. Max N1 can decrease during manual feather check NONE.

## OIL PRESSURE AND TEMPERATURE

1. Normal (green arc) op range for oil pressure 85 – 100psi

2. Normal (green arc) op range for oil temp 10 –99 C

3. Min oil temp for engine start –40C

4. Min oil pressure on the ground 40psig.  Min oil pressure in the air 40psig.

5. Normal oil pressure must be obtained any time the engines are at pwr settings above 72%N1.

## BATTERY SYSTEM

1. The 3 functions of the battery: [1] Emergency power [2] Damper for the electrical system [3] Self-contained starts.

2. The battery is rated 42A-H and 28Vdc.

3. Min volts for a battery start 22V.

4. Min volts for an APU start 20V.

5. Items powered entirely by the hot battery bus: [1] L & R Fire exting [2] Threshold lights – (threshold light, wing spar light, aft compartment light, baggage door open light, cabin door observer light)

6. Items dually powered by hot battery bus: [1] crossfeed valve [2] L &R firewall valves [3] L & R boost pumps

# FLIGHT PROCEDURES

1. Emergency Engine Shutdown on Deck

[1] Condition Levers – FUEL CUTOFF

[2] Firewall Valves – CLOSED

[3] Boost pumps – OFF

[4] Fire extinguishers – AS REQ

[5] Gang Bar – OFF

[6] Evacuate aircraft

2. Min altitude for a seat change: [1] Overland 800’AGL [2] Overwater 1000’AGL

3. Min altitude for approach to stalls 5000’AGL

4. Indications warranting a stall recovery: [1] Stall warning horn [2] Wing buffet [3] Uncontrollable loss of altitude [4] Inability

to maintain wings level/ selected roll attitude [5] 25 Units AOA

5. Procedures for a start malfunctions

### ABNORMAL STARTS

Monitor ITT during start.  If ITT rate of increase likely to exceed 925C or no rise in ITT within 10 sec after selecting LOW IDLE with the Condition Lever: *[1] Condition Lever – FUEL CUTOFF (note ITT decrease below 790C) [2] Starter – OFF

Caution: Starter use limited to 40 sec on, 60 sec off, 40 sec on, 60 sec off, 40 sec on, then 30 min off.

Note: If starting attempt discontinued, allow 60 sec delay to drain fuel and cool starter, then motor starter for 15 sec min (toggle starter switch DOWN).  Allow engine to come to complete stop before another start attempted.

6. Conditions warranting a waveoff in the landing pattern: [1] Excessive overshoot/  >30AOB during appr turn [2] Landing clearance not received by short final [3] Any verbal or visual waveoff signal issued by IP, wheels watch, Tower, or RDO [4] 3 green lights for gear not visible after rolling onto final [5] pilot feels unsafe condition exists [6] Consider waveoff if can’t touchdown in first 1/3 of rwy

## BRAKE EMERGENCIES

1. Procedures for BRAKE FIRE (CALL, STOP, CHECK)

*1.  Firefighting assistance – REQUEST

*2.  Stop aircraft

*3.  Emergency Engine Shutdown on Deck

[1] Condition Levers – FUEL CUTOFF

[2] Firewall Valves – CLOSED

[3] Boost pumps – OFF

[4] Fire extinguishers – AS REQ

[5] Gang Bar – OFF

[6] Evacuate aircraft

Warning: Step well away from aircraft (preferably well behind incase wheel explodes sideways).  Don’t fire the fire!

2. Procedures for LOSS OF BRAKES

[1] Maintain directional control with rudder, nosewheel steering, or differential power. [2]Use prop reversing or beta to decelerate. [3] If possible, maneuver to open area and let aircraft stop [3] Don’t taxi! [4] Try to reseat shuttle valve by pulling aft on the top of the brake pedals.

3. Procedures for HOT BRAKES

[1] Stop aircraft by reverse thrust and minimal braking

[2] Request assistance, if needed

[3] Cool brakes with propwash

[4] Inspect brakes before using them again.

Caution: Brakes can lock during max braking, which may blow tires.

## FLIGHT PROCEDURES

1. The LaMirage arrival altitude to Cabiness is flown at  ________ feet.

2. The appropriate squawk to and from Cabiness while flying the LaMirage departure/arrival is _________.

3. The stall recovery procedures:

## 1. (MAX, RELAX, ROLL)

MAX ALLOW PWR (2nd redline, 790 ITT)

RELAX yoke pressure

ROLL wings level

2. Once recovered, incr pitch and accel

3. Once level or climbing (>85K) immed F-APPR

4. G-UP

5. F-UP

6. Complete when climb on hdg < 110K

4. The difference between an approach turn stall vise all other stalls is that after 90deg turn, maintain a 30AOB turn.  Then pilot will also reduce pwr to 300ft-lbs, Pitch 10deg UP, Trim to 100K, then stop trim.  Maintain turn and pitch.