FLIGHT NOTES UP TO FAM-4 (AS OF 29 APR 02)

1. Can relight an engine for: [1] engine fire without secondary indications [2] chip light [3] oil temp or oil press out of limits [4] overspeeding prop.

2. If aircraft is below 2000’, don’t do windmilling checklist.  Proceed directly to emergency engine shutdown checklist & ditching checklist.

3. Don’t exceed 500’ when leaving Cabaniss until clear of traffic pattern.  When clear, call Cabaniss tower (CH 15) “Cabaniss tower, Montana XXX, clear of traffic pattern.”  Switch to Navy Twr (CH 4) prior to Pt. Sunrise.

4. When computing Accel-Stop Distance – use 30C, Sea Level, 9400lbs.

5. Abort when > 91K or past 2 boards.

6. Waveoff:  [1] Power- AS REQ [2] Gear – UP (when not going to touchdown) [3] Flaps – UP [4] Props – 1900RPM.  Call “waveoff” to tower.  Offset towards pattern side.  Max speed = 130K (shoot for 120K).  Max altitude = 500’.

7. Dual powered items for hot battery bus: (1) Boost Pumps (2) Firewall valves (3) crossfeed valve.

8. For start: when condition levers move to low idle, say “Hack.”  At light off, say “Light Off.”  Note Max ITT.  Stop clock when starter switch turns off and say time.

9. Before leaving Seagull, [1] off freq [2] ATIS [3] Cabaniss landing [4] Inbound to Cabaniss [5] Back up Seagull [6] Within 5 radials of descent radial, terminate with Seagull.  [7] Come up Approach.

10. Break (start at 1000’): [1] Offset towards pattern side [2] Call the numbers [3] Roll 45 AOB [4] While in level roll [a] Power – 400 [b] Flaps – APPR [c] Gear down landing checklist [d] begin descent to arrive at 180 position at 800’ (pattern alt) & 120K.  The remainder of the pattern is the same as a normal pattern.

11. During ignition, secondary fuel nozzles kick in at N1 30%.

12. PY Air – measures air pressure into FCU.  Also controls fuel topping of primary governor to 2,332RPM.

13. If Fuel Transfer Light comes on, but goes out when Transfer switch is put in ‘Overrride,’ have a failure of the 42G float switch.

14.  Also crossfeed from nonfeeding (failed boost pump) side!

15. Fuel Syphoning – occurs only when fuel streaming from filler caps; fuel leaking from any other location is a fuel leak emergency.  For syphoning, do a straight-in, no flap landing.

16. During an emergency, always run through EVERY memory item.

17. Leave Seagull working area with at least 800lbs per side.

18.  During stalls & landing, don’t call for gear down landing checklist until below 155K!  Once plane stalls: MAX power, RELAX yoke (slightly—don’t dump nose, let it drop slightly, then catch it, keep nose up around 5 degrees nose high, accelerate & climb when > 110K), ROLL wings level & ball centered (must initially apply right rudder).

19.  When working Seagull, make sure navaids are tuned up Corpus Vortac (115.5)

20.  On appr stalls, rolling wings level will break the stall, can put nose up to climb away as soon as level, don’t dump nose!

21. To keep pressure on pressure gauge with both engines running, N1 must be at least 75%.

22. To keep pressure on pressure gauge with one engine running, N1 must be at least 85%.

23. Radio call during Smoke/Fume Elimination checklist should include position and that we will be NORDO for a few minutes.

24. The ON/OFF for the TACAN, MKR BCNs, VOR, & ADF is the volume knobs on the audio control panel!!!

25. For emergency descent [PP FLAW]: Power Levers – IDLE, Props – FULL FWD, Flaps – AS REQ, Landing Gear – AS REQ,

Airspeed – AS REQ, Windshield heat – AS REQ.

26. For split flaps: [1] get uncommanded roll [2] visually check flap position [3] reposition flaps to last position [4] brief CP on new landing speeds [5] if no flap landing, off 180, pull pwr to 200ft-lbs [6] need more nose up attitude & trim [7] flatter appr – (final = 110K) (threshold = 105K) [8] don’t close out power levers until touchdown.

27. On approach, if crossing over threshold while hot and power at IDLE – add a little power to soften landing (100 to 200ft-lbs).

28. On approach stalls – after stall, wait a little bit to put flaps to approach so the airspeed builds faster without settling.

29. Slow flight – establish more rudder trim!!!

30. Dynamic Engine Cut “3-2-1-P-G”   300ft-lbs, 2 degrees (takeoff) nose up trim, 1 Prop Sync – OFF, Props- FULL FWD, Gear

Down Landing Checklist

31. Dynamic Engine Cut – Power UP, Rudder UP, Clean UP, Speed UP, Checklist.  After completing engine shutdown checklist, “Sir, give me the first 2 items of the dead engine checklist, take the comms, & declare an emergency.”

32. During single engine, the power lever and rudder are “connected by a string.”  If I push or pull one, I must do the same to the other.

33. Power On glide – 1000 on VSI at 130K until 300’AGL.  At 300’, level off, Flaps – APPR, slow to (90K dual) (91K single), power up to catch VSI of 100K.  Touch down WINGS LEVEL!

34. In Seagull south, when coming up approach (CH 8), “Corpus Christi Approach, Montana__, VFR advisories, southern entry to Cabaniss.”

35. When 5 miles out of Cabaniss, “Cabaniss tower, Montana___, 5 miles to South, entering VFR.”