FAM-5 STUDY GUIDE (29 Apr 02)

ENGINE FAILURE AFTER TAKEOFF

If sufficient runway to stop on ground, ABORT!

If insufficient runway to abort takeoff:

(POWER UP, RUDDER UP, CLEAN UP, SPEED UP, CHECKLIST)

1. Power – AS REQUIRED

2. Gear – UP

3. Airspeed – (102 (obstacles) or 110)

4. EMERGENCY SHUTDOWN CHECKLIST

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

Note: Before addressing the dead engine checklist, transfer comm to CP and have him declare an emergency.

# SSE FULL STOP LANDING

Upon touchdown:

[1] Power – IDLE

[2] Reverse – AS REQ

[3] Rudder & Braking – AS REQ (counter act yaw)

[4] Scan to end of runway

[5] If yaw excessive, reduce reversing and stop with brakes (don’t lock brakes)

[6] Complete when at slow taxi on rwy

[7] Clear rwy if practicable, then shutdown

[8] Don’t taxi on one engine!

[9] On rollout, dead engine into wind.

[10] In air, good engine into wind.

[11] Must complete a SSE Full Stop brief to perform SSE Full Stop Landing.  Initiate from a Case One or Two scenario and only when required in syllabus.

### RETURN TO DUAL ENGINE FLIGHT FOLLOWING SINGLE-ENGINE TRAINING

Note: max torque is 2100ft-lbs for 2 sec.  Never add power to a feathered prop without first placing prop out of feather and allowing it to come up to idle RPM first.  Slowly add power.  Match both engines.  Adjust both power levers and match props to 1900RPM.  Take out trim as needed.

# SSE LANDING PATTERN

Refer to the write-up in the FTI (1-22 to 1-24)

# ELECTRIC SYSTEM

1. Sources of DC power: 1 24V/42A battery, 2 28V/250A starter-generators

2. Sources of AC power: DC power routed through 2 750V-A single-phase inverters.

3. The generator control box: overvoltage protection (31V), undervoltage protection (18V), reverse current protection, & automatic paralleling.

4. The generator control boxes are under the cabin center aisle aft of the main spar.

5. The inverters are located ____________________________________________.

6. The current limiters: tie the main buses & provide fault protection.

7. To get around a faulty master avionics switch, pull the master power CB.

8. Limitations of the generators: 28.25 (+/- 0.8)V.

9. Limitations of the inverters: 114 (+/-7)V, 400 (+/-6)Hz.

10. For APU start, the unit must be able to provide 1000 Amps for 0.1sec and 300Amp continuous load thereafter.

11. A failed Inverter Select Relay Fuse will cause failure of respective AC bus without illum of inverter light (switch over impossible).

12. Illum of INST INV OUT light indicates loss of 26VAC inverter select relay & torquemeters.

13. Procedures for INVERTER failure: [1] Fail Inverter – OFF [2] AC Bus switchover [3] Land ASA practicable

Total ac failure, still have: fuel flow, oil press, oil temp, turn & bank, pitot static, N1, ITT, prop RPM, clocks, NCS-31A, AOA indexers, RADALT, wet compass14. Procedures for GENERATOR failure: OFF, reset momentarily, ON.

15. If generator fails to reset, yet registers normal volts in reset position, indicates failure of generator control.

16. Total loss of power, still have: copilot turn and slip, VSI, altimeters, airspeed indicators, wet compass, N1 and N2, ITT, & clocks.

17.  Dual inverter failure, still have DME, radar, VOR (audio), ADF-LF (audio), xponder, MKR BCNs.  For comm still have, UHF & VHF.

18. Excessive load indications (>1.0): [1] Battery/ammeter – Note.  If charge rate in excess of 30A: [2] Battery – OFF. If loadmeter still over 1.0: [3] Non-essential loads – OFF.  Splits of >0.1 indicate abnormal generator paralleling.

19. If get a GEN OUT light, check Current Limiters: [1] Battery- no discharge, no other equip failures = CL intact. [2] Battery – discharge, no other equip failures = opposite CL failed. [3] Battery – no discharge, other equip failures = same side CL failed.

ELECTRICAL SYSTEM MALFUNCTIONS: refer to NATOPS (15-7 to 15-10)

# STARTER-ASSISTED AIRSTARTS

Don’t airstart if engine died from MOVEOFF – Mechanical malfunction, Overheat condition, Vibration, Explosion, 0 N1, Fire, or Fuel fumes.

1. Power lever (failed engine) – IDLE

2. Prop lever (failed engine) – FEATHER

3. Condition lever (failed engine) – FUEL CUTOFF

4. Cabin Temp Mode – AUTO

5. Vent Blower – AUTO

6. Radar – OFF/STANDBY

7. Anti-ice/deice – AS REQUIRED

8. Firewall valve – OPEN

9. Transfer Pump – AUTO

10. Boost Pump – ON

11. Crossfeed – AUTO

12. Generator (failed engine) – OFF

13. Starter (failed engine) – IGN & ENG START (check annunc light illums)

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 sec.  Op engine ITT may increase approx 50C because of gen loading.

14. Condition lever – LOW IDLE

15. Starter (N1 > 50%) – OFF

16. Prop – UNFEATHER

17. Power – AS REQUIRED

18. Generator – RESET/ON

19. Fuel Control Heat – ON

20. Bleed Air – OPEN

21. Electrical Equipment – AS REQUIRED
22. Condition lever – HIGH IDLE

23. Instruments/Nacelles – CHECKED

### SSE WAVEOFF AT ALTITUDE: allows for safe transition from SSE descending flight to max power, SSE climbing flight; prepares for SSE waveoffs in traffic pattern.

[1] Level off at 1000’ altitude + 800’ (i/e 4800, 5800, 6800), 120K, on a numbered hdg.

[2] Align CDI with hdg (tail at top)

[3] IP will reduce one power lever to IDLE

[4] Power – UP, Rudder – UP, Clean – UP

[5] Complete Emergency Shutdown Checklist

[6] When IP calls ‘Approaching the 180,’ complete the Gear Down Landing Checklist

[7] When checklist complete, immediately start a descending left turn to arrive at the 90 position at 500’& 120K (min 110K).

[8] Continue turn to roll out on final at the head of the CDI, 250’, min 110K (max 120K)

[9] Smoothly place Props – FULL FWD.

[10] When IP calls waveoff:

[a] Power – MAX ALLOW.  Transition to climb, add right rudder, keep ball nearly centered (1/4  to ½ out twds op

eng) while banking 5 deg twds op engine.  Min 102K.  Max 110K (preferred).

[b] Flaps – APPR

[c] Gear – UP (When descent stopped or no possibility of touchdown)

[d] Flaps – UP

[e] Props – 1900RPM (after CP calls ‘Gear Up.’)

DITCHING (SSE)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR (at 300’ AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT) approx 19 – 21 Units AOA.

[4] 91K (No flap = 100K)