FAM-3 Study Guide (21 Apr 02)

Environmental system (NATOPS 2-39 to 2-46)

1. Primary purpose of the environment system is to provide for heating, cooling, and pressurization of the aircraft.

2. Safety valve: completely relieves all pressure differential when pressure control switch in DUMP or switch is in PRESS and aircraft is on the ground.  The LH landing gear safety squat switch opens the valve on the ground.

3. Outflow valve: closed on ground.  Moving the pressure control switch to TEST opens the solenoids, closing the safety valve and giving control of the outflow valve to the controller.  The LH landing gear safety squat switch opens the switch in the air.

4. Excessive pressure bump is controlled by the LH landing gear safety squat switch which: [1] removes power from the preset and safety valve solenoids [2] actuates time delay relay that allows left engine to commence pressurization 6 seconds prior to right engine.

5. Max psid for outflow valve is 4.7psid.

6. Max psid for safety valve is 4.9psid.

7. Actual control of psid in safety valve operation is by the pressure control switch when in DUMP or the LH landing gear safety squat switch when on the ground (the safety valve solenoid opens to supply vacuum to the safety valve holding it in OPEN or DUMP position).

8. Electric heater lockout system: prevents use of electric heater if windshield anti-ice, prop deice, and/or lip boot heaters are operating.

9. Max cooling: [1] Bleed Air Valves – CLOSE one or both [2] Vent Blower – HIGH [3] All overhead outlets – OPEN

Max heat: [1] Electric heat – GND MAX [2] Vent Blower – HIGH [3] All overhead outlets – CLOSE  [4] Bleed Air Valves – OPEN (after engine start)

10. The electric heater will work only when CABIN TEMP MODE is in AUTO.

11. With the vent blower in auto, the fan is operating in low speed.

12. With a total loss of electrical power, the T-44 will depressurize!

13.  None of the pressurization systems work in conjunction with the RIGHT squat switch.

14. Procedures for an altitude warning light:[1] Descend [2] Restore cabin pressurization [3] Check pressurization and/or don O2 mask.

15. Loss of cabin pressurization (NATOPS 15-11):

If gradual pressurization loss:

[1] Cabin altitude – Checked (P)

[2] Pressurization controller – Checked (CP)

[3] Bleed Air – Checked (CP)

[4] Press dump test switch – TEST (CP)  (hold for 15 sec)

If test switch restores press, may be necessary to hold switch in TEST until cabin profile at 10,000’ or less.  If press

is regained through test switch, pull the PRESS CONTROL CB to maintain pressurization.

If unable to restore pressurization:

[5] Oxygen mask/microphone switch (100%) – AS REQ (P,CP,OBS)

[6] Descend – AS REQ (P)

On descent when cabin alt matches press alt, ensure PRESS CONTROL CB is reset to preclude landing

pressurized.

16. Explosive decompression: occurs in less than 1 second, causes fog, if high alt – rush of air from lungs, dazed sensation, gas pains, hypoxia if oxygen not used immediately.

[1] Oxygen mask/ MIC switches (100%) – AS REQ

[2] Descend – AS REQ

17. Supplemental oxygen is required when at cabin altitudes above 10,000’.

18. The oxygen system capacity is 1850 (+/-50) psig for one 49 cubic foot cylinder.

19. Between 17,000’ to 20,000’ the diluter demand regulator should supply 100% oxygen while in the normal position.

20.  The oxygen mask should be stored in the 100% position.

21. The min O2 requirement for local flights is 1000psig.  For cross-country flights: 1500psig.

23. Smoke and fume elimination (NATOPS 15-5): Locate, isolate, and extinguish fire or source of smoke and fumes prior to initiating smoke removal procedures.  Isolate the engine as a possible source by closing the bleed air valves.  Prior to depressurizing, consider MEA and crew O2 requirements.

If immediate smoke removal necessary:

[1] Oxygen masks/MIC switches (100%) – AS REQ

[2] Pressurization – DUMP

After depressurizing, storm windows may be opened (may draw smoke into flight station).  Consider emergency descent and immediate landing.  Nauseating fumes from desalination solution can enter cockpit via bleed air for air-conditioning/pressurization and is often detected immediately after takeoff.  Providing no secondary indications exist, closing the bleed air valves will significantly reduce the fumes.

Setting nonstandard pressurization: [1] Round off given altimeter setting to nearest tenth (exp. 29.77 = 29.8) [2] Subtract that number from 30.4 (30.4 – 29.8 = 0.6) [3] Multiply that number by 1000 (0.6 X 1000 = 600) [4] Add that number to field elvevation (600 + 19 = 619’)  [5] set that number in latitude knob for cabin altitude.

Emergency descent procedure (FTI 1-16, NATOPS 15-12)

[1] Power levers – IDLE

[2] Props – FULL FWD

[3] Flaps – AS REQ

[4] Landing Gear – AS REQ

[5] Airspeed – AS REQ (Don’t exceed VNE (227K))

[6] Windshield heat – AS REQ

LOST COMM PROCEDURES (VFR) (Course Rules III-10, FIH Section A):

[1] Squawk lost comm (7600)

[2] Obtain current ATIS (CH 1 or 114.0 Truax VORTAC)  ATC may attempt to contact you on ATIS.

If unable to obtain ATIS and wx permits, overfly Navy Corpus > 3500’MSL to determine duty rwy.

[3] Intercept course rules via appropiate VFR arrival route.  Make all radio calls “in the blind”  Rock wings inbound for

break and check for interval prior to breaking.

[4] On short final, if no ALDIS lamp signal observed, perform a waveoff.  On 2nd pass, if no lamp signal observed and

rwy is clear, land.

# SMOKE/FIRE OF UNKNOWN ORIGIN (NATOPS 15-4)

The last five items of the checklist: (1) Isolate problem’s source (2) restore power for safety of flight.  Pausing between the last five steps may help isolate the fire if its electrical.

1. Alert Crew

2. Cabin Temp Mode – OFF

3. Vent Blower – AUTO

Note: If fire source is known immediately turn off all affected electrical circuits and fight fire with handheld fire extinguishers.  If fire source cannot be isolated, continue checklist.

4. Oxygen masks/microphone switch (100%) – AS REQ

Note: While using oxygen, if interphones selected, the speaker function will deactivate interphones.  A headset is required to monitor external comm while simultaneously monitoring interphones.

5. Bleed Air – CLOSED

6. Emergency descent – AS REQ (when oxygen provided for entire crew, staying at high altitude and depressurizing may help control fuselage fires.

7. Pressurization – DUMP

8. Emergency transmission – AS REQ

9. Gang Bar – OFF (after securing electric power, the following instruments still available: copilot turn and slip, pitot-static, N1, ITT, prop RPM, clocks, and standby compass)

10. All electrical switches – OFF (If fire can’t be controlled, land or ditch immediately)

11. Battery – ON

12. Generators (one at a time) – ON

13. Inverters (one at a time) – ON

14. Essential equipment – ON (individually until fire source located)

## NO FLAP LANDINGS (FTI 1-19): Same airspeeds and altitudes through the 90, then roll onto final at 110K.  Cross threshold at 105K.