# TAKE OFF BRIEF

## “Back me up on the power quadrant and monitor engine and flight instruments.  Call out any malfunctions.  All emergencies will be handled in accordance with NATOPS.  Call rotate at 91K & note the takeoff time.  Any Questions?”

### TOUCH & GO BRIEF

“Once safely on the deck, I will reset the flaps and advance the power levers to 12o’clock.  You reset the flaps, call go when the engines spool up, and rotate at 91K with takeoff power set.”

#### SSE FULL STOP BRIEF

This will be a simulated single engine full stop landing.  Once safely on the deck, I will pull both power levers over the detent, smoothly apply reverse on the operating engine, countering any swerve with rudder and aileron as necessary to maintain directional control.  As rudder effectiveness is lost, I will return both power levers toward flight idle until directional control is regained.