VFR RADIO COMM

NGP to SEAGULL

-Departing line, CH 3 (GND) “Navy Corpus Ground, Montana (063), taxi from (Echo) line/base of Hangar___, with information(Lima).”

-In Runup Ariea, CH 3 (GND) “Navy Corpus Ground, Montana (063), further taxi.”

-Approaching hold short, CH 4 (TWR) “Navy Corpus Tower, Montana (063), number 1, VFR to (Segull/homefield bounce/Sunrise departure, departure option.

-Initial contact with departure, CH 5 (DEP) “Corpus Departure, Montana (063), passing (500) for (500).

- Approaching Seagull climb radial, CH 5 (DEP) “Corpus Departure, Montana (063), terminate.”  Switch to Seagull CH 17.

# SEAGULL

-Initial contact with Seagull, CH 17 (SEA) “Seagull, Montana (063), on climb radial, passing (3500) for block 3 North.”

-Established in block, CH 17 (SEA) “Seagull, Montana (063), established in block 3 North.”

-Prior to departing area, CH 17 (SEA) “Seagull, Montana (063), request off frequency for 2 mics, will monitor guard.”

- Switch & copy ATIS (CH 1).

- Switch to Cabaniss tower (CH 15)

-Cabaniss tower, CH 15 (CAB) “Cabaniss landing.”  Note rwy in use, alt, # aircraft in pattern (5 max)

- Cabaniss tower, CH 15 (CAB) “ Cabaniss tower, Montana (063), inbound VFR, ETA (15 minutes).”

- Return to Seagull, CH 17 (SEA) “Seagull, Montana (063), back up.”

-Leaving area VFR, CH 17 (SEA) “Seagull, Montana (063), approaching descent radial at (45) DME, leaving block (3 North), terminate.”  Listen for other aircraft on descent radial.  Switch to Corpus Approach CH 5 (APR).

SEAGULL TO CABANISS

- Initial contact with approach, CH 5 (APR) “Corpus Approach, Montana (063).”  Wait for reply, then state “Corpus Approach, Montana (063), on the Corpus Christi (100) radial at (30).  VFR advisories to Pt. (Shamrock), with information (Lima).

- Approaching Pt. Shamrock, CH 5 (APR) “Corpus Approach, Montana (063), terminate.”  Switch to Corpus Tower, CH 4 (TWR).

-Pt. Shamrock, CH 4 (TWR) “Navy Corpus Tower, Montana(063), Point Shamrock, (Touch & Go/Full Stop/Sunrise Transition/ Pt. Sunrise Touch & Go or Full Stop).

# CABANISS

-At Point Sunrise, switch CH 15 (CAB) “Cabaniss Tower, Montana (063), Pt. Sunrise, Touch and Go.”

-At Intitial, CH 15 (CAB) “Cabaniss Tower, Montana (063), intitial runway 13.”

-Over approach end of duty rwy, CH 15 (CAB) “Cabaniss Tower, Montana (063), over numbers for the break.”

-Abeam point of landing, CH 15 (CAB) “Cabaniss Tower, Montana (063), (Lt/Rt 180), 3 down and locked, (touch and go/full stop).”

-Prior to departing Cabaniss, direct CP or OBS to copy ATIS (CH 1 or 114.0)

-Ready to depart Cabaniss, CH 15 (CAB) “Cabaniss Tower, Montana (063), Sunrise on request, (touch and go/full stop).”

-Leaving Cabaniss, contact Corpus Navy Tower when directed, CH 4 (TWR).  Ensure switch to tower prior to Pt. Sunrise.

CABANISS TO NGP

-Contacting Navy Corpus Tower, CH 4 (TWR) “Navy Corpus Tower, Montana (063), inbound Pt. Sunrise, (Touch & Go/Full Stop), with (Lima).”

-When clear of active duty rwy, switch to Ground, CH 3 (GND) and call for taxi “Navy Corpus Ground, Montana (063), clear on (Echo), taxi to (my line/ VT-31 hotspot).