SLOW FLIGHT

(ASS ALT/HDG, PROPS 1900, 150K)

P400

__________________________________

FAP (<174)

GD   (<155)

100K (600-700)

T90REV (30AOB)

__________________________________

FFULL

90K   (800-900) TRIM FULL UP

T90REV (30AOB)

__________________________________

RECOVER

PMAX, FAP, GUP, FUP, 150

# APPR TO STALLS

(ASS ALT/HDG, 150K)

PRE-STALL CHECK

(LACY PC)

Loose Items-Stowed

Alt- 5000 (VMC to deck)

8000 (5000 above clouds)

Cabin Sign – BOTH

Yaw Damp – OFF

Props – 1900

Clear Turn (90 at 30AOB)

Clean Off (COF)

1. Start clear turn, set 400

2. Roll out after 90

3. P – IDLE, Pitch 10 up, Trim to 100K.  Maintain.

4. Stall – Recover

Clean On (CON)

Same as COF, except don’t put pwr IDLE

Dirty Off (DOF)

1. Start clear turn, set 400

2. FAP.  Keep alt!

3. Gear down landing checklist

4. Roll out after 90

5. FD, P – IDLE, Pitch 10 up, Trim to 100K.  Maintain.

6. Stall – Recover

Dirty On (DON)

Same as DOF, except don’t put pwr IDLE

Appr Turn (APT)

1. Start clear turn, set 400

2. FAP.  Keep alt!

3. Gear down landing checklist

4. After 90, contin turn w/ 30AOB

5. P-300, Pitch 10 up, Trim 100K.  Maintain.

6. Stall – Recover

Recovery

“Relax, max, level, ball, 85 & level, flaps, gear, flaps”

1. RELAX, MAX, LEVEL, BALL

2. Incr pitch, accel

3. Once level or climb (>85), FAP

4. GUP

5. FUP

6. Done when in climb on ass hdg > 110K