T-44 Touch and Go Pattern Procedures

1. Takeoff:

a. Taxi onto centerline (set 70-80% N1)

b. Check both sides

c. Release brakes and smoothly add max pwr

d. Add right rudder

e. Monitor torque (lwr red line with props full fwd 1315ft-lbs)

f. Monitor ITT (redline, 790C)

g. Auto feather armed lights illum (passing 90% N1 pwr lvr position)

h. At 91KIAS (VSSE), CP calls “rotate”

i. Smoothly pitch up 7-10 degrees

j. Once positive rate of climb noted, call “gear up.”

k. CP gives confirmation of pos. rate and responds “gear selected up”

l. Once gear indicates up, CP reports “gear up”

2. Departure:

a. Adjust climb pwr (top red line for torque 765C ITT)

b. Once CP reports “gear up,” reduce prop speed (N2) to 1900RPM

c. Pwr is reduced before prop rpm in touch and go pattern

3. Upwind:

a. maintain extended rwy centerline at 300-500’ and max a/s of 140KIAS

b. After receiving tower clnc for downwind, report clear and turn at 300’

4. Crosswind:

a. Max AOB 30 degrees

b. Maintain 130KIAS

c. Passing 600 ft, set approach flaps and adjust pwr to 500-600ft-lbs

d. Level off downwind at 800 ft. with 120KIAS

5. Downwind:

a. Maintain 800 ft and 120KIAS

b. Fly 1 mile abeam by keeping AOA probe on rwy edge (left pattern)

c. Midfield, call for “Gear Down, Landing Checklist”

d. Complete checklist while adding pwr to maintain 120KIAS

e. Respond “down and locked” only when ALL 3 gear are green

6. 180 position (abeam):

a. Complete landing checklist

b. Report gear to tower (nlt the 90)

7. Approach Turn (Base):

a. Start turn abeam approach end

b. Reduce pwr (to300-375 ft-lbs) and roll 15-20 degrees AOB

8. 90 position:

a. 110KIAS, 500ft AGL, perpendicular to rwy

9. Final:

a. Max 30 degrees AOB

b1. (Using Appr Flaps) Arrive at extended rwy centerline at 105KIAS

b2. (Using Appr Flaps) Arrive 1000-1200 feet from threshhold at 250ft and 95KIAS

c. On final, smoothly place props full forward, verify 3 green and gear handle down

d. Respond “3 down and locked, review me complete”

e. CP verifies props and gear and responds “reviewed complete”

f. Land on first one-third of rwy

g. Slowly close pwr levers while bringing nose up in a flare

10. Full stop landing:

a. Once nosewheel on runway, lift power levers over detent and smoothly pull into

reverse range

b. Do not commence After Landing Checklist until clear of runway