FAM-12 Study Guide (17 May 02)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES

__ RADIO CALLS

__ START CHECKLIST

__ BOLDFACE

__ SSE PATTERN

ABORTED TAKEOFF (NATOPS 14-1, FTI 1-4):

[1] NATOPS: When aborting: (1) Announce “ABORT.” (2) Power levers – IDLE.  (3) Reverse – AS REQ.  Warning: If directional control problems while reversing, advance both power levers to flight idle to minimize effects of asymmetric prop reversal.  (4) Brakes – AS REQ.  [2] FTI: If activation of MASTER CAUTION LIGHT, verbal call of “ABORT,” or verbal “simulated low oil pressure, etc...” initiate an ABORT (prior to 91K and before the 2 board) [5] Emergency Shutdown on Deck Checklist

HIGH WORK

TURN PATTERNS: Enter on numbered hdg, maintain 150K.  Lead all turns by 1/3 AOB.

15 AOB, 90 turn (reverse)

30 AOB, 180 turn (reverse)

45 AOB, 360 turn (reverse)

SLOW FLIGHT: Begin on assigned altitude and hdg with props at 1900RPM and 150K.  Maintain altitude throughout maneuver.

[1] Power, set at 400 ft-lbs

[2] Flaps (<174K) – APPR

[3] Gear (<155K) – Gear down, Landing Checklist

[4] Airspeed – stabilize at 100K (600-700ft-lbs)

[5] Turn – 90 both directions at 30AOB

[6] Flaps (<140K) – FULL (balloon, full nose up trim)

[7] Airspeed – stabilize at 90K (800-900ft-lbs)

[8] Turn – 90 both directions at 30AOB

[9] Recover: (1) Power – Max Allowable

(2) Flaps – APPR

(3) Gear – UP

(4) Flaps – UP

(5) Airspeed – 150K

APPROACH TO STALLS (FTI 1-8 to 1-10): Begin on assigned altitude and hdg at 150K.  Calculate stall speeds prior to for clean and dirty stalls prior to performing approach to stalls.

1. Pre-Stall checks: [LACY PC]	[1] Loose Items – Stowed

[2] Altitude – 5000’AGL min (VMC to deck) or 8000’AGL min (5000’ above

clouds)

[3] Cabin Sign – Both

[4] Yaw Damp – OFF

[5] Props – 1900RPM

[6] Clearing Turn – 90 degrees of turn at 30AOB

2. Clean, Power Off (COF)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

3. Clean, Power On (CON)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Pitch 10 degrees up and trim for 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

4. Dirty, Power Off (DOF)

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing

checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K. Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

5. Dirty, Power On (DON)

[1] As soon as clearing turn starts:  [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing

checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Pitch up 10 degrees and trim for 100K.  Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

6. Approach Turn

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing

checklist

[2] After 90 degrees, continue turn with 30 AOB!

[3] Simultaneously: Power – 300ft-lbs & Pitch up 10 degrees.  Trim to 100K.  Maintain 30 AOB and 10 up.

[4] 1st indication of stall – Recover

7. Recovery procedures

[1] MAX, RELAX, ROLL: Max allowable power, Relax back pressure on yoke, Roll wings level (with ball

centered)

[2] Once level or climbing (with 85K or more): Flaps – APPR, Gear – UP, Flaps – UP (For APPR STALLS,

Gear – UP before Flaps – UP)

[3] Maneuver complete when climbing on assigned hdg, > 110K.

*Stall indications: [1] Stall warning horn [2] Wing buffet [3] Uncontrollable loss of altitude [4] Inability to maintain

wings level/ selected roll attitude [5] 25 units AOA

# DYNAMIC ENGINE CUT: simulates engine failure immediately after takeoff with a windmilling prop.  Begin on numbered hdg, 150K, prop sync – OFF.  Maintain level flight prior to setting takeoff attitude.  Min altitude – 5000’ AGL.

# Setup: (3-2-1-P-G) = 300ft-lbs, 2 UP (Takeoff) trim, 1 Prop Sync – OFF, Props – FULL FWD, Gear Down Landing Checklist

Takeoff – at 95K, apply takeoff power and rotate (7-10 degrees nose up).  Maintain hdg.  (Add right rudder as needed)  Once takeoff power set, IP will call rotate.

Above 91K, the IP will pull one pwr lever to IDLE.  (Raise my hand slightly when he pulls the power back.)  Once power is pulled, scan outside on the horizon.  Pick a point (a cloud) to assist in controlling yaw.  Immediately stop yaw using rudder and aileron while lowering the nose to the horizon.  Substantial rudder pressure may be needed.  Bank to a max of 5 degrees into the operating engine.

Recovery: (Power UP, Rudder UP, Clean UP, Speed UP [102K or 110K], Checklist)

1. Power – AS REQ (max on operating engine)

2. Gear – UP

3. Airspeed – AS REQ (at 102K, raise nose to stop altitude loss & accelerate to 110K if possible)

ID failed engine using the engine instruments (torque, ITT, N1,  fuel flow) and rudder.  The foot working hard to maintain hdg is on the same side as the operating engine (Dead Foot (foot back) = Dead Engine).Don’t look at the power levers to determine failed engine (won’t have this luxury in an actual emergency)!

4. Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

“Sir, give me the first 2 items of the dead engine checklist, you have the comms, declare an emergency, this will be a full stop.”

Maneuver is complete when trimmed to 110K (102K min), on takeoff hdg, and Emergency Shutdown Checklist complete.

DITCHING (POWER OFF)

1. Gear – UP

2. Flaps – UP

3. Rate of descent – MUST BE 130K (max glide) by 200’AGL

4. At 200’AGL, Flaps – APPR

5. Bleed off airspeed with slight nose up.

6. Water entry at 90K, 21units AOA, max 500’ rate of descent on VSI.

Notes: Flaps and/or RADALT may be inop.  If no flap, enter water at 100K with max 500’ rate of descent on VSI.   Most essential to be wings level at time of water impact.

DITCHING (SSE)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR (at last 300’)

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT) approx 19 – 21 Units AOA.

[4] 91K (No flap = 100K)

DITCHING (POWER ON)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR (300’ AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT); approx 21U AOA

[4] 90K (No flap = 100K)

# WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1 & FTI 1-25)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Min altitude for IP to intiate SSE waveoff is 200’AGL.

FLAP LANDINGS

[a] Approach 	Final = 105K	Threshold = 95K

[b] Full		Final = 105K	Threshold = 95K   (Select after 90, but before rolling onto final) Additional

power required.

[c] No flap	Final = 110K	Threshold = 105K Slowly close power levers while gradually flaring.  Tends to

balloon and then sink rapidly.

SINGLE ENGINE LANDINGS: refer to SSE PATTERN notes (02May 02)