FAM-2 Study Guide (21 Apr 02)

1. Porpoised Landing: (FTI 1-20): Occurs if nosewheel touches down before mainmounts.  Nose will bounce up and induce an uncontrollable oscillation until airspeed <40-50K.  If porpoising: [1] Power levers – IDLE [2] Maintain flare until oscillation stops [3] Full stop landing.  Waveoff NOT recommended!

2. Crosswind Takeoffs & Landings (NATOPS 7-14,7-16,26-1): [1] Takeoff: Lead with upwind power at beginning of roll.  Keep nosewheel on surface and apply upwind aileron.  Firmly pull off when flying speed achieved.  Initial drift correction by turning into the wind with a shallow bank, then roll wings level into wind. (Refer to fig 26-2)  [2] Landing: No unusual technique.  Use wing low method.  Allow for drift on final so not to overshoot.  Use ailerons and top rudder to counteract drift during flare and touchdown.  Aileron corrections should be maintained during ground roll.

3. TTO/ NATOPS Brief: Find a copy of the expanded NATOPS brief (copy Capt. Sharpe’s copy).

4. Aircraft Operating Limits (airframe) (NATOPS 4-7):

[1] Speed limits (clean): SL to 15,500ft = 227KIAS.  15,500 to 30,000ft = decr. speed 4K for every 1000’ above

15,000’ (Mach limit = 0.48)

[2] Smooth Air (Dirty) 		140 (FULL Flap Max Speed)

[3] Turbulent Air (Dirty)		140 (FULL Flap Max Speed)

[4] Landing Gear (Retract)		145

Landing Gear (Extend)		155

[5] Flaps (Appr/ 35%)		174

Flaps (Full/ 100%)		140

[6] C.G. Limits (@ 9650lbs)		19.4% thru 31.8% M.A.C. [140.6” thru 160.0”]

(@ 9168lbs)		17.3% thru 31.8% M.A.C. [149.0” thru 160.0”]

(@7850lbs)		11.6% thru 31.8% M.A.C. [144.7” thru 160.0”]

(M.A.C. = Mean Aerodynamic Chord)

[7] Max permissable acceleration	+ 3.0g to –1.0g (clean level fight & clean max bank)

(9650 lbs gross weight)		+ 2.0g to –1.0g (flaps extended, dirty flight)

[8] Ceiling (Navy approved)		31,000’

[9] Max allowable gross weight	9650 lbs (field takeoff)

9168 lbs (field landing)

[10] Aerobatic maneuvers & intentional spins are prohibited

5. Fuel System (NATOPS 2-3 to 2-4, 2-11 to 2-17)

1. Total fuel system capacity 387.6 G

2. Total usable fuel 384 G

3. Crossfeed allows use of all tanks to either engine.

4. Fuel is transferred from the wing tanks to the center section tank by gravity.

5. Fuel is transferred to the nacelle tank by the transfer pump.

6. Nacelle float switches: 	High level (59G)– turns transfer pump off if 51 Gal switch fails.

Mid level  (51G)—turns transfer pump off

Low level (49G)—turns transfer pump on

7. In override, the transfer pump runs continuously, the float switches are bypassed, & the 3psi press switch reset

8. With a failed transfer pump, the fuel gets to the nacelle tank by gravity feed.

9. There are 10 fuel drains on the aircraft.

10. A fuel capacitance gauging system is used in the T-44.

11. The only diff between the left and right fuel system: (1) the left has crossfeed (2) the right has thermal relief valve & line from crossfeed system..

12. A minimum of 42 gallons of fuel is req in the Nacelle tank to obtain a good test of the transfer pump.

13. The boost pump provides 30 psi of pressure to the engine driven fuel pump.

14. It takes 5psi of pressure to activate the FUEL PRESS light.

15. It takes a 30 second pressure delay to activate the NO FUEL TRANSFER light.

16. The engine driven fuel pump supplies 800 psi pressure to the engine.

17. The indications of a boost pump failure: with crossfeed in ‘auto’: [1] red ‘FUEL PRESS’ light illums and goes out very quickly [2] yellow ‘FUEL CROSSFEED’ light illums [3] master caution light flashes [4] gradual loss of power on affected engine beginning at 13,000’.

18. The TBO for an engine driven fuel pump operating on suction lift is 10 hours.

19. The TBO for an eng running on AVGAS is 150 hours.

20. The crossfeed is not left open with the boost pumps operating because fuel may be inadvertently crossfed from either fuel system to both engines because of variances in pump pressure

21. Since the crossfeed valve is electrically opened, if it losses total electric power (i/e battery drained and generators out), it will be closed.

22. The firewall valve is not used to secure an engine because severe damage could occur to the engine-driven high pressure fuel pump (while cavitating) because it obtains essential lubrication from fuel flow.

23. Total fuel quantity error in total can be as high as ______ % and in nacelle is ____ %

24. Fuel quantity error can in total can by as high as 100 lbs, while nacelle can only by 50 lbs.

25. The syphon break is located on the outer wing and opens to prevent negative pressure build up in the fuel system (normally caused by thermal expansion).

26. The purpose of the fuel vents are to provide ram air pressure to prevent tanks from collapsing as they lose fuel to the engine.

27. If either fuel bus fails, the boost pump will still work because it’s wired to the hot battery bus.

28. If the transfer pump CB popped, I would get the following indications: red ‘NO FUEL TRANSFER’ light WON’T ILLUM!

29. The min distance the fuel truck can come to the airplane while refueling it is ______ ft.  For the refueling point ______ft.

30. Fuel flow from outboard tanks to the engine: gravity feeds to the center inboard tank, through transfer pump to the nacelle tank, through the boost pump, by the firewall shutoff valve, through the fuel filter, through the fuel flow transmitter, through the fuel heater, through the engine driven fuel pump, controlled by FCU, controlled by start control unit, to the engine fuel nozzles, into the combustion chamber.

# 31. Refueling distances: (1) 10’ from any port of an aircraft (2) 20’ from fuel vehicle or fuel filler point.

# FUEL SYSTEM EMERGENCIES (NATOPS 15-4, 15-6 to 15-7)

1. NO FUEL TRANSFER LIGHT

[1] Check total and nacelle fuel quantity

If no fuel in wing tanks (if nacelle quantity = total quantity):

[2] Transfer pump – OFF

If fuel in wing tanks:

[3] Transfer pump – OVERRIDE

If light remains on:

[4] Transfer pump – OFF (lose 28 Gallons)

[5] Land as soon as practicable

2. CROSSFEED light illumination: illuminates when boost pump fails or engine fails.  If boost pump failure: [1] close crossfeed switch.  If boost pump failed: [1] Failed boost pump – OFF [2] crossfeed – OPEN [3] Land as soon as practicable.  If not a boost pump failure, reset the crossfeed valve to ‘AUTO.’  If engine failure, accomplish EMERGENCY ENGINE SHUTDOWN CHECKLIST.

3. Failure of the engine driven fuel pump – Engine WILL FLAMEOUT.  Accomplish EMERGENCY ENG SHUTDOWN CHECKLIST.

4.  If fuel leak – consider securing electric systems which may contribute to wing fire.  Secure nav lights, strobe lights, fuel vent heaters, and AOA.  Secure inboard wing items with gang bar.  If destination can be reached without dual engines, consider shutting down engine on side of fuel leak.  Secure engine as follows:

[1] Condition Lever – FUEL CUTOFF
[2] Emergency Shutdown Checklist – EXECUTE

5. If fuel syphoning – [1] Airspeed – 140K [2] Land as soon as practicable.  Avoid extreme nose low attitudes.

6. Single engine crossfeed:

[1] Boost pumps – ON

[2] Transfer pumps – AUTO

[3] Crossfeed – OPEN (check crossfeed light on)

[4] Boost pump (nonfeeding tank, tank w/o FUEL PRESS light ON) – OFF

6. Full Flap Landings (FTI 1-19): Select FULL FLAPS after 90, but before rolling onto final at 105K and placing props full forward.  Additional power required because of increased drag.  Cross threshold at 95K.  Slowly close power levers to idle and flare.  Beware of porpoising and/or flat landings.

7. Taxiing (NATOPS 7-11, FTI 1-3): [1] NATOPS: Taxi speed between boundary of beta and reverse. [2] FTI: From a slow taxi, close the power levers and smoothly apply brakes for a brake check.  Further taxi without a radio call can be made in the immediate line area.  Commence turns AFTER moving forward.  Reverse is not required to stop at normal taxi speeds.  If stopping for longer than a few seconds, set the parking brake (except at the position and hold on the runway).  At the intersection of all major taxiways/runways, PAC will call “Clear Right.”

8. Aborting Takeoff (NATOPS 14-1, FTI 1-4): [1] NATOPS: When aborting: (1) Announce “ABORT.” (2) Power levers – IDLE.  (3) Reverse – AS REQ.  Warning: If directional control problems while reversing, advance both power levers to flight idle to minimize effects of asymmetric prop reversal.  (4) Brakes – AS REQ.  [2] FTI: If activation of MASTER CAUTION LIGHT, verbal call of “ABORT,” or verbal “simulated low oil pressure, etc...” initiate an ABORT (prior to 91K and before the 2 board).

9. Takeoff (NATOPS 7-13, FTI 1-4): When approaching hold short line, VFR traffic offset to right and IFR to left.  Once cleared for takeoff, scan for traffic and verbalize “left side clear.”  Complete the last 5 items of the Takeoff checklist while taxiing into position on runway centerline.  Don’t respond “high idle” until N1 64-67%.  Set N1 70-80% when alligned with centerline.  Check wings, fuel caps, fuel quantities and say “checked left.”  Check engine instruments and runway allignment.  [Fuel checks (with N1 at > 70%) and instruments can also be checked on hold short line.]  Release brakes and smoothly apply max allowable power (torque at 1st red line and ITT < 790C).  Autofeather lights should illum as N1 passes through 90%.  Scan airspeed, instruments, and annunciator panel lights.  At 91K, rotate.  Pitch up 7-10 degrees and relieve stick pressure with electric trim.  Once receive 2 positive rates of climb (altimeter and VSI), call “gear up.”  Once CP confirms “gear up,” reduce prop levers to 1900RPM and set power at 2nd redline torque (1520ft0lbs) (power is set before props in touch and go pattern!)

10. Departure (FTI 1-5):

Departure from NGP to Seagull: After CP calls “Gear Up,” reduce props to 1900RPM and set power at 2nd red line torque (765C ITT).  (power is set before props in touch and go pattern!)

Departing NGP on Rwy 13 – level off at 500’ and accelerate to 170K.  Abeam high bridge, turn left to hdg 105 and climb at 150K to 2,500’.  Over beachline, head towards Seagull entry climb radial.

Departure from Seagull to NGP or Cabaniss: Prior to departing area, tell Seagull (CH17) “Seagull, Montana ___, request off frequency for 2 mics, will monitor guard.”  Switch to Cabaniss tower (CH 15) and state “Cabaniss landing.”  Note the rwy landing, altimeter setting, & number of aircraft in pattern (5 max).  Tell Cabaniss “Cabaniss Tower, Montana ____ inbound VFR, ETA (15 minutes).”  Switch to CH 1 and copy ATIS.  Retune Segull (CH 17) and state “Seagull, Montana___, back up.”  If VFR not possible, terminate with Seagull and contact Corpus Approach (CH 6) for IFR clearance.  If leaving area VFR, call Seagull as approaching descent radial and state “Seagull, Montana 063, approaching ____ descent radial at __ DME, leaving block _______.”  Listen for any traffic on decsent radial.  Descend after intercepting descent radial.  Complete the Approach Checklist and brief touch and go procedures.  After departing descent radial, set RADIOS/NAVAIDS as appropriate.  Set CDI to landing runway and hdg bug to wind direction.

Departure from Cabaniss to NGP (FTI 1-25): VFR: Prior to requesting departure, direct CP or OBS to copy ATIS (CH 1) or VOR 114.0.  Place a “Sunrise departure on request, full stop/touch and go.”  If wx marginal, check ATIS continuously or contact METRO (344.6), allow extra fuel for approach (must be on deck prior to either nacelle in yellow arc).  No fuel available at Cabaniss.  Depart in accordance with course rules.  Report “Sunrise for break/full stop/touch and go.”  If instructed base entry, immediate action may be required to arrive at base (90 position) configured to land, at 500’/110K.  Don’t land prior to arresting gear if rigged!  Only slow taxi is allowed over cable.  IFR: If NGP is IFR, place a “GCA-1 on request, full stop” or “Tango-3, (type of approach desired), full stop.”  Allow extra fuel for approach.  Must land with nacelles out of yellow arc.  Departing IFR, squawk assigned code and contact Corpus Approach when directed.  Typical clearance is “Runway heading to 1500’, contact approach on Channel 5.”  Report off Cabaniss, altitude passing and assigned altitude, and then comply with instructions.

11. Level Speed Change (FTI 1-7):

Normal cruise	150	(650-700)

Fast 		170	(900-1000)

Slow     		130	(450-500)

Norm		150	(650-700)

Stabilize momentarily at each new airspeed prior to transitioning to next.  100ft-lb torque differential to expedite changes.

12. Climbs/Descents (FTI 1-6)

Cruise (slow)	130	(450-500)

(normal)	150	(650-700)

(fast)	170	(900-1000)

Climb (SL-10K)	150	Max allow

(10-20K)	130	Max allow

(20-25)	120	Max allow

(25K+)	110	Max allow

VXSE & VX (Angle)	102	Max allow

VY (Rate)		108 	Max allow

VYSE (Rate)	110	Max allow

Constant rate

Climb & Descent	150	As required

*Lead cruise from climb or descents 50-75 feet prior.

13. Turn Pattern (FTI 1-7): Enter on numbered hdg, maintain 150K.  Lead all turns by 1/3 AOB.

15 AOB, 90 turn (reverse)

30 AOB, 180 turn (reverse)

45 AOB, 360 turn (reverse)

14. Slow flight (FTI 1-8): Begin on assigned altitude and hdg with props at 1900RPM and 150K.  Maintain altitude throughout maneuver.

[1] Power, set at 400 ft-lbs

[2] Flaps (<174K) – APPR

[3] Gear (<155K) – Gear down, Landing Checklist

[4] Airspeed – stabilize at 100K (600-700ft-lbs)

[5] Turn – 90 both directions at 30AOB

[6] Flaps – FULL (balloon, trim full nose UP)

[7] Airspeed – stabilize at 90K (800-900ft-lbs)

[8] Turn – 90 both directions at 30AOB

[9] Recover: (1) Power – Max Allowable

(2) Flaps – APPR

(3) Gear – UP

(4) Flaps – UP

(5) Airspeed – 150K

15. Approaches to Stalls (FTI 1-8 to 1-10): Begin on assigned altitude and hdg at 150K.  Calculate stall speeds prior to for clean and dirty stalls prior to performing approach to stalls.

1. Pre-Stall checks: [LACY PC]	[1] Loose Items – Stowed

[2] Altitude – 5000’AGL min (VMC to deck) or 8000’AGL min (5000’ above clouds)

[3] Cabin Sign – Both

[4] Yaw Damp – OFF

[5] Props – 1900RPM

[6] Clearing Turn – 90 degrees of turn at 30AOB

2. Clean, Power Off (COF)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover! (Must add right rudder!)

3. Clean, Power On (CON)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Pitch 10 degrees up and trim for 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover! (Must add right rudder!)

4. Dirty, Power Off (DOF)

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

(airspeed < 155 KIAS)

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN (airspeed < 140 KIAS)

[4] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K. Maintain wings level and 10 up.

[5] 1st indication of stall – Recover (Must add right rudder!)

5. Dirty, Power On (DON)

[1] As soon as clearing turn starts:  [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

(airspeed < 155 KIAS)

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN (airspeed < 140 KIAS)

[4] Pitch up 10 degrees and trim for 100K.  Maintain wings level and 10 up.

[5] 1st indication of stall – Recover (Must add right rudder!)

6. Approach Turn

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR (airspeed < 174 KIAS) [C] Gear down

landing checklist (airspeed < 155 KIAS)

[2] After 90 degrees, continue turn with 30 AOB!

[3] Simultaneously: Power – 300ft-lbs & Pitch up 10 degrees.  Trim to 100K.  Maintain 30 AOB and 10 up.

[4] 1st indication of stall – Recover: Gear is raised before flaps!!! (Must add right rudder!)

7. Recovery procedures

[1] MAX, RELAX, ROLL: Max allowable power, Relax back pressure on yoke, Roll wings level (with ball centered)

[2] Once level or climbing (with 85K or more): Flaps – APPR, Gear – UP, Flaps – UP

[3] Maneuver complete when climbing on assigned hdg, > 110K.

*Stall indications: [1] Stall warning horn [2] Wing buffet [3] Uncontrollable loss of altitude [4] Inability to maintain wings level/ selected roll attitude [5] 25 units AOA

16. Landing Pattern: (FTI 1-17 to 1-20)  Refer to FTI.

17. Approach Flap Landings (FTI 1-19)  Roll onto final at 105K.  Cross threshold at 95K.  Slowly close power levers to idle while flaring when over numbers.

18. Radio Communications (FTI Appendix A & Home Posters) Note: Good discussion of radio comms from Segull to NGP and Cabaniss (FTI 1-17 to 1-18)!

19. Waveoff: (FTI 1-21 to 1-22, NATOPS 7-19)

[1] NATOPS: (Checklist must be committed to memory)

(1) Power – AS REQ (Establish + rate of climb, VX (102K) minimum)

(2) Flaps – APPR

(3) Gear – UP (don’t raise until rate of descent stopped or no possibility of touchdown)

(4) Flaps – UP

(5) Props – 1900RPM (decreases drag & increases climb performance)

[2] FTI: Min altitude for IP initiated waveoff is 100’ AGL.  IP make take controls below 100’AGL.  Waveoff for the following:

[1] Excessive overshoot/ >30AOB during approach turn

[2] Landing clearance not received on short final

[3] IP, wheels watch, tower, or RDO issues visual or verbal waveoff signal (Red ALDIS lamp, Red flare)

[4] 3 green lights not visible rolling onto final

[5] Pilot feels unsafe condition exists

[6] Consider waveoff if can’t touchdown in 1st 1/3 of runway

Waveoff procedures: [1] Call tower and state “Waveoff” [2] Reduce power and level at 500 until cleared downwind [3] Don’t exceed 130K or overtake another aircraft [4] Intiate waveoff by adding power as required & establishing a + rate of climb. [5] Offset from runway [6] Report waveoff and reason to tower.

## FAM-2 Study Questions

1. Main tires should be inflated between 51-57psi.  Nose tires should be inflated between 50-55psi.

2. For soft field landings, tires should be inflated to 30-32psi.

3. Winds above 75K can cause structural damage to the aircraft.

4. Max aft CG for all gross weights is 160 inches.

5. Max airspeed in the T-44 is 227K.  Up to an altitude of 15,500’.  Above 15,500, subtract 4K from max a/s per 1000’.

6. Max takeoff weight 9650 lbs.

7. Max ramp weight 9710 lbs.

8. Max landing weight 9168 lbs.

9. Navy approved service ceiling is 31,000’.

10. The structural service life of each aircraft’s airframe is limited to 15,000 hours and 37,500 landings.

11. The prohibited maneuvers are: [1] Intentional spins [2] Aerobatics.

12. Max G-load:(clean) +3.0g to –1.0g. (dirty) +2.0g to –1.0g.

13. Landing limitations: [1] Flared landings only [2] max rate of descent 600fpm [3]  max crosswind 20K

14. Max crosswind 20K

15. Airspeeds:

Vx	102		VNE	227

VY	108		VA	153

VXSE	102		VLE	155

VYSE	110		VFE35	174

VSSE	  91 		VFE100	140

16. VMCA (NATOPS 24-1): Min speed at which aircraft is directionally controllable.  Based on: Standard day conditions at Sea Level with one engine inop and windmilling, 5 degree bank into good engine, takeoff power set, gear up, flaps up, & most rearward cg. VMCA = 86 KIAS. VMCG = 63 KIAS.