FAM-11 Study Guide (15 May 02)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES

__ RADIO CALLS

__ START CHECKLIST

__ BOLDFACE

INADVERTANT IMC: Must be avoided while VFR.  No set procedures.  Good headwork and situational awareness.  Fly out of the IMC, relying on instrument scan until in VMC again, fly safe altitudes for terrain and obstacle avoidance, get ATC assistance.

COPILOT RESPONSIBILITIES: Refer to NATOPS 22-1 to 22-5

ENGINE FIRE ON DECK: Perform Emergency Shutdown on Deck Checklist

[1] Condition Levers – FUEL CUTOFF

[2] Firewall Valves – CLOSED

[3] Boost pumps – OFF

[4] Fire extinguishers – AS REQ

[5] (Before turning off Gang Bar, make radio call to Tower/Ground alerting them of situation)

Gang Bar – OFF

[6] (Before evacuating aircraft, set the Parking Brake)

Evacuate aircraft (meet 50 meters directly behind the aircraft, avoid firefighting vehicles)

HIGH WORK

TURN PATTERNS: Enter on numbered hdg, maintain 150K.  Lead all turns by 1/3 AOB.

15 AOB, 90 turn (reverse)

30 AOB, 180 turn (reverse)

45 AOB, 360 turn (reverse)

SLOW FLIGHT: Begin on assigned altitude and hdg with props at 1900RPM and 150K.  Maintain altitude throughout maneuver.

[1] Power, set at 400 ft-lbs

[2] Flaps (<174K) – APPR

[3] Gear (<155K) – Gear down, Landing Checklist

[4] Airspeed – stabilize at 100K (600-700ft-lbs)

[5] Turn – 90 both directions at 30AOB

[6] Flaps (<140K) – FULL (balloon, full nose up trim)

[7] Airspeed – stabilize at 90K (800-900ft-lbs)

[8] Turn – 90 both directions at 30AOB

[9] Recover: (1) Power – Max Allowable

(2) Flaps – APPR

(3) Gear – UP

(4) Flaps – UP

(5) Airspeed – 150K

APPROACH TO STALLS (FTI 1-8 to 1-10): Begin on assigned altitude and hdg at 150K.  Calculate stall speeds prior to for clean and dirty stalls prior to performing approach to stalls.

1. Pre-Stall checks: [LACY PC]	[1] Loose Items – Stowed

[2] Altitude – 5000’AGL min (VMC to deck) or 8000’AGL min (5000’ above

clouds)

[3] Cabin Sign – Both

[4] Yaw Damp – OFF

[5] Props – 1900RPM

[6] Clearing Turn – 90 degrees of turn at 30AOB

2. Clean, Power Off (COF)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

3. Clean, Power On (CON)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Pitch 10 degrees up and trim for 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

4. Dirty, Power Off (DOF)

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing

checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K. Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

5. Dirty, Power On (DON)

[1] As soon as clearing turn starts:  [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing

checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Pitch up 10 degrees and trim for 100K.  Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

6. Approach Turn

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing

checklist

[2] After 90 degrees, continue turn with 30 AOB!

[3] Simultaneously: Power – 300ft-lbs & Pitch up 10 degrees.  Trim to 100K.  Maintain 30 AOB and 10 up.

[4] 1st indication of stall – Recover

7. Recovery procedures

[1] MAX, RELAX, ROLL: Max allowable power, Relax back pressure on yoke, Roll wings level (with ball

centered)

[2] Once level or climbing (with 85K or more): Flaps – APPR, Gear – UP, Flaps – UP (For APPR STALLS,

Gear – UP before Flaps – UP)

[3] Maneuver complete when climbing on assigned hdg, > 110K.

*Stall indications: [1] Stall warning horn [2] Wing buffet [3] Uncontrollable loss of altitude [4] Inability to maintain wings level/ selected roll attitude [5] 25 units AOA

# DYNAMIC ENGINE CUT: simulates engine failure immediately after takeoff with a windmilling prop.  Begin on numbered hdg, 150K, prop sync – OFF.  Maintain level flight prior to setting takeoff attitude.  Min altitude – 5000’ AGL.

# Setup: (3-2-1-P-G) = 300ft-lbs, 2 UP (Takeoff) trim, 1 Prop Sync – OFF, Props – FULL FWD, Gear Down Landing Checklist

Takeoff – at 95K, apply takeoff power and rotate (7-10 degrees nose up).  Maintain hdg.  (Add right rudder as needed)  Once takeoff power set, IP will call rotate.

Above 91K, the IP will pull one pwr lever to IDLE.  (Raise my hand slightly when he pulls the power back.)  Once power is pulled, scan outside on the horizon.  Pick a point (a cloud) to assist in controlling yaw.  Immediately stop yaw using rudder and aileron while lowering the nose to the horizon.  Substantial rudder pressure may be needed.  Bank to a max of 5 degrees into the operating engine.

Recovery: (Power UP, Rudder UP, Clean UP, Speed UP [102K or 110K], Checklist)

1. Power – AS REQ (max on operating engine)

2. Gear – UP

3. Airspeed – AS REQ (at 102K, raise nose to stop altitude loss & accelerate to 110K if possible)

ID failed engine using the engine instruments (torque, ITT, N1,  fuel flow) and rudder.  The foot working hard to maintain hdg is on the same side as the operating engine (Dead Foot (foot back) = Dead Engine).Don’t look at the power levers to determine failed engine (won’t have this luxury in an actual emergency)!

4. Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

“Sir, give me the first 2 items of the dead engine checklist, you have the comms, declare an emergency, this will be a full stop.”

Maneuver is complete when trimmed to 110K (102K min), on takeoff hdg, and Emergency Shutdown Checklist complete.

EMERGENCY DESCENT (FTI 1-16, NATOPS 15-12)

[1] Power levers – IDLE

[2] Props – FULL FWD

[3] Flaps – AS REQ

[4] Landing Gear – AS REQ

[5] Airspeed – AS REQ (Don’t exceed VNE (227K))

[6] Windshield heat – AS REQ

# STARTER-ASSISTED AIRSTARTS

Don’t airstart if engine died from MOVEOFF – Mechanical malfunction, Overheat condition, Vibration, Explosion, 0 N1, Fire, or Fuel fumes.

1. Power lever (failed engine) – IDLE

2. Prop lever (failed engine) – FEATHER

3. Condition lever (failed engine) – FUEL CUTOFF

4. Cabin Temp Mode – AUTO

5. Vent Blower – AUTO

6. Radar – OFF/STANDBY

7. Anti-ice/deice – AS REQUIRED

8. Firewall valve – OPEN

9. Transfer Pump – AUTO

10. Boost Pump – ON

11. Crossfeed – AUTO

12. Generator (failed engine) – OFF

13. Starter (failed engine) – IGN & ENG START (check annunc light illums)

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 sec.  Op engine ITT may increase approx 50C because of gen loading.

14. Condition lever – LOW IDLE

15. Starter (N1 > 50%) – OFF

16. Prop – UNFEATHER

17. Power – AS REQUIRED

18. Generator – RESET/ON

19. Fuel Control Heat – ON

20. Bleed Air – OPEN

21. Electrical Equipment – AS REQUIRED
22. Condition lever – HIGH IDLE

23. Instruments/Nacelles – CHECKED

LOST COMM PROCEDURES (VFR) (Course Rules III-10, FIH Section A):

[1] Squawk lost comm (7600)

[2] Obtain current ATIS (CH 1 or 114.0 Truax VORTAC)  ATC may attempt to contact you on ATIS.

If unable to obtain ATIS and wx permits, overfly Navy Corpus > 3500’MSL to determine duty rwy.

[3] Intercept course rules via appropiate VFR arrival route.  Make all radio calls “in the blind”  Rock wings

inbound for break and check for interval prior to breaking.

[4] On short final, if no ALDIS lamp signal observed, perform a waveoff.  On 2nd pass, if no lamp signal

observed and rwy is clear, land.

# WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1 & FTI 1-25)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Min altitude for IP to intiate SSE waveoff is 200’AGL.

FLAP LANDINGS

[a] Approach 	Final = 105K	Threshold = 95K

[b] Full		Final = 105K	Threshold = 95K   (Select after 90, but before rolling onto final) Additional

power required.

[c] No flap	Final = 110K	Threshold = 105K Slowly close power levers while gradually flaring.  Tends to

balloon and then sink rapidly.

SINGLE ENGINE LANDINGS: refer to SSE PATTERN notes (02May 02)