FAM-6 STUDY GUIDE (20 Apr 02)

PIC/Crew Coordination (SSE)

P-Factor: pertinent force after takeoff rotation; causes aircraft to yaw left; caused by imbalance of thrust in props plane of rotation; must add right rudder to compensate for.

VMCA: (86K) airspeed below which it is not possible to maintain hdg despite max rudder deflection and 5 degrees AOB into operating engine.  Set under following conditions: takeoff power on operating engine, critical engine (left) windmilling, flaps at takeoff setting, gear up, 5 AOB into operating engine, max allowable aft CG, full rudder deflection.  Demonstration only flown by instructor.  Speed decreases with altitude.

Flying procedure: Numbered hdg, 150K, prop sync – OFF

[1] Altitude – 8000’AGL

[2] Power – 400ft-lbs

[3] Flaps – APPR

[4] Gear – UP

[5] Props – FULL FWD

[6] Airspeed – 110K

[7] Power – IDLE on left & Max Allowable on right

[8] Attitude – Raise nose so a/s bleeds off at 1K per second, ball centered, trim rudder, bank 5 deg into op eng.  Observe rudder full throw with full rudder applied and significant aileron to maintain 5 degrees bank.  Nose yaws into dead engine (vis, turn needle, RMI) at approx 86K.  Note position of rudder and ailerons.

[9] Recover – Slowly reduce power on op eng and decr AOA.  As airspeed rises above 86K, reapply power to op engine.  At 102K, execute single engine waveoff.  Complete when in a climb , clean, with > 102K.

VSSE: (91K) Single engine safety speed.  Intentionally failing one engine below this speed is prohibited

VMCG: (63K) The min control speed on the ground.  The min speed aircraft can maintain runway centerline (using max rudder) with critical engine windmilling, and opposite engine at full power.  Below 63K, insufficient airflow exists to directionally control the aircraft.

# Landing Gear System (NATOPS 2-27 to 2-32, 16-6 to 16-9)

1. Nosewheel steering actuates nose gear 12 degrees left and 14 degrees right.  (48 degrees either side with braking)

2. The function that up and down limit switches share is deactivates landing gear motor and prevents overtravel of gear.

3. The up limit switch also deactivates the g-meter when the landing gear is extended.

4. The down limit switch also activates the AOA indexer lights.  On the ground, the right main squat switch deactivates the AOA indexer lights.

5. The three up locks all will turn on the red lights in the gear selector handle, illum the wheels up light, and activate the gear warning horn if the gear don’t fully retract.

6. The right up lock also deactivates the prop sync light when the prop sync switch is ON.  If the right up lock has gear fully retracted, the prop sync light will not come on when the prop sync switch in ON.

7. The left up lock also keeps the loading for the electric heat in NORM & GND MAX within slight fluctuation or no significant generator load difference.   If the left up lock has gear fully retracted, switching the electric heat from NORM to GND MAX will yield only a slight or no significant change in generator loading.

8. The nose up lock also deactivates the landing lights.  If up lock has gear fully retracted, no illumination of landing lights and no increase in generator loading when landing lights are turned ON.

9. The right squat switch: [GFISH] on ground

Gear Circuit inop

Flight hour meter inop

Inlet lip heat inop

Stall warning & AOA indexer lights inop

Handle (gear) & hook (downlock) activated

10. The left squat switch: [HHHAP] on ground

Heat for left engine lip boot inop

Heat for AOA sensor – LOW (14V)

Heat – GND MAX

Ambient air solenoid – CLOSED

Pressure control safety valve – OPEN

11. Down lock switches: close when strut fully extends.  When all 3 down lock switches activated (all 3 gear down & locked): [1] WHEELS UP light deactivates [2] Gear horn deactivates [3] 3 green down lights illum

12. The red light in the gear handle will illuminate when: [1] the gear is unlocked, in transit, or gear warning horn activated [2] both power levers retarded below 79%N1 when gear not down and locked [3] pressing the Landing Gear Handle Warning Light Test Button [4] Flaps below APPR & gear is not down and locked.

13. The warning horn silence button and the WHEELS UP light can be turned off in flight (with power retarded, gear up, and flaps up by pressing the WARN HORN SILENCE button.  After silencing the horn, it will remain silent until the flaps are extended or at least one power lever is advanced beyond 79%N1 and then pulled back below 79%N1.  Don’t silence the horn in the traffic pattern or during the final segment of an IAP when potential to land exists.  Only the PIC will cancel the horn.

14. During manual gear extension, the first green down lock indicator to illum will be the nosegear light.

15. The extension time of the gear is 4-6 seconds.  The retraction time of the gear is 5-7 seconds.

16. The prop reverse not ready light will not illum with the landing gear relay CB pulled (during manual emergency extension of gear).

-- The warning horn, wheels up light, and gear handle will all come on when all 3 landing gear struts are not down and locked and: [1] both power levers retarded < 79(+/-2)%N1  OR  [2]Flaps extended beyond approach position (Flaps – FULL)

-- The wheels up light & gear handle light illum when: [1] both power levers retarded < 79(+/-2)%N1 with Gear UP  OR  [2]Flaps extended beyond approach position with Gear UP.

17. It is possible to have a safe gear indication and not have safe gear (such as not having 3 complete tire or wheel assemblies).

18.  The main gear are held down and locked by spring loaded locks. The nose gear is held down by a jackscrew actuator.

19.  The gear are held up by the jackscrew in the actuator.

20. I can tell the difference between an electrical and mechanical malfunction on an unsafe gear by comparing visual indications with cockpit gear indications and gear circuit breaker positions.

Pull the following circuit breakers: [1] Landing gear relay CB [2] Landing gear indicator CB [3] Landing gear motor CB [4] Cockpit gear position indicators.  If during pulling any of the following , the gear indicates safely down and is visually confirmed safely down, land as soon as practicable.  ***Absence of illuminated AOA indexers could indicate premature landing gear motor cutoff which is indicative of an electrical problem.

# SSE FULL STOP LANDING

Upon touchdown:

[1] Power – IDLE

[2] Reverse – AS REQ

[3] Rudder & Braking – AS REQ (counter act yaw)

[4] Scan to end of runway

[5] If yaw excessive, reduce reversing and stop with brakes (don’t lock brakes)

[6] Complete when at slow taxi on rwy

[7] Clear rwy if practicable, then shutdown

[8] Don’t taxi on one engine!

[9] On rollout, dead engine into wind.

[10] In air, good engine into wind.

[11] Must complete a SSE Full Stop brief to perform SSE Full Stop Landing.  Initiate from a Case One or Two scenario and only when required in syllabus.

LANDING GEAR SYSTEM MALFUNCTION & LANDING GEAR EMERGENCY EXTENSION

Refer to NATOPS 16-6 to 16-9

# SSE LANDING PATTERN

Refer to the write-up in the FTI (1-22 to 1-24) and SSE Pattern Notes (30 Apr 02)

# DYNAMIC ENGINE CUT: simulates engine failure immediately after takeoff with a windmilling prop.  Begin on numbered hdg, 150K, prop sync – OFF.  Maintain level flight prior to setting takeoff attitude.

# Setup: (3-2-1-P-G)

*Altitude – Min 5000’AGL

1. Power – 300ft-lbs.  Trim 2 degrees up and don’t retrim until after rotation.  1: Prop Sync – OFF.

Use pitch to maintain attitude as airspeed bleeds off.

3. Props – FULL FWD

4. Gear Down Landing Checklist - Complete.

5. Takeoff – at 95K, apply takeoff power and rotate (7-10 degrees nose up).  Maintain hdg.  (Add right rudder as needed)

IP should call go as a/s reaches 95K.  Once takeoff power set, IP will call rotate.

Above 91K, the IP will pull one pwr lever to IDLE.  (Raise my hand slightly when he pulls the power back.)  Once power is pulled, scan outside on the horizon.  Pick a point (a cloud) to assist in controlling yaw.  Immediately stop yaw using rudder and aileron while lowering the nose to the horizon.  Substantial rudder pressure may be needed.  Bank to a max of 5 degrees into the operating engine.

Recovery: (Power UP, Rudder UP, Clean UP, Speed UP, Checklist)

1. Power – AS REQ (max on operating engine)

2. Gear – UP

3. Airspeed – AS REQ (at 102K, raise nose to stop altitude loss & accelerate to 110K if possible)

ID failed engine using the engine instruments (torque, ITT, N1,  fuel flow) and rudder.  The foot working hard to maintain hdg is on the same side as the operating engine (Dead Foot (foot back) = Dead Engine).Don’t look at the power levers to determine failed engine (won’t have this luxury in an actual emergency)!

4. Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

*Before addressing the dead engine checklist: (1) transfer comms to CP (2) declare an emergency (3) then address Dead Engine Checklist.  Do so by stating, “Sir, you have the comms, declare an emergency, this will be a full stop landing, give me the first two items of the Dead Engine Checklist.”

Maneuver is complete when trimmed to 110K (102K min), on takeoff hdg, and Emergency Shutdown Checklist complete.

# SSE WAVEOFF (NATOPS 16-1 & FTI 1-25)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Min altitude for IP to intiate SSE waveoff is 200’AGL.

### RETURN TO DUAL ENGINE FLIGHT FOLLOWING SINGLE-ENGINE TRAINING

Note: max torque is 2100ft-lbs for 2 sec.  Never add power to a feathered prop without first placing prop out of feather and allowing it to come up to idle RPM first.  Slowly add power.  Match both engines.  Adjust both power levers and match props to 1900RPM.  Take out trim as needed