FAM-8 Study Guide (06 May 02)

***JACKET REVIEW REQUIRED***

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES (CHAIRFLY EACH AT LEAST 3 TIMES)

__ RADIO CALLS

ENGINE FIRE IN FLIGHT (NATOPS 15-4): Illum of FIRE warning light.  Confirm by checking eng instruments and nacelles.  Even if no secondary indications, consider shutdown by EMERGENCY ENGINE SHUTDOWN checklist.

(POWER UP, RUDDER UP, CLEAN UP, SPEED UP, Checklist)

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

“Sir, you have the comms, declare an emergency, this will be a full stop landing.”

[1] Handle emergency [2] Declare emergency [3] Squawk 7700 [4] If fire extinguishes, turn towards nearest suitable landing field and land.  [5] If fire will not extinguish, perform immediate forced landing or power-on ditch.

DITCHING (POWER ON)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR (at last 300’ AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT); (Dual Eng) approx 21 units

AOA.

[4] Dual Engine: 90K (No flap = 100K)

DITCHING (SSE)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR (at last 300’AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT) approx 19 – 21 Units AOA.

[4] 91K (No flap = 100K)

SMOKE AND FUME ELIMINATION (memory item)

Smoke should be isolated by closing the bleed air valve of the eng with the fire.  Prior to depressurizing, consider min safe route altitude (MEA) and crew oxygen requirements.  If immediate, smoke removal necessary:

1. Oxygen masks/MIC switches (100%) – AS REQ

2. Pressurization – DUMP

After depressurization, storm windows can be opened (can draw more smoke in to cockpit).  Consider emergency descent and immediate landing.

HIGH WORK

TURN PATTERNS: Enter on numbered hdg, maintain 150K.  Lead all turns by 1/3 AOB.

15 AOB, 90 turn (reverse)

30 AOB, 180 turn (reverse)

45 AOB, 360 turn (reverse)

SLOW FLIGHT: Begin on assigned altitude and hdg with props at 1900RPM and 150K.  Maintain altitude throughout maneuver.

[1] Power, set at 400 ft-lbs

[2] Flaps (<174K) – APPR

[3] Gear (<155K) – Gear down, Landing Checklist

[4] Airspeed – stabilize at 100K (600-700ft-lbs)

[5] Turn – 90 both directions at 30AOB

[6] Flaps (<140K) – FULL (balloon, full nose up trim)

[7] Airspeed – stabilize at 90K (800-900ft-lbs)

[8] Turn – 90 both directions at 30AOB

[9] Recover: (1) Power – Max Allowable

(2) Flaps – APPR

(3) Gear – UP

(4) Flaps – UP

(5) Airspeed – 150K

APPROACH TO STALLS (FTI 1-8 to 1-10): Begin on assigned altitude and hdg at 150K.  Calculate stall speeds prior to for clean and dirty stalls prior to performing approach to stalls.

1. Pre-Stall checks: [LACY PC]	[1] Loose Items – Stowed

[2] Altitude – 5000’AGL min (VMC to deck) or 8000’AGL min (5000’ above clouds)

[3] Cabin Sign – Both

[4] Yaw Damp – OFF

[5] Props – 1900RPM

[6] Clearing Turn – 90 degrees of turn at 30AOB

2. Clean, Power Off (COF)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

3. Clean, Power On (CON)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Pitch 10 degrees up and trim for 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

4. Dirty, Power Off (DOF)

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K. Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

5. Dirty, Power On (DON)

[1] As soon as clearing turn starts:  [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Pitch up 10 degrees and trim for 100K.  Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

6. Approach Turn

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

[2] After 90 degrees, continue turn with 30 AOB!

[3] Simultaneously: Power – 300ft-lbs & Pitch up 10 degrees.  Trim to 100K.  Maintain 30 AOB and 10 up.

[4] 1st indication of stall – Recover

7. Recovery procedures

[1] MAX, RELAX, ROLL: Max allowable power, Relax back pressure on yoke, Roll wings level (with ball centered)

[2] Once level or climbing (with 85K or more): Flaps – APPR, Gear – UP, Flaps – UP

[3] Maneuver complete when climbing on assigned hdg, > 110K.

*Stall indications: [1] Stall warning horn [2] Wing buffet [3] Uncontrollable loss of altitude [4] Inability to maintain wings level/ selected roll attitude [5] 25 units AOA

# DYNAMIC ENGINE CUT: simulates engine failure immediately after takeoff with a windmilling prop.  Begin on numbered hdg, 150K, prop sync – OFF.  Maintain level flight prior to setting takeoff attitude.

# Setup: (3-2-1-P-G)

*Altitude – Min 5000’AGL

1. Power – 300ft-lbs.  Trim 2 degrees up and don’t retrim until after rotation.  1: Prop Sync – OFF.

Use pitch to maintain attitude as airspeed bleeds off.

3. Props – FULL FWD

4. Gear Down Landing Checklist - Complete.

5. Takeoff – at 95K, apply takeoff power and rotate (7-10 degrees nose up).  Maintain hdg.  (Add right rudder as needed)

IP should call go as a/s reaches 95K.  Once takeoff power set, IP will call rotate.

Above 91K, the IP will pull one pwr lever to IDLE.  (Raise my hand slightly when he pulls the power back.)  Once power is pulled, scan outside on the horizon.  Pick a point (a cloud) to assist in controlling yaw.  Immediately stop yaw using rudder and aileron while lowering the nose to the horizon.  Substantial rudder pressure may be needed.  Bank to a max of 5 degrees into the operating engine.

Recovery: (Power UP, Rudder UP, Clean UP, Speed UP, Checklist)

1. Power – AS REQ (max on operating engine)

2. Gear – UP

3. Airspeed – AS REQ (at 102K, raise nose to stop altitude loss & accelerate to 110K if possible)

ID failed engine using the engine instruments (torque, ITT, N1,  fuel flow) and rudder.  The foot working hard to maintain hdg is on the same side as the operating engine (Dead Foot (foot back) = Dead Engine).Don’t look at the power levers to determine failed engine (won’t have this luxury in an actual emergency)!

4. Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

*Before addressing the dead engine checklist: (1) transfer comms to CP (2) declare an emergency (3) then address Dead Engine Checklist.  Do so by stating, “Sir, you have the comms, declare an emergency, this will be a full stop landing, give me the first two items of the Dead Engine Checklist.”

Maneuver is complete when trimmed to 110K (102K min), on takeoff hdg, and Emergency Shutdown Checklist complete.

WAVEOFF (DUAL ENG & SSE): Min altitude for IP initiated waveoff is 100’ AGL.  IP make take controls below 100’AGL

(1) Power – AS REQ (Establish + rate of climb, VX (102K) minimum)

(2) Flaps – APPR

(3) Gear – UP (don’t raise until rate of descent stopped or no possibility of touchdown)

(4) Flaps – UP

(5) Props – 1900RPM (decreases drag & increases climb performance)

Waveoff for the following:

[1] Excessive overshoot/ >30AOB during approach turn

[2] Landing clearance not received on short final

[3] IP, wheels watch, tower, or RDO issues visual or verbal waveoff signal

[4] 3 green lights not visible rolling onto final

[5] Pilot feels unsafe condition exists

[6] Consider waveoff if can’t touchdown in 1st 1/3 of runway

Waveoff procedures: [1] Call tower and state “Waveoff” [2] Reduce power and level at 500 until cleared downwind [3] Don’t exceed 130K or overtake another aircraft [4] Intiate waveoff by adding power as required & establishing a + rate of climb. [5] Offset from runway [6] Report waveoff and reason to tower.

DITCHING (POWER ON)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR (at last 300’ AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT); (Dual Eng) approx 21 units

AOA.

[4a] Dual Engine: 90K (No flap = 100K)

FLAP LANDINGS

[a] Approach 	Final = 105K	Threshold = 95K

[b] Full		Final = 105K	Threshold = 95K   Select after 90, but before rolling onto final) Additional

power required.

[c] No flap	Final = 110K	Threshold = 105K Slowly close power levers while gradually flaring.  Tends to

ballon and then sink rapidly.

ENGINE FIRE IN FLIGHT STUDY QUESTIONS

1. If the fuselage is on fire, remaining at higher altitudes and/ or depressurizing instead of an immediate descent may help to extinguish the fire by limiting the oxygen (less O2 at higher altitudes) to fuel the fire.

2. Confirm an engine fire by: [1] Visual indications of smoke or flames from nacelle [2] Engine instruments: high ITT, loss of torque [3] Illum of FIRE WARNING annunc light and FAULT WARNING light.

3. If encounter a wing fire: Accomplish EMERGENCY ENGINE SHUTDOWN checklist, then: [1] shutoff fuel and electrical systems that may contribute to fire [2] slip away from the burning wing [3] Individually secure outboard wing electric items (nav lights, strobe lights, fuel vent heaters, AOA sensors, & AOA heating elements) [4] Secure inboard wing instruments with the gang bar [5] consider landing or ditching immediately if fire is severe (can destroy wing spar as quickly as 90 seconds).

4. Fumes can enter the cockpit via bleed air for the air-conditioning/pressurization system.  Closing the bleed air valves will significantly reduce the intensity of the fumes.

5. If Jammed Power Lever: [1] Check all engine instruments and nacelle for abnormal indications.  [2] If no abnormal indications detected, consider keeping the engine running unless controllability a factor.  Land as soon as practicable.  Prior to landing, secure the engine as follows:

[1] Condition lever (affected engine) – FUEL CUTOFF (concur)

[2] Emergency Engine Shutdown Checklist – EXECUTE (P)

Warning: The landing gear warning system won’t function if the power lever is jammed forward of 79 (+/-2)% N1.

6. If Chip Light in flight, perform Emergency Engine Shutdown.  Engine can be relit.

7. If fuel leak – consider securing electric systems which may contribute to wing fire.  Secure nav lights, strobe lights, fuel vent heaters, and AOA.  Secure inboard wing items with gang bar.  If destination can be reached without dual engines, consider shutting down engine on side of fuel leak.  Secure engine as follows:

[1] Condition Lever – FUEL CUTOFF
[2] Emergency Shutdown Checklist – EXECUTE

If fuel syphoning – [1] Airspeed – 140K [2] Land as soon as practicable.  Avoid extreme nose low attitudes.

8. Oil press < 85psi undesirable – tolerated only to complete flight. Monitor engine instruments and nacelles for secondary indications.  Consider shutting down engine and landing as soon as POSSIBLE; otherwise, reduce power on engine and landing as soon as PRACTICABLE.  Oil pressure < 40psi & oil temp >99C unsafe requiring engine shutdown or landing made as soon as possible using min power to sustain flight.  Note both discrepancies on maintenance forms.

9. If Boost Pump Failure:

A boost pump failure with crossfeed in AUTO noted by illum of yellow FUEL CROSSFEED annunc light.  The failed boost pump is identified by momentarily placing crossfeed switch in CLOSED.  The red LH or RH FUEL PRESSURE annunc light will illum indicating the failed boost pump.

1. Failed boost pump – OFF

2. Crossfeed – OPEN

Note: Determination of rang without resulting to suction lift is dependant upon fuel load remaining on side opposite failed boost pump.

3. Land as soon as practicable

Note: If range because of crossfeed operation is critical, suction lift may be used at all cruise altitudes but should be discontinued in favor of crossfeed (boosted pressure) when initiating descent for landing in event of a missed approach.  Boost pump failure during rapid climbout will cause a gradual power loss on affected engine beginning at 13,000 ft.  The higher fuel temp, the lower the altitude the gradual power loss will occur.  Complete power loss will occur if climb is continued.  Once pressure stabilized, max power settings available up to 31,000 ft.  Fuel stabilization occurs after a few min of stabilized cruising ops.  Descents don’t affect engine op.  If eng power loss during climbout or initial phase or cruise because of inop boost pump and a condition of aerated fuel, eng op can be regained by initiating crossfeed, reducing power, and/or descending.  If crossfeed is continued for long periods, a major unbalancing of fuel load will occur and a range loss will be encountered.   Engine-driven fuel pump op without the boost pump is limited to 10 hours (record this time).

10. Prop Deice: Normal amp (14-18A).  Abnormal:

[1] 0 Amps – check position of switch

[2] 0 to 14 Amps – continue ops, if prop imbalance, increase RPM

[3] 18 to23 Amps -- continue ops, if prop imbalance, increase RPM

[4] > 23 Amps – don’t operate the system!

11. If deice boots fail to inflate: operate manually.  If fail to deflate: pull SUR DE-ICE CB and reset as needed to operate boots.

# 12. Loss of Pressurization

# If gradual press loss:

1. Cabin altitude – CHECKED (P)

2. Pressurization controller – CHECKED (CP)

3. Bleed Air – CHECKED (CP)

4. Press dump test switch – TEST (CP) (hold 15 sec)

Note: If activating test switch restores press, it may be necessary to hold switch in ‘TEST’ until cabin alt profile is adjusted to 10,000’ or less.  If press is regained by using the ‘TEST’ switch, pull the ‘PRESS CONTROL’ CB to remove electrical power from the system.

If unable to restore pressurization:

5. Oxygen mask/ microphone switch (100%) – AS REQ

(P, CP, OBS)

6. Descend – AS REQ (P)

Caution: On descent, when cabin alt matches press alt, ensure ‘PRESS CONTROL’ CB is reset to preclude landing pressurized.

# 13. Cabin Door Open Light

1. Ensure all passengers and crewmembers are seated with seatbelts fastened.

2. Depressurize cabin

Warning: Don’t check the door until aircraft depressurized and on the ground!

On the ground, check the security of the door by lifting the cabin door step and checking the position of the arm and plunger.  If it isn’t locked, turn the door handle toward the locked position until positive locking assured.

14. Autopilot disengagement: Automatically by: [1] power failure [2]  vert gyro failure [3] vert gyro fast errect [4] flight control system power loss of CB pops [5] activation of electric elevator trim [6] autopilot trim failure.  Intentionally by: [1] AP/YD disconnect switch (control wheel) [2] Autopilot engage lever to DIS [3] Go-around button (left power lever, yaw damp remains on) [4] Flight director/autopilot CB [5] turn off BATT/GENS (Gang Bar) or Avionic Master Switch

# 15. Inflight Damage (Controlability Check)

Most important concern is maintaining or regaining aircraft control.  Monitor eng and flight instruments for unusual indications.

1. Check controls for freedom and correct response.

2. Prior to landing, perform a landing configuration check above 5,000 ft.  Check for controllability in the landing configuration by slowly decreasing airspeed in 10 knot increments to determine min controllable airspeed for landing.

3. Land as soon as possible with min control movement.

Warning: Make careful consideration before making ANY configuration changes.  Airframe deformation may significantly increase stall speed.  Fly approach a minimum of 10 knots above min controllable speed to provide a safe margin for landing and possible waveoff.

16. Cracked Windshield: [1] Crack on external panel, no immediate action required. Windshield wipers may be damaged.  Heating elements inop at crack.  [2] Crack on inner panel, cabin window, or if crack can’t be determined: gradually descend and slowly depressurize to 2.5psid or less within 10 minutes.  Visibility through window could be significantly impaired.

17. Split Flaps: [1] Visually check flap condition (indicator compared to actual setting) [2] Reposition lever to last flap setting [3] Brief new flap speeds

18. Forced Landing: If suitable altitude, perform a circular pattern to observe surface terrain and wind conditions.  After terrain and wind noted, set up normal pattern and extend gear when landing assured.  Plan to overshoot.  Use flaps as needed.  If positive gear down and locked indication can’t be determined, do a gear up landing.  Night egress: preposition threshold, spar and aft compartment lighting ON.

1. Configuration:  [A] Don’t extend full flaps until landing assured [B] Gear up landing based on typ of touchdown surface

2. Landing Area: Adequate size to land, free of obstacles, smooth.  Cultivated fields are best.  Swamps, bogs, shallow lakes, and forests should be avoided. Land into wind.

3. Landing: Make at slowest speed that retains complete control.  If landing into trees, fly into them; don’t attempt to land on the tops.  (Refer to NATOPS Figure 16-5 on page 16-12 for Emergency Landing Pattern illustration)

ANTI-ICE/DEICE SYSTEM & EMERGENCIES (NATOPS 2-50 to 2-53, 15-11)

Windshield Wipers: Don’t operate on dry glass.

Windshield anti-ice: Temp sensing unit maintains 95F at windshield surface.  Electric heater lockout.  Mag compass will be unreliable when windshield anti-ice in use.  May blur objects.

Powerplant Ice Protection: Mechanically operated ice vanes or electrical lip boot heat.  With ice vanes extended slight decr in engine torque (40-60 ft-lbs) (Range decreases 10-12%)

Fuel System Anti-icing: Oil-to-fuel heat exchanger; operates continuously; no controls; one external fuel vent in each wing for both nacelle and wing tanks (protected from icing by electric heat elements).

Fuel Control Heat: Each FCU pneumatic line protected from ice by electrically heated jackets.  To prevent overheat, FUEL CONTROL HEAT should not be turned on unless cooling air will soon pass over the jackets.

Propeller Electrothermal Deicer System: Deicing boots on each prop blade; outboard and inboard element that receive electrical power through the deicer timer, slip ring, and brush block.; current measured by the prop ammeter.  Sequence: RH OUT, RH IN, LH OUT, LH IN (RORI LOLI); each segment heats for 34+/-6 sec; req 2 minutes for entire cycle; old mechanical timers sequence to next segment (show amperage fluctuation at timer switches to next segment); electrical timers always reset to RH OUT when power applied (no flux in amperage); each blade draws 5 to 6 amps (3 blades working correctly read between 14 to 18 amps).

Surface Deicer System: Ice removed from outboard wing leading edge, horizontal stab, and vertical stab by flexing pneumatic deicer boots.  Engine bleed air from the engine compressor supplies press to inflate boots and vacuum to hold down boots; press regulator protects system from overinflation;  when not operating, a distributor valve applies vacuum to hold boots down; when the DEICE CYCLE switch is toggled to SINGLE or MANUAL, a servo valve changes the inlet boots from vacuum to pressure that allows the boots to inflate; when the solenoid is deenergized, airflow through the valve is cutoff and it discharges out the boots through the intregal check valve until press reaches approx 1-inch Hg, at which time the boots port to vacuum and the remaining air is evacuated; the boots are again held down by vacuum; either engine can provide the bleed air for the system; bleed air passes through an 18-psi regulator and then enters the deicer and vacuum systems; vacuum press is created by the distribution valve and is proportional to pneumatic pressure by the deicer press regulator valve.  Stall speeds significantly increase with the wing deice boots inflated; never cycle the system rapidly, may cause ice to accumulate outside the contour of the inflated boots and prevent ice removal.  In SINGLE, boot inflate for 7 to 8 seconds, then deflate.  In MANUAL, all boots inflate and stay inflated until switch is released.  The boots can’t be overinflated.

Pitot & AOA Heat Systems: On the ground, AOA heating elements pass through a current limiter lowering voltage to 14 volts DC (low heat).  At lift off , a squat switch on the left main gear causes the current limiter to be bypassed and heating elements receive 28 Volts DC (high heat); except for takeoff roll, PITOT heat should not be used on the ground (might overheat because of lack of cooling airflow over heating elements).

Surface Deice Emergency Procedure: If boots fail to inflate, operate in MANUAL.  If boots fail to deflate, pull the SUR DE-ICE CB and reset as necessary to operate boots.