FAM-9 Study Guide (08 May 02)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES (CHAIRFLY EACH AT LEAST 3 TIMES)

__ RADIO CALLS

AIRCRAFT OPERATING LIMITS: Refer to NATOPS Chapter 4, Pocket NATOPS, and Systems notes.

# FLIGHT CONTROL SYSTEM

Primary flight control system: rudder, elevator, and aileron.  Manually operated by mechanical linkage using control wheel for ailerons and elevators and adjustable rudder/brake pedals for rudder.  Trim control through manually actuated cable-drum system.

Control Wheels: Pitch sync on pilot control wheel only.  Go-around switch on CP control wheel only.  Control wheel map lights power through ENG INST CB.

Rudder Pedals: Two positions – fore and aft.

Trim Tabs: Manually actuated and mechanically controlled by cable-drum and jackscrew actuator system.  Elevator trim tab is anti-servo (as elevators displace, trim tab moves in same direction, increases effective control surface area and increases force required to apply it).

Manual Elevator Trim Tab Control: Elevator tab deflection indicated by position arrow.

Electric Elevator Trim: Controlled by dual element thumb switches and trim disconnect switch on control wheel.  CP takes priority.  ELEC TRIM CB.  Operates only when both elements of thumb switch moved.  Electric trim can’t be overridden by manual trim wheel.  Disconnect is a bi-level, momentary push-type switch button.  First depress disconnects autopilot and yaw damp.  Second level disconnects electric trim.

Aileron Trim Control: Adjusts left aileron trim tab.  Relative only, not in degrees.  Full travel 15 +/-2 degrees up or down.

Flight Controls Lock: Don’t tow with rudder lock installed; could result in serious damage to steering linkage.

Wing Flaps: All metal, slot-type, electrically operated, two sections for each wing.  Both sections operate as a single unit, actuated by a separate jackscrew actuator.  Driven through flexible shafts by a single, reversible electric motor mounted on fwd side of the rear spar.  Motor powered by right main bus.  WING FLAP MOTOR CB.  Motor uses dynamic braking, prevents over travel.  Flaps indicated in percent of travel.

Wing Flap Selector Handle: 0% = UP.  35% = APPR.  100% = DOWN.  Limit switches, actuated by right inboard flap control flap travel.  FLAP & STAB CB.  Intermediate flap positions can’t be selected.

Wing Flap Position Indicator: Show percent of travel from 0 to 100.  APPR = 15 degrees.  FULL DOWN = 43 degrees.  FLAP & STAB CB.

Forced Landing: If suitable altitude, perform a circular pattern to observe surface terrain and wind conditions.  After terrain and wind noted, set up normal pattern and extend gear when landing assured.  Plan to overshoot.  Use flaps as needed.  If positive gear down and locked indication can’t be determined, do a gear up landing.  Night egress: preposition threshold, spar and aft compartment lighting ON.

1. Configuration:  [A] Don’t extend full flaps until landing assured [B] Gear up landing based on typ of touchdown surface

2. Landing Area: Adequate size to land, free of obstacles, smooth.  Cultivated fields are best.  Swamps, bogs, shallow lakes, and forests should be avoided. Land into wind.

3. Landing: Make at slowest speed that retains complete control.  If landing into trees, fly into them; don’t attempt to land on the tops.  (Refer to NATOPS Figure 16-5 on page 16-12 for Emergency Landing Pattern illustration)

# FLIGHT CONTROL SYSTEM MALFUNCTION

On Ground Flight Control Failure: Check yaw damp & autopilot off; leave controls in jammed position; call for immediate inspection from maintenance.

Flap System Failure: If flaps inop, land in existing flap configuration.  If split flaps, return flaps to previously selected position.  Pull the wing flap CB to prevent inadvertent flap movement.

In pattern: Select Flaps – APPR.  [2] During “Flaps” call in Gear Down Landing Checklist, check position of flaps on flap indicator.  If indicator doesn’t correspond to position of flap lever: [1] Visually check position of flaps on right and left.  [2a] If flaps selected APPR, indicator shows UP, but visually checked APPR – leave flap lever at APPR and pull WING FLAP CB.  [2b] If flaps selected APPR, indicator shows UP, and flaps visually checked UP – position flap lever UP, pull WING FLAP CB, and brief new speeds (110 on final, 105 over threshold).

HIGH WORK

TURN PATTERNS: Enter on numbered hdg, maintain 150K.  Lead all turns by 1/3 AOB.

15 AOB, 90 turn (reverse)

30 AOB, 180 turn (reverse)

45 AOB, 360 turn (reverse)

SLOW FLIGHT: Begin on assigned altitude and hdg with props at 1900RPM and 150K.  Maintain altitude throughout maneuver.

[1] Power, set at 400 ft-lbs

[2] Flaps (<174K) – APPR

[3] Gear (<155K) – Gear down, Landing Checklist

[4] Airspeed – stabilize at 100K (600-700ft-lbs)

[5] Turn – 90 both directions at 30AOB

[6] Flaps (<140K) – FULL (balloon, full nose up trim)

[7] Airspeed – stabilize at 90K (800-900ft-lbs)

[8] Turn – 90 both directions at 30AOB

[9] Recover: (1) Power – Max Allowable

(2) Flaps – APPR

(3) Gear – UP

(4) Flaps – UP

(5) Airspeed – 150K

APPROACH TO STALLS (FTI 1-8 to 1-10): Begin on assigned altitude and hdg at 150K.  Calculate stall speeds prior to for clean and dirty stalls prior to performing approach to stalls.

1. Pre-Stall checks: [LACY PC]	[1] Loose Items – Stowed

[2] Altitude – 5000’AGL min (VMC to deck) or 8000’AGL min (5000’ above clouds)

[3] Cabin Sign – Both

[4] Yaw Damp – OFF

[5] Props – 1900RPM

[6] Clearing Turn – 90 degrees of turn at 30AOB

2. Clean, Power Off (COF)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

3. Clean, Power On (CON)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Pitch 10 degrees up and trim for 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall – Recover!

4. Dirty, Power Off (DOF)

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Simultaneously: Power – IDLE & Pitch up 10 degrees.  Trim to 100K. Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

5. Dirty, Power On (DON)

[1] As soon as clearing turn starts:  [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

[2] Roll out after 90 degrees

[3] Flaps – FULL DOWN

[4] Pitch up 10 degrees and trim for 100K.  Maintain wings level and 10 up.

[5] 1st indication of stall – Recover

6. Approach Turn

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps – APPR [C] Gear down landing checklist

[2] After 90 degrees, continue turn with 30 AOB!

[3] Simultaneously: Power – 300ft-lbs & Pitch up 10 degrees.  Trim to 100K.  Maintain 30 AOB and 10 up.

[4] 1st indication of stall – Recover

7. Recovery procedures

[1] MAX, RELAX, ROLL: Max allowable power, Relax back pressure on yoke, Roll wings level (with ball centered)

[2] Once level or climbing (with 85K or more): Flaps – APPR, Gear – UP, Flaps – UP (For APPR STALLS, Gear – UP

before Flaps – UP)

[3] Maneuver complete when climbing on assigned hdg, > 110K.

*Stall indications: [1] Stall warning horn [2] Wing buffet [3] Uncontrollable loss of altitude [4] Inability to maintain wings level/ selected roll attitude [5] 25 units AOA

# DYNAMIC ENGINE CUT: simulates engine failure immediately after takeoff with a windmilling prop.  Begin on numbered hdg, 150K, prop sync – OFF.  Maintain level flight prior to setting takeoff attitude.

# Setup: (3-2-1-P-G)

*Altitude – Min 5000’AGL

1. Power – 300ft-lbs.  Trim 2 degrees up and don’t retrim until after rotation.  1: Prop Sync – OFF.

Use pitch to maintain attitude as airspeed bleeds off.

3. Props – FULL FWD

4. Gear Down Landing Checklist - Complete.

5. Takeoff – at 95K, apply takeoff power and rotate (7-10 degrees nose up).  Maintain hdg.  (Add right rudder as needed)

IP should call go as a/s reaches 95K.  Once takeoff power set, IP will call rotate.

Above 91K, the IP will pull one pwr lever to IDLE.  (Raise my hand slightly when he pulls the power back.)  Once power is pulled, scan outside on the horizon.  Pick a point (a cloud) to assist in controlling yaw.  Immediately stop yaw using rudder and aileron while lowering the nose to the horizon.  Substantial rudder pressure may be needed.  Bank to a max of 5 degrees into the operating engine.

Recovery: (Power UP, Rudder UP, Clean UP, Speed UP, Checklist)

1. Power – AS REQ (max on operating engine)

2. Gear – UP

3. Airspeed – AS REQ (at 102K, raise nose to stop altitude loss & accelerate to 110K if possible)

ID failed engine using the engine instruments (torque, ITT, N1,  fuel flow) and rudder.  The foot working hard to maintain hdg is on the same side as the operating engine (Dead Foot (foot back) = Dead Engine).Don’t look at the power levers to determine failed engine (won’t have this luxury in an actual emergency)!

4. Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

*Before addressing the dead engine checklist: (1) transfer comms to CP (2) declare an emergency (3) then address Dead Engine Checklist.  Do so by stating, “Sir, you have the comms, declare an emergency, this will be a full stop landing, give me the first two items of the Dead Engine Checklist.”

Maneuver is complete when trimmed to 110K (102K min), on takeoff hdg, and Emergency Shutdown Checklist complete.

DITCHING (SSE)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR (at last 300’)

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT) approx 19 – 21 Units AOA.

[4] 91K (No flap = 100K)

# WINDMILLING AIRSTART

Use for emergency relight if engine inadvertently shutdown.

1. Power Lever (failed engine) – IDLE

2. Prop lever (failed engine) – FULL FORWARD

3. Condition lever (failed engine) – FUEL CUTOFF

4. Firewall Valve – OPEN

5. Autoignition – ARM

Note: Windmilling airstarts above 20,000 ft or below 2,200 prop RPM may exceed ITT limit.

6. Condition Lever – LOW IDLE

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 seconds

7. Power – AS REQUIRED

8. Generator – RESET/ON

9. Autoignition – OFF

10. Condition lever – HIGH IDLE

RIGHT HAND PATTERN: Same as left pattern except, on downwind, use right wing position half way between wing edge and fuel filler cap for runway displacement.

FLAP LANDINGS

[a] Approach 	Final = 105K	Threshold = 95K

[b] Full		Final = 105K	Threshold = 95K   (Select after 90, but before rolling onto final) Additional

power required.

[c] No flap	Final = 110K	Threshold = 105K Slowly close power levers while gradually flaring.  Tends to

balloon and then sink rapidly.

WAVEOFF (DUAL ENG & SSE): Min altitude for IP initiated waveoff is 100’ AGL.  IP make take controls below 100’AGL

(1) Power – AS REQ (Establish + rate of climb, VX (102K) minimum)

(2) Flaps – APPR

(3) Gear – UP (don’t raise until rate of descent stopped or no possibility of touchdown)

(4) Flaps – UP

(5) Props – 1900RPM (decreases drag & increases climb performance)

Waveoff for the following:

[1] Excessive overshoot/ >30AOB during approach turn

[2] Landing clearance not received on short final

[3] IP, wheels watch, tower, or RDO issues visual or verbal waveoff signal

[4] 3 green lights not visible rolling onto final

[5] Pilot feels unsafe condition exists

[6] Consider waveoff if can’t touchdown in 1st 1/3 of runway

Waveoff procedures: [1] Call tower and state “Waveoff” [2] Reduce power and level at 500 until cleared downwind [3] Don’t exceed 130K or overtake another aircraft [4] Intiate waveoff by adding power as required & establishing a + rate of climb. [5] Offset from runway [6] Report waveoff and reason to tower.

SINGLE ENGINE LANDINGS: refer to SSE PATTERN notes (02May 02)