DYNAMIC ENGINE CUT

Simulate eng fails immed after T/O with windmilling prop.

Begin on assigned hdg, 150K, PROP SYNC – OFF.  Maintain level flight.

1. ALT- min 5000AGL

2. P-300ft-lbs, Trim 2deg UP.  Don’t trim again until after rotation.

3. F-UP

4. G-Down (Landing Checklist complete)

5. PROP- FULL FWD

6. At 95K, rotate and set (7-10 nose up)

Above 91K, IP pull one power level to idle.

Scan outside, pick ref pt, stop yaw while lowering nose.  Substantail rudder required.  Bank max 5deg into op eng

1. P-AS REQ (MAX on op eng)

2. G-UP

3. AS-AS REQ (At 102K, raise nose, accel to 110K)

4. ID failed engine & execute Emergency Eng Shutdown checklist.  After 1st 3 items hold checklist, pull props to 1900RPM, rest max pwr, & continue checklist if fuel or fire related.

5. Transfer comms to CP, declare emerg, address Dead Eng Checklist

6. Maneuver done when: trimmed at 110K (min 102K), T/O hdg, Emerg Shutdown Checklist done.