FAM-7 Study Guide (01 May 02)

PROPELLERS (NATOPS 2-9 to 2-11)

1. Prop characteristics: hydraulic controlled, constant-speed, full-feathering, and reversible.

2. Feathering: feathering springs assisted by centrifugal force applied to the blade shank counterweights.  Moving Prop Lever to FEATHER lifts pilot valve to allow complete dump of high press oil.

Manual feathering: pull prop lever to feather.

Automatic feathering (ARMED): sense loss of torque oil press and feather an unpowered prop – only can occur when: [1]

autofeather – ARMED [2] both power levers > 90 (+/-2)%N1 [3] torque on one engine < 260 (+/-50)lbs.  Before auto prop

feather, interlock disarms autofeather circuit of opposite propeller at 410 +/- 50 ft-lbs.

Feathers for: [1] loss of boosted prop governor oil pressure (engine shutdowns on ground and anytime prop not turning).

Once one prop feathered, other prop must be manually feathered.

Right prop sync may not feather with prop sync on.

3. Low Pitch Blade angles: 	Feathered = 87	Low Pitch = 15	0 Thrust = -5	Reverse = -11	Cruise = 25 to 35

Idle = 15 to –5	Beta = -5 to –11

4. Max Reverse = -5 to –11 at 86% max N1 at 2100 prop RPM

5. Max N1 in BETA _______ (condition levers – LOW IDLE)

6. Centrifugal counterweights, assisted by feathering spring, move blades to low RPM (high pitch) and feather.  Governor boosted oil press moves prop to high RPM (low pitch) hydraulic stop and reverse.  Low pitch by mechanically activated hydraulic stop, causes beta valve on primary gov to stop oil flow to prop dome.  Low pitch blade pos by primary low pitch stop.  Allows blades to rotate beyond low pitch into reverse.  Beta and reverse provided by adjusting low pitch stop, controlled by power lever when aft of idle.  Prop is at low pitch when prop RPM is below selected governor PRM (when prop lever full forward position 2200, but N2 reads 2000).  No high pitch stop so prop can feather.

7. Norm op range of primary gov is 1800-2200RPM.

8. If prop RPM exceeds 2200 by more than 4%, the overspeed gov 2288 +/-40.  Fuel topping gov 2332.

9. In reverse, pneumatic section of primary gov reset to allow max 2100RPM.

10. Prop gov test switch – actuates solenoid so overspeed gov reset to 1900 to 2100

--  Condition levers – HIGH IDLE – Max RPM 2200.

--  Power levers – IDLE & Condition levers – LOW IDLE – Prop RPM 900 to 1100.

--  If prop RPM not btwn 900 to 1100 with power levers – IDLE & condition levers – LOW IDLE, perform low pitch torque check (NATOPS 10-5).

11. Reduction gearbox failure and/or N2 turbine damage occurs at 2420RPM.

12. Prop reverse not ready light: gear down and prop levers not HIGH IDLE.

13. Autofeathering: used only on t/o & landing.  ARM.  Engine loses power, ARMED, power levers > 90 (+/-2)% N1, 410 (+/-50)ft-lbs & 260 (+/-50)ft-lbs torque switches actuated by loss of torque pressure.  Current through autofeather relay to dump valve, releases oil press holding pitch angle on affected prop.  Feathering springs assisted with centrifugal force applied to blade shank counter weights.  TEST pos on switch enables check < 90% N1 on ground only.

14. Syncrophaser master engine = left.  Inflight use only.  Maintains blade positions of both props via magn speed pickup on prop overspeed gov and mag phase pickups on prop deice slip ring adjacent to brush block.  Creates pulse rate, transmitted to actuator motor on right eng.  Motor trims right eng to left.  Trimmer screws in  & out.  Limited to +/- 30 RPM gov setting.   Right prop may not fully (manual) feather with prop sync on. If props won’t sync, actuator at limits – turn prop sync OFF, synch props manually, turn sync ON.  Prop Sync On light when in system in use and landing gear extended.

15. The prop lever controls the primary gov – varies tension on Speeder Spring to reposition Pilot Valve, allows more gov boosted oil into prop dome for (Higher RPM, low pitch) or oil dumped from dome (Lower RPM, higher pitch).

16. Power lever IDLE or above – sets low pitch at 15 blade angle.  Beta = -5.  Max reverse = -11.

17. Fuel topping gov – pneumatic section of primary gov, limits fuel flow to eng, reduces N1 RPM, on at 2332RPM.  Pneumatic line routes bleed air from eng compressor to FCU to primary gov.  If >2332RPM, airbleed orifice of prim gov opened by flyweight to bleed off P3 press through primary gov, closes fuel control bellows, reducing fuel flow.

18. In reverse, the pneumatic sect of primary gov reset to allow max of 2100RPM.

19. A prop would feather in flight if autofeathered armed, power levers > 90%N1, drop in torque below 260ft-lbs on same engine OR prop lever manually pulled to feather.

20. Max N1 acceleration 102.6%.

# PROPELLER MALFUNCTIONS (NATOPS 15-10)

Primary Governor Failure (memory item): feather or overspeed (outside normal limits).  If landing with overspeeding prop carry extra airspeed.

If prop rpm out of normal governor range (1800 – 2200RPM):

[1] Propeller RPM  -- Adjust to Normal RPM.

By prop lever. If norm restored, continue ops.  If norm can’t be obtained:

[2] Power Lever – IDLE (concur)

[3] Propeller Lever – FEATHER (concur)

Notes: Prop RPM > 2420 may result in RGB failure and/or N2 turbine damage.  Engine with disabled prop can provide electric power.  Right prop may not full feather with prop sync ON.

[4] Emergency Engine Shutdown Checklist – AS REQ

Prop Linkage Failure: prop gov ctl linkage fails, prop goes to 2200 or maintains last rpm setting.

[1] Manipulate prop lever to determine if cockpit control lost

[2] If cockpit control lost and RPM in safe limits, match opposite prop speed with uncontrollable prop and land as soon as practicable.

Alternate Prop Feathering: Linkage or gov fails and prop doesn’t feather by itself or normal feathering procedures don’t work, use autofeather.

[1] Power Lever – IDLE (failed propeller) (concur)

[2] Condition Lever – FUEL CUTOFF (concur)

[3] Autofeather – ARM

[4] Power Levers -- > 90% N1 *Don’t pull power lever to IDLE during autofeather, may disarm system.

[5] Emergency Shutdown Checklist – EXECUTE

Simulated Dual Engine Failure: initiated above 91K by IP reducing bother power levers to idle.  Sea Level is bottom of deck.  Select appropriate ditch heading.  Simulated restart on both engines simultaneously.

[1] Clean up (Gear – UP, Flaps – UP)

[2] Commence turn towards coastline, desired hdg, IP assigned hdg while transitioning to 130K (max range) or 102 (max endurance) if restart altitude minimal.

[3] Simultaneously commence Windmilling Airstart Checklist.  Simulate both condition levers fuel cutoff by pointing  at both levers.  Autoignition may be ARMED or starters simulated ON at student’s discretion.

[4] IP will state what engines lightoff.  If restart successful, add power and complete the checklist.  If restart fails, complete the Emergency Engine Shutdown checklist and follow ditching procedures.  IP will reduce props to 1900RPM when simulated feather and may add power to 100ft-lbs.  Stop engine restart attempts at some time during the power off ditch.  Secure the engines by doing at least first 3 items of Emergency Engine Shutdown.  Emphasize proper ditch.

# WINDMILLING AIRSTART

Use for emergency relight if engine inadvertently shutdown.

1. Power Lever (failed engine) – IDLE

2. Prop lever (failed engine) – FULL FORWARD

3. Condition lever (failed engine) – FUEL CUTOFF

4. Firewall Valve – OPEN

5. Autoignition – ARM

Note: Windmilling airstarts above 20,000 ft or below 2,200 prop RPM may exceed ITT limit.

6. Condition Lever – LOW IDLE

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 seconds

7. Power – AS REQUIRED (“Continue the checklist”)

8. Generator – RESET/ON

9. Autoignition – OFF

10. Condition lever – HIGH IDLE

ENGINE FAILURE (SECOND ENGINE)

Don’t feather both props if a windmilling airstart intended.  If all attempts fail, transition to max glide range (130KIAS, gear up, flaps up, prop feathered) or max glide endurance (102KIAS, gear up, flaps up, prop feathered).

If dual failure at low altitude and airspeed, consider engaging both starter switches vice using autoignition.  Battery power to starter is significantly reduced when electric heat or air-conditioner is used.  Neither prop being feathered will significantly reduce glide range and endurance.

No wind glide range is 2nm/1,000 ft altitude.  Subtract 0.2nm per 10 knots headwind.

With dual engine failure, only battery power is available.

DITCHING (POWER OFF)

1. Gear – UP

2. Flaps – UP

3. Rate of descent – 130K (max glide)

4. At 200’AGL, Flaps – APPR

5. Bleed off airspeed with slight nose up.

6. Water entry at 90K, 21units AOA, max 500’ rate of descent on VSI.

Notes: Flaps and/or RADALT may be inop.  If no flap, enter water at 100K with max 500’ rate of descent on VSI.   Most essential to be wings level at time of water impact.

# SSE FULL STOP LANDING

Upon touchdown:

[1] Power – IDLE

[2] Reverse – AS REQ

[3] Rudder & Braking (on the side of the dead engine) – AS REQ (counter act yaw)

[4] Scan to end of runway

[5] If yaw excessive, reduce reversing and stop with brakes (don’t lock brakes)

[6] Complete when at slow taxi on rwy

[7] Clear rwy if practicable, then shutdown

[8] Don’t taxi on one engine!

[9] On rollout, dead engine into wind.

[10] In air, good engine into wind.

[11] Must complete a SSE Full Stop brief to perform SSE Full Stop Landing.  Initiate from a Case One or Two scenario and only when required in syllabus.

# DYNAMIC ENGINE CUT: simulates engine failure immediately after takeoff with a windmilling prop.  Begin on numbered hdg, 150K, prop sync – OFF.  Maintain level flight prior to setting takeoff attitude.

# Setup: (3-2-1-P-G)

*Altitude – Min 5000’AGL

1. Power – 300ft-lbs.  Trim 2 degrees up and don’t retrim until after rotation.  1: Prop Sync – OFF.

Use pitch to maintain attitude as airspeed bleeds off.

3. Props – FULL FWD

4. Gear Down Landing Checklist - Complete.

5. Takeoff – at 95K, apply takeoff power and rotate (7-10 degrees nose up).  Maintain hdg.  (Add right rudder as needed)

IP should call go as a/s reaches 95K.  Once takeoff power set, IP will call rotate.

Above 91K, the IP will pull one pwr lever to IDLE.  (Raise my hand slightly when he pulls the power back.)  Once power is pulled, scan outside on the horizon.  Pick a point (a cloud) to assist in controlling yaw.  Immediately stop yaw using rudder and aileron while lowering the nose to the horizon.  Substantial rudder pressure may be needed.  Bank to a max of 5 degrees into the operating engine.

Recovery: (Power UP, Rudder UP, Clean UP, Speed UP, Checklist)

1. Power – AS REQ (max on operating engine)

2. Gear – UP

3. Airspeed – AS REQ (at 102K, raise nose to stop altitude loss & accelerate to 110K if possible)

ID failed engine using the engine instruments (torque, ITT, N1,  fuel flow) and rudder.  The foot working hard to maintain hdg is on the same side as the operating engine (Dead Foot (foot back) = Dead Engine).Don’t look at the power levers to determine failed engine (won’t have this luxury in an actual emergency)!

4. Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

*Before addressing the dead engine checklist: (1) transfer comms to CP (2) declare an emergency (3) then address Dead Engine Checklist.  Do so by stating, “Sir, you have the comms, declare an emergency, this will be a full stop landing, give me the first two items of the Dead Engine Checklist.”

Maneuver is complete when trimmed to 110K (102K min), on takeoff hdg, and Emergency Shutdown Checklist complete.

# SSE WAVEOFF (NATOPS 16-1 & FTI 1-25)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Min altitude for IP to intiate SSE waveoff is 200’AGL.