NFAM-1 Study Guide (29 May 02)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES (TO CAB & CRP ONLY)

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES

__ RADIO CALLS

__ START CHECKLIST

__ BOLDFACE

__ SSE PATTERN

LOCAL OPERATIONS: Refer to Course Rules (particularly NGP, CRP, & Cabaniss).

NIGHT FLYING ENVIRONMENT: (NATOPS 7-9 & 18-2, FTI 3-1 to 3-3)  Night landing procedures same as day landing procedures.  Keep lighting as low as possible to aid night vision.  Avoid using landing and/or strobe lights in thick haze, smoke, or fog (lights could be reflected back by particles and reduce visibility).  Ensure I have a flashlight with clear lens (bring a red lens for night vision).  Allow eyes minimum of 30 minutes to adapt to darkness.   Maintain continuous scan.  Avoid large rates of descent near the surface.  Recommend setting 1000’ (or 200’ below altitude if operating below 1200’) on RADALT when outside the traffic/instrument pattern.  In the pattern, helpful to set the RADALT bug at 50’ or 100’ in the pattern.

FIELD LIGHTING: (FTI 3-2)  Detailed information on lighting is found in the AIM, FIH, Enroute Supp, IAP, enroute charts, & VFR sectionals.

[1] Taxiway edge lights = blue.  Taxiway centerline lights = green.

[2] Runway edge lights = white.  On instrument runways, amber lights replace white lights on last 2000’, or half runway length, whichever is less.  Green end lights = approach end of runway.  Red end lights = departure end of runway.  Runway lighting spaced at 200’ intervals.  Runway edge lights classified by brightness: High (HIRL), Medium (MIRL), Low (LIRL).  Most lights are adjustable by tower upon request.  Naval air stations have runway waveoff lights (red) controlled by tower.  If red flashing lights on runway, waveoff.

[3] Pilot-controlled lighting: technique for control: Key mic on PCL frequency when over airport or at FAF on final approach (frequencies are listed in Enroute Supp & on IAPs).  Quickly key microphone seven times to turn on (puts lights at high intensity), then adjust intensity as required (5 additional clicks for medium or 3 additional clicks for low).  Lights remain on for 15 minutes after keyed.

[4] Beacons: 	[A] Lighted Land = white, green (or green alone)

[B] Lighted Land Military = white, white, green

[C] Lighted Heliport = white, green, yellow

[D] Lighted Water Airport = white, yellow (or yellow alone)

[5] Visual Approach Slope Indicator (VASI): used at CRP to aid in maintaining glideslope.  Visible 3-5 miles in daytime & up to 20 miles at night.  Most common VASI is 2 bar (set at 3 degrees), often aligned with ILS glideslope.  On glideslope = red over white “you’re alright.”  Below glidslope = red over red “you’re dead.’  Above glideslope = white over white “you’re out-of-sight.”  Navy Corpus uses an optical landing system (OLS/Frensel Lens).  If shooting an instrument approach, sighting the visual landing aids (VASI, runway lights, approach lights, etc...)  are part of the runway environment and can be used to land after reaching the DH/MDA.

AIRCRAFT LIGHTING: (FTI 3-1 to 3-3)

[1] BEFORE START: At “Lights,” turn on Position Lights & Beacon Lights

[2] ENGINE START:  [A] To show lineman what engine is being started, have copilot shine his flashlight on Pilot’s extended fingers.  [B] During the start, have either the CP or the observer shine a light on the engine instruments and pedestal (lights dim when starter is energized).  Be alert for hot start due to increased electric load from lights.

[3] TAXI: [A] When ready to taxi, turn on ice lights or flash taxi light momentarily (tells lineman I’m ready to taxi, illuminates the wings, & alerts other traffic I’m pulling forward).  [B] Once forward of parking spot, secure the ice lights (taxi forward only far enough to test the brakes & release the lineman.  [C] Stay well clear of the taxiway at night.  [D] When ready for ‘further taxi,’ turn on “taxi lights.”  [E] If have to shutdown on taxiway, notify Ground and leave the position lights- ON.  Don’t taxi on one engine!  Have aircraft towed to parking.  Shutdown for engine chip light, low oil pressure (<40psi), fire.  Be extremely cautious when taxiing!  Look for fire bottles, chocks, poles, and edge lights.  Use wing walkers if required.  (When are wing walkers required?)

[4]