NFAM-2 Study Guide (04 Jun 02)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES (TO CAB & CRP ONLY)

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES

__ RADIO CALLS

__ START CHECKLIST

__ BOLDFACE

__ SSE PATTERN

__ LANDING PATTERN (REVIEW CABANISS PATTERN NOTES – 16 APR 02)

LOCAL OPERATIONS: Refer to Course Rules (particularly NGP, CRP, & Cabaniss).

NIGHT FLYING ENVIRONMENT: (NATOPS 7-9 & 18-2, FTI 3-1 to 3-3)  Night landing procedures same as day landing procedures.  Keep lighting as low as possible to aid night vision.  Avoid using landing and/or strobe lights in thick haze, smoke, or fog (lights could be reflected back by particles and reduce visibility).  Ensure I have a flashlight with clear lens (bring a red lens for night vision).  Allow eyes minimum of 30 minutes to adapt to darkness.   Maintain continuous scan.  Avoid large rates of descent near the surface.  Recommend setting 1000’ (or 200’ below altitude if operating below 1200’) on RADALT when outside the traffic/instrument pattern.  In the pattern, helpful to set the RADALT bug at 50’ or 100’ in the pattern.

NIGHT TRAFFIC PATTERN: (FTI 3-3)  When landing, look down entire length of runway.  Make sure to flare sufficiently.  Don’t fixate on a spot directly in front of aircraft.  Land with centerline between my legs.  Maintain centerline.

NIGHT EMERGENCIES: (FTI 3-3)  Scan is paramount.  Direct CP to activate switches (generator, etc...) outside my normal scan pattern.  If become disoriented: Confess (I’m a dumbass & I’m lost!), Climb (if needed to clear terrain or get better comm/navaid reception), Conserve (conserve fuel), Consult (consult bingo/max range charts), Crosscheck (verify Navaids, checking DME & needles), Call (ask ATC for help), Comply (follow the instructions received).

FIELD LIGHTING: (FTI 3-2)  Detailed information on lighting is found in the AIM, FIH, Enroute Supp, IAP, enroute charts, & VFR sectionals.

[1] Taxiway edge lights = blue.  Taxiway centerline lights = green.

[2] Runway edge lights = white.  On instrument runways, amber lights replace white lights on last 2000’, or half runway length, whichever is less.  Green end lights = approach end of runway.  Red end lights = departure end of runway.  Runway lighting spaced at 200’ intervals.  Runway edge lights classified by brightness: High (HIRL), Medium (MIRL), Low (LIRL).  Most lights are adjustable by tower upon request.  Naval air stations have runway waveoff lights (red) controlled by tower.  If red flashing lights on runway, waveoff.

[3] Pilot-controlled lighting: technique for control: Key mic on PCL frequency when over airport or at FAF on final approach (frequencies are listed in Enroute Supp & on IAPs).  Quickly key microphone seven times to turn on (puts lights at high intensity), then adjust intensity as required (5 additional clicks for medium or 3 additional clicks for low).  Lights remain on for 15 minutes after keyed.

[4] Beacons: 	[A] Lighted Land = white, green (or green alone)

[B] Lighted Land Military = white, white, green

[C] Lighted Heliport = white, green, yellow

[D] Lighted Water Airport = white, yellow (or yellow alone)

[5] Visual Approach Slope Indicator (VASI): used at CRP to aid in maintaining glideslope.  Visible 3-5 miles in daytime & up to 20 miles at night.  Most common VASI is 2 bar (set at 3 degrees), often aligned with ILS glideslope.  On glideslope = red over white “you’re alright.”  Below glidslope = red over red “you’re dead.’  Above glideslope = white over white “you’re out-of-sight.”  Navy Corpus uses an optical landing system (OLS/Frensel Lens [FTI 4-54]).  If shooting an instrument approach, sighting the visual landing aids (VASI, runway lights, approach lights, etc...)  are part of the runway environment and can be used to land after reaching the DH/MDA.

AIRCRAFT LIGHTING: (FTI 3-1 to 3-3)

*At night: [1] If moving on ground: Taxi Lights – ON. [2] If stopped on ground: Taxi Lights – OFF.

[1] BEFORE START: At “Lights,” turn on Position Lights & Beacon Lights

[2] ENGINE START:  [A] To show lineman what engine is being started, have copilot shine his flashlight on Pilot’s extended fingers.  [B] During the start, have either the CP or the observer shine a light on the engine instruments and pedestal (lights dim when starter is energized).  Be alert for hot start due to increased electric load from lights.

[3] TAXI: [A] When ready to taxi, turn on ice lights or flash taxi light momentarily (tells lineman I’m ready to taxi, illuminates the wings, & alerts other traffic I’m pulling forward).  [B] Once forward of parking spot, secure the ice lights (taxi forward only far enough to test the brakes & release the lineman.  [C] Stay well clear of the taxiway at night.  [D] When ready for ‘further taxi,’ turn on “taxi lights.”  [E] If have to shutdown on taxiway, notify Ground and leave the position lights- ON.  Don’t taxi on one engine!  Have aircraft towed to parking.  Shutdown for engine chip light, low oil pressure (<40psi), fire.  Be extremely cautious when taxiing!  Look for fire bottles, chocks, poles, and edge lights.  Use wing walkers if required.  (When are wing walkers required?) [F] LANDING:  After landing, turn Strobes – OFF & Landing Lights – OFF as soon as practicable.  Turn Ice Lights – ON prior to turning into parking spot.  Immediately after initiating turn, Landing Lights – OFF & Taxi Lights – OFF to prevent blinding the taxi director.

INSTRUMENT APPROACH: (NATOPS 18-2 to 18-5, FTI 4-32 to 4-61)  IP demo item.  Read information in NATOPS.

ABORTING TAKEOFF: (NATOPS 14-1, FTI 1-4): [1] NATOPS: When aborting: (1) Announce “ABORT.” (2) Power levers – IDLE.  (3) Reverse – AS REQ.  Warning: If directional control problems while reversing, advance both power levers to flight idle to minimize effects of asymmetric prop reversal.  (4) Brakes – AS REQ.  [2] FTI: If activation of MASTER CAUTION LIGHT, verbal call of “ABORT,” or verbal “simulated low oil pressure, etc...” initiate an ABORT (prior to 91K and before the 2 board).

# ENGINE SYSTEM MALFUNCTIONS

# OIL SYSTEM FAILURE

Oil press < 85psi undesirable – tolerated only to complete flight. Monitor engine instruments and nacelles for secondary indications.  Consider shutting down engine and landing as soon as POSSIBLE; otherwise, reduce power on engine and landing as soon as PRACTICABLE.  Oil pressure < 40psi & oil temp >99C unsafe requiring engine shutdown or landing made as soon as possible using min power to sustain flight.  Note both discrepancies on maintenance forms.

# CHIP DETECTOR LIGHT ILLUMINATION

Indicates metal particles in prop reduction gearbox.  Perform EMERGENCY SHUTDOWN CHECKLIST.

# EMERGENCY ENGINE SHUTDOWN ON DECK (for FIRE, FIRE WARN LIGHT, CHIP LIGHT)

1. Condition Levers – FUEL CUTOFF

2. Firewall Valves – CLOSED

3. Boost Pumps – OFF

4. Fire Extinguisher – AS REQUIRED

5. Gang Bar – OFF

6. Evacuate aircraft

ABNORMAL START (for HOT START & NO LIGHT OFF)

If ITT likely to exceed 925C or no rise in ITT within 10 sec after condition levers LOW IDLE:

1. Condition Lever – FUEL CUTOFF (ITT < 790C)

2. Starter – OFF

Caution: Starter time limited to 40 sec ON, 60 sec OFF, 40 sec ON, 60 sec OFF, 40 sec ON, 30 minutes OFF.

Note: If start attempt discontinued and another start attempted, allow 60 sec delay to drain fuel and cool starter, then motor up starter for 15 sec minimum.  Allow eng to completely stop before attempting another start.

# ABORT & ENG FAILURE DURING TAKEOFF

1. Announce -- “ABORT” (Initiate < 91KIAS & before the 2 boards)

2. Power Levers – IDLE

3. Reverse – AS REQUIRED (then beta range)

4. Brakes – AS REQUIRED (smooth, even, don’t lock)

Prior to departing prepared surface:

(verbally recite remaining items)

5. Condition Levers – FUEL CUTOFF

As soon as practicable:

6. Firewall valves – CLOSED

7. Boost Pumps – OFF

8. Fire Extinguishers – AS REQUIRED

9. Gang bar – OFF

10. Evacuate aircraft

Warning: If direction control problems occur while reversing, advance both power levers to FLIGHT IDLE.  Maintain directional control with rudder, nosewheel steering, and by pumping brakes.

Warning: Accelerate-stop distances increased 900 ft with condition levers HIGH IDLE and no reverse utilized.

# ENGINE FAILURE AFTER TAKEOFF

If sufficient runway to stop on ground, ABORT!

If insufficient runway to abort takeoff:

(POWER UP, RUDDER UP, CLEAN UP)

1. Power – AS REQUIRED

2. Gear – UP

3. Airspeed – (102 (obstacles) or 110)

4. EMERGENCY SHUTDOWN CHECKLIST

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

Note: address first 2 items of dead engine checklist, transfer comm to CP and have him declare an emergency.

Note: Dynamic Engine Cut complete when trimmed at 110KIAS (min 102KIAS), on takeoff heading, and Emergency Shutdown Checklist complete.

## INFLIGHT EMERGENCIES

Note: 1. Rudder trim sufficient above 100KIAS.  Below 100KIAS, supplement full rudder trim with constant rudder pressure.

2. SSE waveoff with full flaps, result in loss of 200 feet before positive rate of climb established.

3. Impending engine failure or flameout preceded by:

[1] unstable engine operation

[2] fluctuating turbine RPM

[3] torque and ITT temp changes

[4] illumination of fuel warning lights

[5] drop in oil pressure

[6] loss of thrust

4. DON’T attempt an airstart if: [MOVEOFF]

Mechanical malfunction

Overheating

Vibration

Explosion

O N1

Fire

Fuel Fumes

# EMERGENCY SHUTDOWN CHECKLIST

(POWER UP, RUDDER UP, CLEAN UP)

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

AIRSTARTS

Accomplished with battery or generator.  Normally starter-assisted.  Determine cause for engine failure before airstart.  Airstarts above 20,000 ft are hotter – periodically cycle condition lever to FUEL CUTOFF to avoid overtemping.  Reduce electric loads.

# STARTER-ASSISTED AIRSTARTS

Don’t airstart if engine died from MOVEOFF – Mechanical malfunction, Overheat condition, Vibration, Explosion, 0 N1, Fire, or Fuel fumes.

1. Power lever (failed engine) – IDLE

2. Prop lever (failed engine) – FEATHER

3. Condition lever (failed engine) – FUEL CUTOFF

4. Cabin Temp Mode – AUTO

5. Vent Blower – AUTO

6. Radar – OFF/STANDBY

7. Anti-ice/deice – AS REQUIRED

8. Firewall valve – OPEN

9. Transfer Pump – AUTO

10. Boost Pump – ON

11. Crossfeed – AUTO

12. Generator (failed engine) – OFF

13. Starter (failed engine) – IGN & ENG START (check annunc light illums)

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 sec.  Op engine ITT may increase approx 50C because of gen loading.

14. Condition lever – LOW IDLE

15. Starter (N1 > 50%) – OFF

16. Prop – UNFEATHER

17. Power – AS REQUIRED

18. Generator – RESET/ON

19. Fuel Control Heat – ON

20. Bleed Air – OPEN

21. Electrical Equipment – AS REQUIRED
22. Condition lever – HIGH IDLE

23. Instruments/Nacelles – CHECKED

# WINDMILLING AIRSTART

Use for emergency relight if engine inadvertently shutdown.

1. Power Lever (failed engine) – IDLE

2. Prop lever (failed engine) – FULL FORWARD

3. Condition lever (failed engine) – FUEL CUTOFF

4. Firewall Valve – OPEN

5. Autoignition – ARM

Note: Windmilling airstarts above 20,000 ft or below 2,200 prop RPM may exceed ITT limit.

6. Condition Lever – LOW IDLE

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 seconds

7. Power – AS REQUIRED

8. Generator – RESET/ON

9. Autoignition – OFF

10. Condition lever – HIGH IDLE

ELECTRICAL SYSTEM MALFUNCTIONS: refer to NATOPS (15-7 to 15-10)

# PROPELLER MALFUNCTIONS (NATOPS 15-10)

Primary Governor Failure (memory item): feather or overspeed.  If landing with overspeeding prop carry extra airspeed.

If prop rpm out of normal governor range (1800 – 2200RPM):

[1] Propeller RPM  -- Adjust to Normal RPM.

By prop lever. If norm restored, continue ops.  If norm can’t be obtained:

[2] Power Lever – IDLE (concur)

[3] Propeller Lever – FEATHER (concur)

Notes: Prop RPM > 2420 may result in RGB failure and/or N2 turbine damage.  Engine with disabled prop can provide electric power.  Right prop may not full feather with prop sync ON.

[4] Emergency Engine Shutdown Checklist – AS REQ

Prop Linkage Failure: prop gov ctl linkage fails, prop goes to 2200 or maintains last rpm setting.

[1] Manipulate prop lever to determine if cockpit control lost

[2] If cockpit control lost and RPM in safe limits, match opposite prop speed with uncontrollable prop and land as soon as practicable.

Alternate Prop Feathering: Linkage or gov fails and prop doesn’t feather by itself or normal feathering procedures don’t work, use autofeather.

[1] Power Lever – IDLE (failed propeller) (concur)

[2] Condition Lever – FUEL CUTOFF (concur)

[3] Autofeather – ARM

[4] Power Levers -- > 90% N1 *Don’t pull power lever to IDLE during autofeather, may disarm system.

[5] Emergency Shutdown Checklist – EXECUTE

FLAP LANDINGS

[a] Approach 	Final = 105K	Threshold = 95K

[b] Full		Final = 105K	Threshold = 95K   (Select after 90, but before rolling onto final) Additional

power required.

[c] No flap	Final = 110K	Threshold = 105K Slowly close power levers while gradually flaring.  Tends to

balloon and then sink rapidly.

# WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1 & FTI 1-25)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Min altitude for IP to intiate SSE waveoff is 200’AGL.