NFAM-1 Study Guide (29 May 02)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES (TO CAB & CRP ONLY)

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES

__ RADIO CALLS

__ START CHECKLIST

__ BOLDFACE

__ SSE PATTERN

__ LANDING PATTERN (REVIEW CABANISS PATTERN NOTES – 16 APR 02)

LOCAL OPERATIONS: Refer to Course Rules (particularly NGP, CRP, & Cabaniss).

NIGHT FLYING ENVIRONMENT: (NATOPS 7-9 & 18-2, FTI 3-1 to 3-3)  Night landing procedures same as day landing procedures.  Keep lighting as low as possible to aid night vision.  Avoid using landing and/or strobe lights in thick haze, smoke, or fog (lights could be reflected back by particles and reduce visibility).  Ensure I have a flashlight with clear lens (bring a red lens for night vision).  Allow eyes minimum of 30 minutes to adapt to darkness.   Maintain continuous scan.  Avoid large rates of descent near the surface.  Recommend setting 1000’ (or 200’ below altitude if operating below 1200’) on RADALT when outside the traffic/instrument pattern.  In the pattern, helpful to set the RADALT bug at 50’ or 100’ in the pattern.

NIGHT TRAFFIC PATTERN: (FTI 3-3)  When landing, look down entire length of runway.  Make sure to flare sufficiently.  Don’t fixate on a spot directly in front of aircraft.  Land with centerline between my legs.  Maintain centerline.

NIGHT EMERGENCIES: (FTI 3-3)  Scan is paramount.  Direct CP to activate switches (generator, etc...) outside my normal scan pattern.  If become disoriented: Confess (I’m a dumbass & I’m lost!), Climb (if needed to clear terrain or get better comm/navaid reception), Conserve (conserve fuel), Consult (consult bingo/max range charts), Crosscheck (verify Navaids, checking DME & needles), Call (ask ATC for help), Comply (follow the instructions received).

FIELD LIGHTING: (FTI 3-2)  Detailed information on lighting is found in the AIM, FIH, Enroute Supp, IAP, enroute charts, & VFR sectionals.

[1] Taxiway edge lights = blue.  Taxiway centerline lights = green.

[2] Runway edge lights = white.  On instrument runways, amber lights replace white lights on last 2000’, or half runway length, whichever is less.  Green end lights = approach end of runway.  Red end lights = departure end of runway.  Runway lighting spaced at 200’ intervals.  Runway edge lights classified by brightness: High (HIRL), Medium (MIRL), Low (LIRL).  Most lights are adjustable by tower upon request.  Naval air stations have runway waveoff lights (red) controlled by tower.  If red flashing lights on runway, waveoff.

[3] Pilot-controlled lighting: technique for control: Key mic on PCL frequency when over airport or at FAF on final approach (frequencies are listed in Enroute Supp & on IAPs).  Quickly key microphone seven times to turn on (puts lights at high intensity), then adjust intensity as required (5 additional clicks for medium or 3 additional clicks for low).  Lights remain on for 15 minutes after keyed.

[4] Beacons: 	[A] Lighted Land = white, green (or green alone)

[B] Lighted Land Military = white, white, green

[C] Lighted Heliport = white, green, yellow

[D] Lighted Water Airport = white, yellow (or yellow alone)

[5] Visual Approach Slope Indicator (VASI): used at CRP to aid in maintaining glideslope.  Visible 3-5 miles in daytime & up to 20 miles at night.  Most common VASI is 2 bar (set at 3 degrees), often aligned with ILS glideslope.  On glideslope = red over white “you’re alright.”  Below glidslope = red over red “you’re dead.’  Above glideslope = white over white “you’re out-of-sight.”  Navy Corpus uses an optical landing system (OLS/Frensel Lens [FTI 4-54]).  If shooting an instrument approach, sighting the visual landing aids (VASI, runway lights, approach lights, etc...)  are part of the runway environment and can be used to land after reaching the DH/MDA.

AIRCRAFT LIGHTING: (FTI 3-1 to 3-3)

[1] BEFORE START: At “Lights,” turn on Position Lights & Beacon Lights

[2] ENGINE START:  [A] To show lineman what engine is being started, have copilot shine his flashlight on Pilot’s extended fingers.  [B] During the start, have either the CP or the observer shine a light on the engine instruments and pedestal (lights dim when starter is energized).  Be alert for hot start due to increased electric load from lights.

[3] TAXI: [A] When ready to taxi, turn on ice lights or flash taxi light momentarily (tells lineman I’m ready to taxi, illuminates the wings, & alerts other traffic I’m pulling forward).  [B] Once forward of parking spot, secure the ice lights (taxi forward only far enough to test the brakes & release the lineman.  [C] Stay well clear of the taxiway at night.  [D] When ready for ‘further taxi,’ turn on “taxi lights.”  [E] If have to shutdown on taxiway, notify Ground and leave the position lights- ON.  Don’t taxi on one engine!  Have aircraft towed to parking.  Shutdown for engine chip light, low oil pressure (<40psi), fire.  Be extremely cautious when taxiing!  Look for fire bottles, chocks, poles, and edge lights.  Use wing walkers if required.  (When are wing walkers required?) [F] LANDING:  After landing, turn Strobes – OFF & Landing Lights – OFF as soon as practicable.  Turn Ice Lights – ON prior to turning into parking spot.  Immediately after initiating turn, Landing Lights – OFF & Taxi Lights – OFF to prevent blinding the taxi director.

DITCHING (POWER ON)

Complete ‘DITCHING’ checklist on back of cockpit card.  Note: Don’t unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don’t remove raft from carrying case inside aircraft.  Don’t inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells – on crest [2] if perpendicular to swells – on back side.

If power available:

[1] Gear – UP

[2] Flaps – APPR

[3] Rate of descent, 100fpm during final stages of approach (last 300’ using RADALT); (Dual Eng) approx

21 units

AOA, (Single Eng) approx 19 – 21 Units AOA.

[4a] Dual Engine: 90K (No flap = 100K)

[4b]  Single Engine: 91K (No flap = 100K)

DITCHING (POWER OFF)

1. Gear – UP

2. Flaps – UP

3. Rate of descent – 130K (max glide)

4. At 200’AGL, Flaps – APPR

5. Bleed off airspeed with slight nose up.

6. Water entry at 90K, 21units AOA, max 500’ rate of descent on VSI.

Notes: Flaps and/or RADALT may be inop.  If no flap, enter water at 100K with max 500’ rate of descent on VSI.   Most essential to be wings level at time of water impact.

PORPOISED LANDING: (FTI 1-20): Occurs if nosewheel touches down before mainmounts.  Nose will bounce up and induce an uncontrollable oscillation until airspeed <40-50K.  If porpoising: [1] Power levers – IDLE [2] Maintain flare until oscillation stops [3] Full stop landing.  Waveoff NOT recommended!

INSTRUMENT APPROACH: (NATOPS 18-2 to 18-5, FTI 4-32 to 4-61)  IP demo item.  Read information in NATOPS.

ABORTING TAKEOFF: (NATOPS 14-1, FTI 1-4): [1] NATOPS: When aborting: (1) Announce “ABORT.” (2) Power levers – IDLE.  (3) Reverse – AS REQ.  Warning: If directional control problems while reversing, advance both power levers to flight idle to minimize effects of asymmetric prop reversal.  (4) Brakes – AS REQ.  [2] FTI: If activation of MASTER CAUTION LIGHT, verbal call of “ABORT,” or verbal “simulated low oil pressure, etc...” initiate an ABORT (prior to 91K and before the 2 board).

FLAP LANDINGS

[a] Approach 	Final = 105K	Threshold = 95K

[b] Full		Final = 105K	Threshold = 95K   (Select after 90, but before rolling onto final) Additional

power required.

[c] No flap	Final = 110K	Threshold = 105K Slowly close power levers while gradually flaring.  Tends to

balloon and then sink rapidly.

# WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1 & FTI 1-25)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Min altitude for IP to intiate SSE waveoff is 200’AGL.

BRAKE EMERGENCIES: (NATOPS 13-1 to 13-2)  Practice Item!

[1] BRAKE FIRE

*1.  Firefighting assistance – REQUEST

*2.  Stop aircraft

*3.  Emergency Engine Shutdown on Deck

[1] Condition Levers – FUEL CUTOFF

[2] Firewall Valves – CLOSED

[3] Boost pumps – OFF

[4] Fire extinguishers – AS REQ

[5] Gang Bar – OFF

[6] Evacuate aircraft

Warning: Stay well away from aircraft (preferably well behind incase wheel explodes sideways).  Don’t fight the fire!

[2] LOSS OF BRAKES

[1] Maintain directional control with rudder, nosewheel steering, or differential power. [2]Use prop reversing or beta to decelerate. [3] If possible, maneuver to open area and let aircraft stop [3] Don’t taxi! [4] Try to reseat shuttle valve by pulling aft on the top of the brake pedals.

[3] HOT BRAKES

[1] Stop aircraft by reverse thrust and minimal braking

[2] Request assistance, if needed

[3] Cool brakes with propwash

[4] Inspect brakes before using them again.

Caution: Brakes can lock during max braking, which may blow tires.