IFR RADIO COMMUNICATIONS

After Start Checklist  (‘Radar – OFF’): “Hold checklist.”

[1] Copy ATIS

[2] Call CLNC DEL (CH. 2): “Navy Corpus Clearance Delivery, N1G___, GCA on request, 6 GCAs.”

[3] Call GROUND (CH. 3): “Navy Corpus Ground, N1G___, taxi from my line to the runup area with information _____.”

Takeoff Checklist (at ‘Transponder’ in RADIO NAVAIDS): Call CLNC DEL (CH.2): “Navy Corpus Clearance Delivery, N1G___, ready to copy clearance.”

Takeoff Checklist (‘Crew – BRIEFED’): Call GROUND (CH. 3): “Navy Corpus Ground, N1G___, further taxi.”

Prior to reaching hold short (at Yankee taxiway for 13), switch to TOWER (CH. 4).

At hold short, call TOWER (CH. 4): “ Navy Corpus Tower, N1G___, number 1, holding short, IFR release.”

After takeoff (when directed by tower) contact DEPARTURE (CH. 5): “Corpus Departure, N1G___, off Navy Corpus, passing (current altitude) for (assigned altitude).”

After landing, when clear of duty runway, switch from TOWER to GROUND (CH. 3): “ Navy Corpus Ground, N1G___, clear on (Echo), taxi to my line.”