RI-4 Study Guide (12 Jun 02)

__ IFR COMM (APPX. A)

__ IFR BRIEFS (APPX. A & AIGT CHAP. 8)

__ LOCAL DEPARTURE INSTRUCTIONS (BLUE BRAINS)

__ CHECKLIST PROCEDURES (PARTICULARLY ENGINE RADIO/NAVAIDS & ENGINE RUN-UP)

PILOT CONTROLLED LIGHTING: technique for control: Key mic on PCL frequency when over airport or at FAF on final approach (frequencies are listed in Enroute Supp & on IAPs).  Quickly (within 5 seconds) key microphone seven times to turn on (puts lights at high intensity), then adjust intensity as required (5 additional clicks for medium or 3 additional clicks for low).  Lights remain on for 15 minutes after keyed.

CIRCLING: 120K for dual and single engine.  Circling MDA and wx mins are for runway approach is flown to.  Circling mins are for non-radar non-precision approaches.  Don’t circle from a precision approach.  Report “commencing circle” when initiating any circling maneuver.  T-44 is Category B (approach speeds 91-120K).  If need to maneuver at airspeeds >120K, use Category C mins.  When requesting an ASR circling approach, state aircraft category (B) to controller.  When circling, keep runway environment in sight.  Circle in what ever direction is the shortest to reach landing runway, unless directed otherwise by ATC, IAP, or Enroute Supp. SSE: If able, fly circling approach above circling MDA (preferably at VFR pattern altitude of 800’).  In training, we fly at circling MDA.  Don’t put Flaps-APPR & Gear Down Landing Checklist until at 180 pos (latest is 90) and in safe position to land.  Circling Obstruction Clearance Area for Category B aircraft is 1.5nm.  If on an approach and the engine fails after configuring, raise flaps and gear immediately.  Don’t put them down until landing assured (180 pos).  To offset from runway (if landing on opposite side of runway approach was flown to)– turn 45 degrees offset & fly out for 15 seconds.  Missed approach instructions are for runway approach was flown to (not the runway landing)!  Must be familiar with missed approach procedures to know what direction to turn!

RADIO INSTRUMENT MALFUNCTION: (FTI 4-49) (CDI failure for VOR/TACAN): Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.

ENGINE SYSTEMS

1. Each engine is rated at 550 shp.

2. Total compession for each engine is equivalent to 7:1.

3. There are 2 igniter plugs per engine located at the 3 and 9 o’clock position.

4. Approximately 75% of intake air is actually used for cooling.

5. Reduction gear box (RGB) provides reduction ratio of 15:1.

6. The systems operating off the N1 compressor turbine are the compressor and accessories case: [1] fuel control system – engine driven fuel pump, fuel control unit [FCU], starter control unit [SCU], fuel spray nozzles, oil-to-fuel heater [2] fuel drain collector system – [a] fuel collector tank [b] fuel drain collector pump (EPA pump) [3] starter/generators [4] turbine tachometers

7. The systems operating off the N2 (power) section are [1] reduction gear box (RGB) [2] propeller system [3] engine torquemeters [4]propeller tachometers

8. Inputs are sent to the Fuel Control Unit (FCU) are: [1]Power lever [2]Compressor turbine speed (Ng) – fuel control’s governor flyweights driven by accessory section [3]Compressor air (P3 air): air sample in which air density determined [4]Propellor governor speed (Nf) – pneumatic system  determining prop speed (the fuel topping governor).

9. Inputs to the starter control unit are through the condition lever.

10. The compressor progressive bleed valve opens at low N1 to reduce compressor pressure between the axial and centrifugal compressors.  As N1 increases, starts to close at 62%N1 and is fully closed by 75%N1.  When power is reduced, the valve opens to bleed excess pressure in the compressor.

11. The fuel drain collector system consists of the fuel collector tank, fuel drain collector pump, and associated fuel lines.  The fuel collector pump provides automatic transfer of accumulated residual fuel back into the fuel supply by after shutdown when the fuel collector tank is full and its internal float switch actuated.

12. When the starter switch is accuated to ‘STARTER ONLY,’ the engine is motored without providing ignition.  In ‘IGN ENG START’ position: completes the starter circuit, energizes the igniter plugs, opens the purge valve in the FCU, provides fuel to the fuel nozzles, and actuates the ‘IGN IND’ light on the annunc panel.

13.Torque is measured by a movable ring gear located in the first reduction gear stage.  Metered oil pressure across the orifice is measured against a calibrated pressure sensor in proportion to torque sensed.  The measurement is sent to the torquemeters through a 26V AC signal.

14. With a failed torquemeter, auto ignition and autofeather are still available.

15. The oil system has a 14 Quart capacity.

16. The top 5 Quarts of oil is measurable.

17. The oil tank can hold 9 Quarts.

18. There are 4 scavenger pumps per engine.  They return oil to from the oil pickups to the oil cooler.

19. Autofeather is accomplished by: [1] Propellor Autofeather switch in ‘ARM’ [2] both power levers > 90 (+/- 2)% N1 [3] Torque on one engine <260(+/- 50)ft-lbs.  The autofeather relay applies current to the dump valve, causing the release of oil pressure holding pitch angle on the prop.  As the pressure releases, the feathering springs (assisted by centrifugal force applied to the blade shank counterweights) feather the prop.  After one prop feathers automatically, the other prop can only be feathered manually.  The right prop may not fully feather with the prop sync on.  Autofeather only used during takeoff and landing.

20. Oil temp is maintained by running oil through a heat exchanger unit.  They are attached to the nacelles below the engine air intake.  As oil is returned from the integral oil pickups, it is returned via scavenger pumps to the oil cooler.  Air passing through the oil cooler cools the engine oil.  Normal oil temp (10 – 99C) is maintain by a thermal sensor that controls the bypass valve allowing some oil to bypass the oil cooler.

21. The oil-to-fuel heat exchanger utilizes heat from engine oil to preheat engine fuel.  A fuel temp sensing oil bypass valve regulates fuel temp by either allowing oil to flow through the heater core or bypassing it.  A temp sensing element controls the oil inlet valve.  At a fuel temp >70F, the bypass valve begins to open; at 90F, the oil completely bypasses the heater core.

22. The chip detector is located at the bottom of the reduction gearbox housing.  Its purpose is to warn the pilot of ferrous metal in the oil and a possible engine failure.  If a large particle (or several small particles) complete the circuit, the ‘CHIP DETECT’ annunc light and the ‘FAULT WARNING’ light will illum.

23. Approved oil for the T-44 is MIL-L-23699, NATO O-156, or EXXON 2380.  (List found in NATOPS 3-2)

24. The fire bottles are normally pressurized to 450psi.

25. The minimum N1 required on the deck for generator loads exceeding 0.5 are [1] 0.5 – 0.75 = 57% [2] 0.75 –0.9 = 60% [3] 0.9

–1.0 = 63%

26. The oil system provides: [1] cools and lubricates the engine [2] operates prop pitch change and engine torquemeter system.

27. The procedures for a cold oil check: [1] Open the upper right cowling to gain access to the oil filler cap [2] Remove the oil filler cap. [3] Replenish oil to within 2 quart below MAX mark on dipstick. [4] Check oil filler cap for damage and general condition [5] Check for oil leaks.

# ENGINE SYSTEM MALFUNCTIONS

# [1] OIL SYSTEM FAILURE

Oil press < 85psi undesirable – tolerated only to complete flight. Monitor engine instruments and nacelles for secondary indications.  Consider shutting down engine and landing as soon as POSSIBLE; otherwise, reduce power on engine and landing as soon as PRACTICABLE.  Oil pressure < 40psi & oil temp >99C unsafe requiring engine shutdown or landing made as soon as possible using min power to sustain flight.  Note both discrepancies on maintenance forms.

# [2] CHIP DETECTOR LIGHT ILLUMINATION

Indicates metal particles in prop reduction gearbox.  Perform EMERGENCY SHUTDOWN CHECKLIST.

# [3] EMERGENCY ENGINE SHUTDOWN ON DECK (for FIRE, FIRE WARN LIGHT, CHIP LIGHT)

1. Condition Levers – FUEL CUTOFF

2. Firewall Valves – CLOSED

3. Boost Pumps – OFF

4. Fire Extinguisher – AS REQUIRED

5. Gang Bar – OFF

6. Evacuate aircraft

[4] ABNORMAL START (for HOT START & NO LIGHT OFF)

If ITT likely to exceed 925C or no rise in ITT within 10 sec after condition levers LOW IDLE:

1. Condition Lever – FUEL CUTOFF (ITT < 790C)

2. Starter – OFF

Caution: Starter time limited to 40 sec ON, 60 sec OFF, 40 sec ON, 60 sec OFF, 40 sec ON, 30 minutes OFF.

Note: If start attempt discontinued and another start attempted, allow 60 sec delay to drain fuel and cool starter, then motor up starter for 15 sec minimum.  Allow eng to completely stop before attempting another start.  During restart, push engine up to 85% N1 before restarting the other engine.

# [5] ABORT & ENG FAILURE DURING TAKEOFF

1. Announce -- “ABORT” (Initiate < 91KIAS)

2. Power Levers – IDLE

3. Reverse – AS REQUIRED (then beta range)

4. Brakes – AS REQUIRED (smooth, even, don’t lock)

Prior to departing prepared surface:

(verbally recite remaining items)

5. Condition Levers – FUEL CUTOFF

As soon as practicable:

6. Firewall valves – CLOSED

7. Boost Pumps – OFF

8. Fire Extinguishers – AS REQUIRED

9. Gang bar – OFF

10. Evacuate aircraft

Warning: If direction control problems occur while reversing, advance both power levers to FLIGHT IDLE.  Maintain directional control with rudder, nosewheel steering, and by pumping brakes.

Warning: Accelerate-stop distances increased 900 ft with condition levers HIGH IDLE and no reverse utilized.

# [6] ENGINE FAILURE AFTER TAKEOFF

If sufficient runway to stop on ground, ABORT!

If insufficient runway to abort takeoff:

(POWER UP, RUDDER UP, CLEAN UP)

1. Power – AS REQUIRED

2. Gear – UP

3. Airspeed – (102 (obstacles) or 110)

4. EMERGENCY SHUTDOWN CHECKLIST

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

Note: address first 2 items of dead engine checklist, transfer comm to CP and have him declare an emergency.

Note: Dynamic Engine Cut complete when trimmed at 110KIAS (min 102KIAS), on takeoff heading, and Emergency Shutdown Checklist complete.

## [7] INFLIGHT EMERGENCIES

Note: 1. Rudder trim sufficient above 100KIAS.  Below 100KIAS, supplement full rudder trim with constant rudder pressure.

2. SSE waveoff with full flaps, result in loss of 200 feet before positive rate of climb established.

3. Impending engine failure or flameout preceded by:

[1] unstable engine operation

[2] fluctuating turbine RPM

[3] torque and ITT temp changes

[4] illumination of fuel warning lights

[5] drop in oil pressure

[6] loss of thrust

4. DON’T attempt an airstart if: [MOVEOFF]

Mechanical malfunction

Overheating

Vibration

Explosion

O N1

Fire

Fuel Fumes

VOR CHARACTERISTICS: Frequency  108.0 to 117.95.  Line of sight.  Free from atmospheric disturbances.  360 radials.  Course accuracy +/- 1 degree.  Cone of confusion is directly above station, info is unreliable, cone increases with altitude.  Primary indication of station passage is TO/FROM indicator’s first positive change to FROM.  Only positive method of identifying VOR is Morse Code or voice.  VOR with a “W” or underlined means without LIVE voice capability.  For dual VORs, the max they can be off each other is 4 degrees.  VOR Test facility (VOT): [1] Tune to VOT frequency [2] Center CDI on 180 with ‘TO’ flag.  Checkpoints: max error is +/- 4 degrees on the ground and +/- 6 degrees in the air.  With a failure of the RMI, the VOR needle will still indicate the correct radial!

Terminal VOR (T) – 	25 nm – [1,000’ – 12,000’ AGL]

Low Altitude VOR (L) – 	40nm – [1,000’ – 18,000’ AGL]

High Altitude VOR (H) – 	[40nm - 1,000’ – 14,500’ AGL]

[100nm – 14,501’ – 17,999’ AGL & 45,001’ – 60,000’]

[130nm – 18,000’ – 45,000’]

VOR APPROACH (WITH PROCEDURE TURN):  Procedure Turns (PT): 2 methods: [1] 45/180 Technique [2] Holding Technique.

[1] 45/180 Technique:  Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN).  Continue descent to FAF.  Configure and slow to 120K prior to the FAF (unless SSE).  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

[2] Holding Technique:  If heading within 90 degrees of outbound PT Course, use normal lead points (includes all of teardrop entry and most of parallel entry).  If use teardrop entry, course must be within 30 degrees of PT outbound course.  If intercept the PT outbound course, maintain course for remainder of outbound leg, then turn toward the maneuvering side to reverse course.

Timing: begin timing outbound abeam procedure turn fix.  If can’t determine abeam, start timing at completion of outbound turn.  Stay within the remain within distance.  At the completion of the outbound leg, turn to intercept the procedure turn course inbound.  A 1 ½ minute outbound leg normally is sufficient (2 minutes or more can be used in instances such as an approach with no FAF or an approach with a large descent on the inbound course).

Descent: Don’t descend form fix altitude until abeam PT fix outbound.  Don’t descend from completion turn altitude until established on inbound segment.

Teardrop techniques: Rule-of-thumb for proper offset: 30 degrees for one minute on outbound leg, 20 degrees for two minutes on outbound leg, 10 degrees for three minutes on outbound leg.

VOR APPROACH (WITH RADAR VECTORS): Maintain 150K until ready to configure.  Have a backup approach available.  Complete Approach checklist.  Use normal lead points.  Configure, slow to 120K, and complete Landing Checklist prior to FAF.

SSE NON-PRECISION (VOR) APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

HOLDING:  When ATC issues holding clearance, the direction they give points TO the holding fix (exp.  hold west of the NGP VORTAC means the arrow of the inbound course is FROM the west).  Always get an Expected Further Clearance Time (EFC), it determines course of action in a lost comm situation.  Standard holding means all turns to right.  Non-standard holding means all turns to left.  Make all turns either standard rate or max of 30 deg AOB.  If must tack 10 deg into wind on inbound leg, must tack 30 deg (3 times as large a correction) in opposite direction on outbound leg.  TACAN stations are not acceptable as holding fixes.  If extended holding, consult NATOPS for max endurance speed.  Request extended legs if delay exceeds 20 minutes.  ATC should issue holding instructions 5 minutes prior to reaching clearance limit fix.  Always request an EFC prior to accepting a holding clearance if one was not given.

Timing:  Begin outbound timing over/abeam the fix, whichever occurs later.  If abeam position can’t be determined, start timing when turn outbound is complete.  < 14,000’ MSL, time 1 minute. > 14,000’ MSL, time for 1 ½ minutes.  Commence inbound timing at wings level.

Speed: Recommended T-44 holding speed is 150KIAS.  Max holding speed is 175KIAS for turboprop aircraft.  Expect ATC holding clearance 5 minutes prior to arriving at holding fix.  Start a speed reduction to 150K (holding airspeed) 3 minutes or less from the holding fix.

Holding Entry:  Reaching the fix, execute the 6 T’s: Time, Turn, Time, Transition, Twist, Talk

Time: Note the time of station passage (VOR: reversal of TO to FROM)

Turn: Parallel, Teardrop, or Direct: determined using the outbound course on the RMI card.  Use pencil method.  If standard, put pencil on 90deg mark on RMI with tip pointed to right & turn 20 deg UP.  Relative position of 360 to 070 = Teardrop.  Relative position 360 to 250 = parallel.  All headings below the pencil are direct.  If nonstandard, put pencil on 90 deg mark on RMI with tip pointed to left & turn 20 deg UP.  Relative position 290 to 360 = teardrop.  Relative position from 360 to 110 = parallel.  All headings below the pencil are direct.

Parallel:  maneuver to outbound course (non-holding side) for one minute, then turn toward holding

pattern inbound course.

Teardrop: Subtract 30 deg from outbound course for standard & add 30 deg for nonstandard.

Direct: Turn in direction of outbound course.

Time: Time first leg outbound (1 min or 1 ½ min)

Transition: slow to 150KIAS

Twist: put in inbound holding course

Talk: Tell ATC established in holding

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.

IFR NOTES:

1. Must ID navaids that I am using

2. On non-precision approach, put the gear down 3nm prior to the FAF.

3. SSE approach: don’t put the gear down until landing assured.

4. No hdg: center CDI on inbound course.  Put myself on the head of the needle.  Turn in the direction the CDI is deflected!

5. When intercepting outbound course, use either a 30 or 45 degree intercept.

6. Always call “clear left” or “clear right” before turning!

7. Don’t chase needles when close to a navaid.

8.  Start VOR timing outbound when the TO-FROM flag switches to FROM.

9. As part of RADIO/NAVAIDS in Approach Checklist, put VHF Tower freq in VHF Preset.

10. TRIM ON LANDING!!!

11. Dirty final descent, use 350ft-lbs if dual engine & 1000ft-lbs if single engine.

12. Inside the FAF, always fly 120KIAS (indicated)!

13. Descend to MDA early (800fpm descent/ 250ft-lbs) & drive into MAP.

14. Always inform ATC when leaving an altitude to begin an approach.

15. Passing a fix, do the 6 Ts (Time, Turn, Time, Transition, Twist, Talk).  For Transition, remember speed & DESCENT!!!

16. SSE Non-Precision: Don’t configure to landing assured and in a safe position to land (NLT 1nm from threshold on straight-in).

17. Taking off from an approach at one field to an approach on a different navaid (or a different field), MUST RETUNE AT LEAST ONE NAVAID FOR SITUATIONAL AWARENESS OF NEXT APPROACH!