RI-13 Study Guide (24 Jun 02)

__ IFR COMM (KNEEBOARD)

__ IFR BRIEFS (APPX. A & AIGT CHAP. 8)

__ LOCAL DEPARTURE INSTRUCTIONS (BLUE BRAINS)

__ CHECKLIST PROCEDURES (PARTICULARLY ENGINE RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES GUIDE (DEPARTURES & ARRIVALS TO NAVY CORPUS)

__ EMERGENCY PROCEDURES

NDB NAVAID CHARACTERISTICS:

(AIM 1-1-1) NDB = Non-Directional Beacon. It is the station that sends the signal. ADF = Automated Direction Finder. It’s the radio receiver in the airplane that picks up the NDB signal and converts it into bearing information. low or medium freq radio beacon, nondirectional, gives bearings, “home” to the station, freq 190-535 kHz, transmits continuous carrier with either 400 or 1020 Hz modulation. All NDBs except compass locators, transmit a 3-letter ID code, except during voice transmissions. An NDB coupled to ILS markers are a Compass Locator. Voice transmissions are made on all NDBs unless the letter “W” (without voice) is included in the class designator (HW). Subject to disturbances that may cause erroneous bearing information, such as lightning, precipitation, static, etc. . . At night, NDBs are vulnerable to interference from distant stations. Noisy identification usually occurs when the ADF needle is erratic. ADF receivers don’t have flags. The only way to tell if the signal is correct is to CONTINUOUSLY MONITOR the station!!!

(AIGT 1-12) The only information an NDB provides is direction to the station. The LF/MF NDB freq = 190 to 535 KHz. UHF NDB freq = 275 to 287 MHz. Accuracy is = +/- 5 degrees. Pilot must continuously monitor the NDB. Station passage is when the pointer moves through the wingtip position. Due to dip error the ADF bearing pointer gives accurate bearings only in wings-level flight. NDB station generates a NON-DIRECTIONAL nav signal. Compass locators are normally installed with marker beacons. When a compass locator is co-located with a marker beacon, it is a “Locator Marker.” Compass locators freq = 200 – 415 kHz with a reliable range to 15nm, transmit 2-letter identification groups. The outer locator transmits the first 2 letters of the localizer ident group, and the middle locator transmits the last two letters of the localizer identification group. Beacons with code ‘SAB’ indicate continuous weather information. UHF ADF may also be used to provide auxiliary emergency voice reception if the normal radio receiver should fail.

(AUGT 2-28) The T-44 uses the Collins ADF-60. Reception range is based upon power output of the transmitting station and atmospheric conditions (it isn’t line-of-sight). The ADF VOLUME switch is the ON/OFF switch for the ADF! The “R” Range filter filters out all 1020 Hz tone information so that only voice information is heard. The “V”filter only works in conjunction with the ADF. It is used to filter out voice audio signal so that only the ID signal can be heard and continuously monitored. If both switches are on, they cancel each other out. The ADF is only tuned through the NCS-31. ADF information is displayed on the SINGLE needle of the RMI (no ADF information is available on the HSI). On the MSU, there are 3 buttons under “ADF.” The TONE button will cause the received AD carrier signal to be modulated with an audible 1000 Hz tone (the normal position for this button is out). The TEST button causes the single RMI needle, when switched to ADF, to rotate counterclockwise 90 degrees if valid bearing information is received. The ADF button is sort of an ON/OFF switch for the ADF needle (normal position is IN). The ADF signal must be continuosly monitored when used as a navaid (delegate this responsibility to the observer or the copilot). When the ADF receives a weak signal or no signal, the ADF needle will park in a horizontal position (9 to 3 position). The number under the needle’s head is the “course to” the station. The number under the tail is called the “bearing from” the station. If the compass card fails, the ADF needle will continue to point towards the direction of the station, relative to the nose of the aircraft, regardless of the compass card position. To head to the station, put the head of the needle at the top of the RMI card. No-turned headings must be timed, the needle takes a moment to settle down after a turn.

(AIGT 2-30) The T-44 uses the ARA-50 direction finder, in conjunction with teh ARC-159 UHF transceiver, for UHF ADF. Allows the aircraft to home in on UHF NDB stations or other aircraft transmitting on the UHF frequency (very helpful for search and rescue). The double needle will display UHF NDB information in the ‘DF’ position. It homes to the STRONGEST signal on the frequency selected. To make it work: tune the frequency on the UHF radio, select “ADF” on the UHF radio, switch the double needle on the RMI to “DF.” Continuously monitor the aural signal. Azimuth information is relative to the aircraft.

(FTI 4-18) Setting up the ADF: TIMSS [1] Tune the ADF freq in the NCS-31 [2] Identify the station using the ADF switch to get an aural tone (2 or 3 letter Morse Code) [3] Monitor the tone continuously while using the ADF [4] Select FD switch – N/A for the ADF (can only read ADF of the RMI needles) [5] Set the single needle to “ADF” if using LF/MF freq, or set the double needle to “DF” using UHF freq.

NDB COURSE INTERCEPTS/TRACKING: (RMI only): (FTI 4-19) Inbound: Course to Bearing + 30 = Intercept Bearing. Outbound: Tail to Course + 45 = Intercept Bearing. Station Passage: bearing pointer passes to the wingtip. In NDB holding, station passage is the first definite move by the bearing pointer through the 45 degree index on the RMI. Radial to Arc – lead by 0.8 nm for 90 degree turn at 150K. Arc to Radial – lead a 90 degree turn by 5 radials at 10 DME at 150K.

ADF HOLDING:  When ATC issues holding clearance, the direction they give points TO the holding fix (exp.  hold west of the NGP VORTAC means the arrow of the inbound course is FROM the west).  Always get an Expected Further Clearance Time (EFC), it determines course of action in a lost comm situation.  Standard holding means all turns to right.  Non-standard holding means all turns to left.  Make all turns either standard rate or max of 30 deg AOB.  If must tack 10 deg into wind on inbound leg, must tack 30 deg (3 times as large a correction) in opposite direction on outbound leg.  If extended holding, consult NATOPS for max endurance speed.  Request extended legs if delay exceeds 20 minutes.  ATC should issue holding instructions 5 minutes prior to reaching clearance limit fix.  Always request an EFC prior to accepting a holding clearance if one was not given.

Timing:  Begin outbound timing when abeam fix.  If abeam position can’t be determined, start timing when turn outbound is complete.  < 14,000’ MSL, time 1 minute. > 14,000’ MSL, time for 1 ½ minutes.  Commence inbound timing at wings level.

Speed: Recommended T-44 holding speed is 150KIAS.  Max holding speed is 175KIAS for turboprop aircraft.  Expect ATC holding clearance 5 minutes prior to arriving at holding fix.  Start a speed reduction to 150K (holding airspeed) 3 minutes or less from the holding fix.

Holding Entry:  Reaching the fix, execute the 6 T’s: Time, Turn, Time, Transition, Twist, Talk

Time: Note the time of station passage (NDB: needle passes 45 degree benchmark on RMI, cleared to turn)

Turn: Parallel, Teardrop, or Direct: determined using the outbound course on the RMI card.  Use pencil method.  If standard, put pencil on 90deg mark on RMI with tip pointed to right & turn 20 deg UP.  Relative position of 360 to 070 = Teardrop.  Relative position 360 to 250 = parallel.  All headings below the pencil are direct.  If nonstandard, put pencil on 90 deg mark on RMI with tip pointed to left & turn 20 deg UP.  Relative position 290 to 360 = teardrop.  Relative position from 360 to 110 = parallel.  All headings below the pencil are direct.

Parallel:  maneuver to outbound course (non-holding side) for one minute, then turn toward holding

pattern inbound course.

Teardrop: Subtract 30 deg from outbound course for standard & add 30 deg for nonstandard.

Direct: Turn in direction of outbound course.

Time: Time first leg outbound (1 min or 1 ½ min)

Transition: slow to 150KIAS

Twist: put in inbound holding course

Talk: Tell ATC established in holding

ADF NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, check the circuit breakers and the God Box, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

The ADF needle shows relative bearing only.  Mentally place the current heading (given by the CP from the mag compass) under the upper lubber line.  Count the number of degrees the head of the single needle deflected from the lubber line to find the relative bearing to the station.  Add (+ to the right) or subtract (- to the left) from the current mag heading to get the magnetic bearing to the station.  Mag bearing from the station found by using the tail of the needle when heading away from the station.  (Exp. If the current heading is 135, and the needle points 45 degrees to the left of the lubber line, subtract (because the needle is left) 45 from 135 to get a magnetic bearing of 090 to the station.  Unless radar vectors are given, request the 45/180 course reversal.  Turn to place the head of the needle on the nose and fly direct to the station.  Utilized known winds to make a wind-corrected heading to the station.  Continue to ask the CP for heading calls and set the heading bug on the corresponding heading to proceed direct.  Compute a heading to intercept the outbound course (the outbound course + 30 degrees [extra 10 seconds]) and determine the seconds required for a timed turn.  Immediately after station passage, commence a timed turn to intercept the outbound course.  Track outbound 1 minute using heading calls from the CP, then start a 45 degree [15 second] turn to the outbound PT heading.  After 1 minute, commence a 180 degree [1 minute] course reversal.  Inbound on the procedure turn, determine my position for situational awareness.  When the needle reaches the 45 degree benchmark, turn inbound so the head of the needle is at the top of the RMI.  Continue to ask the CP for headings.  Configure at the normal point for a non-precision approach.  Passing over the IAF and the FAF, execute the 6Ts (time, turn, time, transition, twist, talk).  If missed approach, generally requires turning to the head of the needle and flying direct back to the station.

ADF APPROACH (WITH PROCEDURE TURN):  Procedure Turns (PT): 3 acceptable methods: [1] 45/180 Technique [2] Holding Technique [3] Teardrop Technique

[1] 45/180 Technique:  Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 bearings for ADF).  Continue descent to FAF.  Configure and slow to 120K prior to the FAF (unless SSE).  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

[2] Holding Technique:  If heading within 90 degrees of outbound PT Course, use normal lead points (includes all of teardrop entry and most of parallel entry).  If use teardrop entry, course must be within 30 degrees of PT outbound course.  If intercept the PT outbound course, maintain course for remainder of outbound leg, then turn toward the maneuvering side to reverse course.

Timing: begin timing outbound abeam procedure turn fix.  If can’t determine abeam, start timing at completion of outbound turn.  Stay within the remain within distance.  At the completion of the outbound leg, turn to intercept the procedure turn course inbound.  A 1 ½ minute outbound leg normally is sufficient (2 minutes or more can be used in instances such as an approach with no FAF or an approach with a large descent on the inbound course).

Descent: Don’t descend form fix altitude until abeam PT fix outbound.  Don’t descend from completion turn altitude until established on inbound segment.

SSE ADF APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

PAR APPROACH (NORMAL): (NATOPS 18-4, FTI 4-41) Refer to Normal PAR Approach notes (27 May 02)

PAR APPROACH (SINGLE ENGINE): (NATOPS 16-3, FTI 4-41)  Initiated from an Propeller Malfunction

RADIO INSTRUMENT MALFUNCTION: (FTI 4-49) CDI failure: Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.  DME failure:  Don’t shoot approach that requires DME.  Request a non-DME approach (PAR, ASR, ILS, LOC, etc...)  If inside FAF and lose DME (timing NOT available), execute a missed approach unless field is in sight.

PROPELLERS (NATOPS 2-9 to 2-11)

1. Prop characteristics: hydraulic controlled, constant-speed, full-feathering, and reversible.

2. Feathering: feathering springs assisted by centrifugal force applied to the blade shank counterweights.  Moving Prop Lever to FEATHER lifts pilot valve to allow complete dump of high press oil.

Manual feathering: pull prop lever to feather.

Automatic feathering (ARMED): sense loss of torque oil press and feather an unpowered prop – only can occur when: [1] autofeather – ARMED [2] both power levers > 90 (+/-2)%N1 [3] torque on one engine < 260 (+/-50)lbs.  Before auto prop feather, interlock disarms autofeather circuit of opposite propellor.

Feathers for: [1] loss of boosted prop governor oil pressure (engine shutdowns on ground and anytime prop not turning).

Once one prop feathered, other prop must be manually feathered.

Right prop sync may not feather with prop sync on.

3. Low Pitch Blade angles: 	Feathered = 87	Low Pitch = 15	0 Thrust = -5	Reverse = -11

Cruise = 25 to 35 	Idle = 15 to –5	Beta = -5 to –11

4. Max Reverse = -5 to –11 at 86% max N1 at 2100 prop RPM

5. Max N1 in BETA _______ (condition levers – LOW IDLE)

6. Centrifugal counterweights, assisted by feathering spring, move blades to low RPM (high pitch) and feather.  Governor boosted oil press moves prop to high RPM (low pitch) hydraulic stop and reverse.  Low pitch by mechanically activated hydraulic stop, causes beta valve on primary gov to stop oil flow to prop dome.  Low pitch blade pos by primary low pitch stop.  Allows blades to rotate beyond low pitch into reverse.  Beta and reverse provided by adjusting low pitch stop, controlled by power lever when aft of idle.  Prop is at low pitch when prop RPM is below selected governor PRM (when prop lever full forward position 2200, but N2 reads 2000).  No high pitch stop so prop can feather.

7. Norm op range of primary gov is 1800-2200RPM.

8. If prop RPM exceeds 2200 by more than 4%, the overspeed gov 2288 +/-40.  Fuel topping gov 2332.

9. In reverse, pneumatic section of primary gov reset to allow max 2100RPM.

10. Prop gov test switch – actuates solenoid so overspeed gov reset to 1900 to 2100

--  Condition levers – HIGH IDLE – Max RPM 2200.

--  Power levers – IDLE & Condition levers – LOW IDLE – Prop RPM 900 to 1100.

--  If prop RPM not btwn 900 to 1100 with power levers – IDLE & condition levers – LOW IDLE, perform low pitch torque check (NATOPS 10-5).

11. Reduction gearbox failure and/or N2 turbine damage occurs at 2420RPM.

12. Prop reverse not ready light: gear down and prop levers not HIGH IDLE.

13. Autofeathering: used only on t/o & landing.  ARM.  Engine loses power, ARMED, power levers > 90 (+/-2)% N1, 410 (+/-50)ft-lbs & 260 (+/-50)ft-lbs torque switches actuated by loss of torque pressure.  Current through autofeather relay to dump valve, releases oil press holding pitch angle on affected prop.  Feathering springs assisted with centrifugal force applied to blade shank counter weights.  TEST pos on switch enables check < 90% N1 on ground only.

14. Syncrophaser master engine = left.  Inflight use only.  Maintains blade positions of both props via magn speed pickup on prop overspeed gov and mag phase pickups on prop deice slip ring adjacent to brush block.  Creates pulse rate, transmitted to actuator motor on right eng.  Motor trims right eng to left.  Trimmer screws in  & out.  Limited to +/- 30 RPM gov setting.   Right prop may not fully (manual) feather with prop sync on. If props won’t sync, actuator at limits – turn prop sync OFF, synch props manually, turn sync ON.  Prop Sync On light when in system in use and landing gear extended.

15. The prop lever controls the primary gov – varies tension on Speeder Spring to reposition Pilot Valve, allows more gov boosted oil into prop dome for (Higher RPM, low pitch) or oil dumped from dome (Lower RPM, higher pitch).

16. Power lever IDLE or above – sets low pitch at 15 blade angle.  Beta = -5.  Max reverse = -11.

17. Fuel topping gov – pneumatic section of primary gov, limits fuel flow to eng, reduces N1 RPM, on at 2332RPM.  Pneumatic line routes bleed air from eng compressor to FCU to primary gov.  If >2332RPM, airbleed orifice of prim gov opened by flyweight to bleed off P3 press through primary gov, closes fuel control bellows, reducing fuel flow.

18. In reverse, the pneumatic sect of primary gov reset to allow max of 2100RPM.

19. A prop would feather in flight if autofeathered armed, power levers > 90%N1, drop in torque below 260ft-lbs on same engine OR prop lever manually pulled to feather.

20. Max N1 acceleration 102.6%.

* The low pitch stop is controlled by the power lever.  In Idle or above, it sets the blade angle at 15 degrees.

# PROPELLER MALFUNCTIONS (NATOPS 15-10)

Primary Governor Failure (memory item): feather or overspeed.  If landing with overspeeding prop carry extra airspeed.

If prop rpm out of normal governor range (1800 – 2200RPM):

[1] Propeller RPM  -- Adjust to Normal RPM.

By prop lever. If norm restored, continue ops.  If norm can’t be obtained:

[2] Power Lever – IDLE (concur)

[3] Propeller Lever – FEATHER (concur)

Notes: Prop RPM > 2420 may result in RGB failure and/or N2 turbine damage.  Engine with disabled prop can provide electric power.  Right prop may not full feather with prop sync ON.

[4] Emergency Engine Shutdown Checklist – AS REQ

Prop Linkage Failure: prop gov ctl linkage fails, prop goes to 2200 or maintains last rpm setting.

[1] Manipulate prop lever to determine if cockpit control lost

[2] If cockpit control lost and RPM in safe limits, match opposite prop speed with uncontrollable prop and land as soon as practicable.

Alternate Prop Feathering: Linkage or gov fails and prop doesn’t feather by itself or normal feathering procedures don’t work, use autofeather.

[1] Power Lever – IDLE (failed propeller) (concur)

[2] Condition Lever – FUEL CUTOFF (concur)

[3] Autofeather – ARM

[4] Power Levers -- > 90% N1 *Don’t pull power lever to IDLE during autofeather, may disarm system.

[5] Emergency Shutdown Checklist – EXECUTE

Simulated Dual Engine Failure: initiated above 91K by IP reducing bother power levers to idle.  Sea Level is bottom of deck.  Select appropriate ditch heading.  Simulated restart on both engines simultaneously.

[1] Clean up (Gear – UP, Flaps – UP)

[2] Commence turn towards coastline, desired hdg, IP assigned hdg while transitioning to 130K (max range) or 102 (max endurance) if restart altitude minimal.

[3] Simultaneously commence Windmilling Airstart Checklist.  Simulate both condition levers fuel cutoff by pointing  at both levers.  Autoignition may be ARMED or starters simulated ON at student’s discretion.

[4] IP will state what engines lightoff.  If restart successful, add power and complete the checklist.  If restart fails, complete the Emergency Engine Shutdown checklist and follow ditching procedures.  IP will reduce props to 1900RPM when simulated feather and may add power to 100ft-lbs.  Stop engine restart attempts at some time during the power off ditch.  Secure the engines by doing at least first 3 items of Emergency Engine Shutdown.  Emphasize proper ditch.

# WINDMILLING AIRSTART

Use for emergency relight if engine inadvertently shutdown.

1. Power Lever (failed engine) – IDLE

2. Prop lever (failed engine) – FULL FORWARD

3. Condition lever (failed engine) – FUEL CUTOFF

4. Firewall Valve – OPEN

5. Autoignition – ARM

Note: Windmilling airstarts above 20,000 ft or below 2,200 prop RPM may exceed ITT limit.

6. Condition Lever – LOW IDLE

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 seconds

7. Power – AS REQUIRED

8. Generator – RESET/ON

9. Autoignition – OFF

10. Condition lever – HIGH IDLE

ENGINE FAILURE (SECOND ENGINE)

Don’t feather both props if a windmilling airstart intended.  If all attempts fail, transition to max glide range (130KIAS, gear up, flaps up, prop feathered) or max glide endurance (102KIAS, gear up, flaps up, prop feathered).

If dual failure at low altitude and airspeed, consider engaging both starter switches vice using autoignition.  Battery power to starter is significantly reduced when electric heat or air-conditioner is used.  Neither prop being feathered will significantly reduce glide range and endurance.

No wind glide range is 2nm/1,000 ft altitude.  Subtract 0.2nm per 10 knots headwind.

With dual engine failure, only battery power is available.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.