RI-9 Study Guide (19 Jun 02)

__ IFR COMM (KNEEBOARD)

__ IFR BRIEFS (APPX. A & AIGT CHAP. 8)

__ LOCAL DEPARTURE INSTRUCTIONS (BLUE BRAINS)

__ CHECKLIST PROCEDURES (PARTICULARLY ENGINE RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES GUIDE (DEPARTURES & ARRIVALS TO NAVY CORPUS)

__ EMERGENCY PROCEDURES

__ POCKET NOTES

TACAN CHARACTERISTICS: (AIGT 1-10)

TACAN: Military navaid.  Tuned on pedestal.  Supplies directional info and DME.  UHF.  126 channels for X and 126 for Y.  Morse Code ident every 37.5 seconds.  Course accuracy +/- 1 degree.  Provides directional navigation azimuth.  A weak airborne receiver may false lock-on with azimuth indications of multiples of 40  (verify position with VOR).  Recycling the receiver may fix the problem.  Even if the NCS-31 completely fails, the TACAN still works from the pedestal.  If the TACAN parks in the 3 to 9 o’clock position, it isn’t working.  Cone of confusion – up to 100 degrees or more in width.  Station passage is minimum DME (DME indicates altitude above station in nm).  With a failed compass card (indicated by a “HDG” Warning Flag), a functional TACAN needle will continue to indicate radials to station.  If the card is spinning, the needle spins with the card (still showing the correct radial).  Can’t use VOR1 & TACAN at same time in T-44!  To use the TACAN, place the FD Select Button to “TACAN.”  The TACAN switch only monitors the signal from the NAVAID providing DME info.  If DME is coming from VOR/DME, identification heard through the TACAN switch on the audio control panel is for the VOR/DME station.  The TACAN switch must be in “OPER” to receive DME!!!  Pilot’s CDI (not the copilot’s) shows TACAN information.  A “NAV” Flag on the pilot’s CDI in TACAN mode indicates the TACAN info is unreliable.  The pilot & copilot receive TACAN info on their Flight Directors by the magenta needle.

Terminal TACAN (T) – 	25 nm – [1,000’ – 12,000’ AGL]

Low Altitude TACAN (L) – 	40nm – [1,000’ – 18,000’ AGL]

High Altitude TACAN (H) – 	[40nm - 1,000’ – 14,500’ AGL]

[100nm – 14,501’ – 17,999’ AGL & 45,001’ – 60,000’ AGL]

[130nm – 18,000’ – 45,000’ AGL]

DME: line of sight, range of 199 miles, tuned indirectly through VOR1 or TACAN.  accurate within ½ mile or 3% of distance, whichever is greater, identifies every 37.5 seconds in Morse Code on UHF (TACAN).  Receiving a DME signal identification ensures TACAN azimuth only when collocated with TACAN azimuth.  Freq = 1025-1150MHz, TACAN pairing frequencies with DME are located in FIH, Section D.  DME provides slant range.  For most accurate DME, aircraft should be at least 1 nm from DME for every 1000’AGL of altitude.  TACAN limits DME reception range to max 250nm.

TACAN APPROACH (WITH PROCEDURE TURN):  Procedure Turns (PT): 3 acceptable methods: [1] 45/180 Technique [2] Holding Technique [3] Teardrop Technique

[1] 45/180 Technique:  Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN).  Continue descent to FAF.  Configure and slow to 120K prior to the FAF (unless SSE).  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

[2] Holding Technique:  If heading within 90 degrees of outbound PT Course, use normal lead points (includes all of teardrop entry and most of parallel entry).  If use teardrop entry, course must be within 30 degrees of PT outbound course.  If intercept the PT outbound course, maintain course for remainder of outbound leg, then turn toward the maneuvering side to reverse course.

Timing: begin timing outbound abeam procedure turn fix.  If can’t determine abeam, start timing at completion of outbound turn.  Stay within the remain within distance.  At the completion of the outbound leg, turn to intercept the procedure turn course inbound.  A 1 ½ minute outbound leg normally is sufficient (2 minutes or more can be used in instances such as an approach with no FAF or an approach with a large descent on the inbound course).

Descent: Don’t descend form fix altitude until abeam PT fix outbound.  Don’t descend from completion turn altitude until established on inbound segment.

[3] TEARDROP TECHNIQUE: Rule-of-thumb for proper offset: 30 degrees for one minute on outbound leg, 20 degrees for two minutes on outbound leg, 10 degrees for three minutes on outbound leg.  If flying for extended time or distance, turn 30 degrees for 1 minute & then turn to parallel PT course until time/DME is reached.

HOLDING:  When ATC issues holding clearance, the direction they give points TO the holding fix (exp.  hold west of the NGP VORTAC means the arrow of the inbound course is FROM the west).  Always get an Expected Further Clearance Time (EFC), it determines course of action in a lost comm situation.  Standard holding means all turns to right.  Non-standard holding means all turns to left.  Make all turns either standard rate or max of 30 deg AOB.  If must tack 10 deg into wind on inbound leg, must tack 30 deg (3 times as large a correction) in opposite direction on outbound leg.  TACAN stations are not acceptable as holding fixes.  However, holding at DME off of a TACAN is acceptable and used.  If extended holding, consult NATOPS for max endurance speed.  Request extended legs if delay exceeds 20 minutes.  ATC should issue holding instructions 5 minutes prior to reaching clearance limit fix.  Always request an EFC prior to accepting a holding clearance if one was not given.

Timing:  Begin outbound timing over/abeam the fix, whichever occurs later.  If abeam position can’t be determined, start timing when turn outbound is complete.  < 14,000’ MSL, time 1 minute. > 14,000’ MSL, time for 1 ½ minutes.  Commence inbound timing at wings level.

Speed: Recommended T-44 holding speed is 150KIAS.  Max holding speed is 175KIAS for turboprop aircraft.  Expect ATC holding clearance 5 minutes prior to arriving at holding fix.  Start a speed reduction to 150K (holding airspeed) 3 minutes or less from the holding fix.

Holding Entry:  Reaching the fix, execute the 6 T’s: Time, Turn, Time, Transition, Twist, Talk

Time: Note the time of station passage (VOR: reversal of TO to FROM)

Turn: Parallel, Teardrop, or Direct: determined using the outbound course on the RMI card.  Use pencil method.  If standard, put pencil on 90deg mark on RMI with tip pointed to right & turn 20 deg UP.  Relative position of 360 to 070 = Teardrop.  Relative position 360 to 250 = parallel.  All headings below the pencil are direct.  If nonstandard, put pencil on 90 deg mark on RMI with tip pointed to left & turn 20 deg UP.  Relative position 290 to 360 = teardrop.  Relative position from 360 to 110 = parallel.  All headings below the pencil are direct.

Parallel:  maneuver to outbound course (non-holding side) for one minute, then turn toward holding

pattern inbound course.

Teardrop: Subtract 30 deg from outbound course for standard & add 30 deg for nonstandard.

Direct: Turn in direction of outbound course.

Time: Time first leg outbound (1 min or 1 ½ min)

Transition: slow to 150KIAS

Twist: put in inbound holding course

Talk: Tell ATC established in holding

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

OXYGEN SYSTEM: (NATOPS 2-46, 3-8)

*Location of Oxygen bottle = aft fuselage.  Location of recharging point for Oxygen system = starboard aft fuselage near the ELT.

Explosive decompression: occurs in less than 1 second, causes fog, if high alt – rush of air from lungs, dazed sensation, gas pains, hypoxia if oxygen not used immediately.

[1] Oxygen mask/ MIC switches (100%) – AS REQ

[2] Descend – AS REQ

Smoke and fume elimination (NATOPS 15-5): Locate, isolate, and extinguish fire or source of smoke and fumes prior to initiating smoke removal procedures.  Isolate the engine as a possible source by closing the bleed air valves.  Prior to depressurizing, consider MEA and crew O2 requirements.

Immediate smoke removal:

[1] Oxygen masks/MIC switches (100%) – AS REQ

[2] Pressurization – DUMP

After depressurizing, storm windows may be opened (may draw smoke into flight station).  Consider emergency descent and immediate landing.  Nauseating fumes from desalination solution can enter cockpit via bleed air for air-conditioning/pressurization and is often detected immediately after takeoff.  Providing no secondary indications exist, closing the bleed air valves will significantly reduce the fumes.

Supplemental oxygen is required when at cabin altitudes above 10,000’.   The oxygen system capacity is 1850 (+/-50) psig for one 49 cubic foot cylinder.  Between 17,000’ to 20,000’ the diluter demand regulator should supply 100% oxygen while in the normal position.  Above, 20,000’, select 100%.  O2 duration increases at higher altitudes due to partial pressure increase.  Passenger masks are constant flow at all altitudes.  The oxygen mask should be stored in the 100% position.  The min O2 requirement for local flights is 1000psig.  For cross-country flights: 1500psig.

ENGINE FIRE IN FLIGHT: (NATOPS 15-5, 15-1)  First indication will be the Master Caution Light & the “FIRE” Warning Light.  Confirm the fire by looking at the engine instruments and nacelles.  The instruments should show a high ITT and the nacelles should show smoke.  Even if no secondary indications, consider shutting down the engine anyway.  If fire confirmed or secondary indications noted, must perform Emergency Shutdown Checklist.

Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

*Before addressing the dead engine checklist: (1) transfer comms to CP (2) declare an emergency (3) then address Dead Engine Checklist.

TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.

SSE TACAN APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

TACAN MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.