RI-4 Study Guide (12 Jun 02)

__ IFR COMM (APPX. A)

__ IFR BRIEFS (APPX. A & AIGT CHAP. 8)

PILOT CONTROLLED LIGHTING: technique for control: Key mic on PCL frequency when over airport or at FAF on final approach (frequencies are listed in Enroute Supp & on IAPs).  Quickly (within 5 seconds) key microphone seven times to turn on (puts lights at high intensity), then adjust intensity as required (5 additional clicks for medium or 3 additional clicks for low).  Lights remain on for 15 minutes after keyed.

CIRCLING: 120K for dual and single engine.  Circling MDA and wx mins are for runway approach is flown to.  Circling mins are for non-radar non-precision approaches.  Don’t circle from a precision approach.  Report “commencing circle” when initiating any circling maneuver.  T-44 is Category B (approach speeds 91-120K).  If need to maneuver at airspeeds >120K, use Category C mins.  When requesting an ASR circling approach, state aircraft category (B) to controller.  When circling, keep runway environment in sight.  Circle in what ever direction is the shortest to reach landing runway, unless directed otherwise by ATC, IAP, or Enroute Supp.  If able, fly circling approach above circling MDA (preferably at VFR pattern altitude of 800’).  In training, we fly at circling MDA.  Don’t put Flaps-APPR & Gear Down Landing Checklist until at 180 pos (latest is 90) and in safe position to land.  Circling Obstruction Clearance Area for Category B aircraft is 1.5nm.  If engine fails after configuring, raise flaps and gear immediately.  Don’t put them down until landing assured.  Use 60 rule to offset from runway (if landing on opposite side of runway approach was flown to)– turn 30 degrees offset & fly out for 30 seconds OR turn 45 degrees offset and fly outbound for 15 seconds.  Missed approach instructions are for runway approach was flown to (not the runway landing)!  Must be familiar with missed approach procedures to know what direction to turn!

VOR CHARACTERISTICS: Frequency  108.0 to 117.95.  Line of sight.  Free from atmospheric disturbances.  360 radials.  Course accuracy +/- 1 degree.  Cone of confusion is directly above station, info is unreliable, cone increases with altitude.  Primary indication of station passage is TO/FROM indicator’s first positive change to FROM.  Only positive method of identifying VOR is Morse Code or voice.  VOR with a “W” or underlined means without LIVE voice capability.  For dual VORs, the max they can be off each other is 4 degrees.  VOR Test facility (VOT): [1] Tune to VOT frequency [2] Center CDI on 180 with ‘TO’ flag.  Checkpoints: max error is +/- 4 degrees on the ground and +/- 6 degrees in the air.  With a failure of the RMI, the VOR needle will still indicate the correct radial!

Terminal VOR (T) – 	25 nm – [1,000’ – 12,000’ AGL]

Low Altitude VOR (L) – 	40nm – [1,000’ – 18,000’ AGL]

High Altitude VOR (H) – 	[40nm - 1,000’ – 14,500’ AGL]

[100nm – 14,501’ – 17,999’ AGL & 45,001’ – 60,000’]

[130nm – 18,000’ – 45,000’]

VOR APPROACH (WITH PROCEDURE TURN):  Procedure Turns (PT): 2 methods: [1] 45/180 Technique [2] Holding Technique.

[1] 45/180 Technique:  Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN).  Continue descent to FAF.  Configure and slow to 120K prior to the FAF (unless SSE).  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

[2] Holding Technique:  If heading within 90 degrees of outbound PT Course, use normal lead points (includes all of teardrop entry and most of parallel entry).  If use teardrop entry, course must be within 30 degrees of PT outbound course.  If intercept the PT outbound course, maintain course for remainder of outbound leg, then turn toward the maneuvering side to reverse course.

Timing: begin timing outbound abeam procedure turn fix.  If can’t determine abeam, start timing at completion of outbound turn.  Stay within the remain within distance.  At the completion of the outbound leg, turn to intercept the procedure turn course inbound.  A 1 ½ minute outbound leg normally is sufficient (2 minutes or more can be used in instances such as an approach with no FAF or an approach with a large descent on the inbound course).

Descent: Don’t descend form fix altitude until abeam PT fix outbound.  Don’t descend from completion turn altitude until established on inbound segment.

Teardrop techniques: Rule-of-thumb for proper offset: 30 degrees for one minute on outbound leg, 20 degrees for two minutes on outbound leg, 10 degrees for three minutes on outbound leg.

VOR APPROACH (WITH RADAR VECTORS): Maintain 150K until ready to configure.  Have a backup approach available.  Complete Approach checklist.  Use normal lead points.  Configure, slow to 120K, and complete Landing Checklist prior to FAF. Configure 3 miles prior to FAF.

SSE NON-PRECISION (VOR) APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

HOLDING IN LIEU OF PROCEDURE TURN (HILO PT) APPROACH:  (FTI 4-37, AIGT 8-8)  The holding pattern track is depicted on the IAP by heavy black line (bold).  Must fly at least one pattern (orbit).  HILO PT will not be issued for ‘SnoR’: Straight-in approach, No PT course, Radar vector to final.   When giving a time to depart the holding pattern at the FAF inbound, ATC expects a straight-in approach.     Fly pattern using holding procedures outlined below in the HOLDING section.  If cleared for the approach, descent can be made to the minimum holding altitude anytime established in holding (initial passage of the holding fix or descent on the inbound leg [depicted on plate]).  If cleared for the approach while holding on the HILO PT, complete holding pattern and begin the approach without making additional turns in holding.  If additional turns are required, request it from ATC.  If at an altitude from which the approach can be safely executed and ready to turn inbound immediately, may request an early turn.

HOLDING:  When ATC issues holding clearance, the direction they give points TO the holding fix (exp.  hold west of the NGP VORTAC means the arrow of the inbound course is FROM the west).  Always get an Expected Further Clearance Time (EFC), it determines course of action in a lost comm situation.  Standard holding means all turns to right.  Non-standard holding means all turns to left.  Make all turns either standard rate or max of 30 deg AOB.  If must tack 10 deg into wind on inbound leg, must tack 30 deg (3 times as large a correction) in opposite direction on outbound leg.  TACAN stations are not acceptable as holding fixes.  If extended holding, consult NATOPS for max endurance speed.  Request extended legs if delay exceeds 20 minutes.  ATC should issue holding instructions 5 minutes prior to reaching clearance limit fix.  Always request an EFC prior to accepting a holding clearance if one was not given.

Timing:  Begin outbound timing over/abeam the fix, whichever occurs later.  If abeam position can’t be determined, start timing when turn outbound is complete.  < 14,000’ MSL, time 1 minute. > 14,000’ MSL, time for 1 ½ minutes.  Commence inbound timing at wings level.

Speed: Recommended T-44 holding speed is 150KIAS.  Max holding speed is 175KIAS for turboprop aircraft.  Expect ATC holding clearance 5 minutes prior to arriving at holding fix.  Start a speed reduction to 150K (holding airspeed) 3 minutes or less from the holding fix.

Holding Entry:  Reaching the fix, execute the 6 T’s: Time, Turn, Time, Transition, Twist, Talk

Time: Note the time of station passage (VOR: reversal of TO to FROM)

Turn: Parallel, Teardrop, or Direct: determined using the outbound course on the RMI card.  Use pencil method.  If standard, put pencil on 90deg mark on RMI with tip pointed to right & turn 20 deg UP.  Relative position of 360 to 070 = Teardrop.  Relative position 360 to 250 = parallel.  All headings below the pencil are direct.  If nonstandard, put pencil on 90 deg mark on RMI with tip pointed to left & turn 20 deg UP.  Relative position 290 to 360 = teardrop.  Relative position from 360 to 110 = parallel.  All headings below the pencil are direct.

Parallel:  maneuver to outbound course (non-holding side) for one minute, then turn toward holding

pattern inbound course.

Teardrop: Subtract 30 deg from outbound course for standard & add 30 deg for nonstandard.

Direct: Turn in direction of outbound course.

Time: Time first leg outbound (1 min or 1 ½ min)

Transition: slow to 150KIAS

Twist: put in inbound holding course

Talk: Tell ATC established in holding

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.