RI-14 Study Guide (26 Jun 02)

__ IFR COMM (KNEEBOARD)

__ IFR BRIEFS (APPX. A & AIGT CHAP. 8)

__ LOCAL DEPARTURE INSTRUCTIONS (BLUE BRAINS)

__ CHECKLIST PROCEDURES (PARTICULARLY ENGINE RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES GUIDE (DEPARTURES & ARRIVALS TO NAVY CORPUS)

__ EMERGENCY PROCEDURES

ASR APPROACH (NORMAL): (NATOPS 18-5, FTI 4-42) Request controller provide recommended altitudes on final.  Controller will tell pilot approach to fly, MDA, MAP location, and advance notice of when descent to MDA begin.  Optimum final descent gradient = 600fpm (3 degrees at 120K) on VSI.

ASR APPROACH (SSE): (FTI 4-42) Treat as SSE PRECISION approach for configuration.  If get well below recommended altitudes, revert to non-precision configuration (i/e don’t configure until landing assured and in a safe position to land).  If well below guide path & would have to over-torque the engine to get back up to glideslope, retract flaps and gear to get at or above MDA.  If not going to use recommended altitudes on final, treat as non-precision approach, slowing to 120K after configuration point, but before descent to MDA, with flaps and gear left UP.

ASR APPROACH (NO HEADING/ RADIO INSTRUMENT MALFUNCTION): (FTI 4-42) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.  *Request no-gyro vectors to a no-gyro PAR or ASR!  Turns up to final = SRT (not to exceed 30 AOB).  On final (after controller calls for half SRT), all turns half standard rate (not to exceed 15 AOB).

ENVIRONMENTAL/ PRESSURIZATION SYSTEM: (NATOPS 2-39 to 2-46, *T-44A Gouge 27-30)

1. Primary purpose of the environment system is to provide for heating, cooling, and pressurization of the aircraft.

2. Safety valve: completely relieves all pressure differential when pressure control switch in DUMP or switch is in PRESS and aircraft is on the ground.  The LH landing gear safety squat switch opens the valve on the ground.

3. Outflow valve: closed on ground.  Moving the pressure control switch to TEST opens the solenoids, closing the safety valve and giving control of the outflow valve to the controller.  The LH landing gear safety squat switch opens the switch in the air.

4. Excessive pressure bump is controlled by the LH landing gear safety squat switch which: [1] removes power from the preset and safety valve solenoids [2] actuates time delay relay that allows left engine to commence pressurization 6 seconds prior to right engine.

5. Max psid for outflow valve is 4.7psid.

6. Max psid for safety valve is 4.9psid.

7. Actual control of psid in safety valve operation is by the pressure control switch when in DUMP or the LH landing gear safety squat switch when on the ground (the safety valve solenoid opens to supply vacuum to the safety valve holding it in OPEN or DUMP position).

8. Electric heater lockout system: prevents use of electric heater if windshield anti-ice, prop deice, and/or lip boot heaters are operating.

9. Max cooling: [1] Bleed Air Valves – CLOSE one or both [2] Vent Blower – HIGH [3] All overhead outlets – OPEN [4] Manual Temp Switch – FULL DECREASE.  Max heat: [1] Electric heat – GND MAX [2] Vent Blower – HIGH [3] All overhead outlets – CLOSE  [4] Bleed Air Valves – OPEN (after engine start)

10. The electric heater will work only when CABIN TEMP MODE is in AUTO.

11. With the vent blower in auto, the fan is operating in low speed.

12. With a total loss of electrical power, the T-44 will depressurize!

13.  None of the pressurization systems work in conjunction with the RIGHT squat switch.

ENVIRONMENTAL/ PRESSURIZATION EMERGENCIES/MALFUNCTIONS:

Procedures for an altitude warning light:[1] Descend [2] Restore cabin pressurization [3] Check pressurization   and/or don O2 mask.

Loss of cabin pressurization (NATOPS 15-11):

If gradual pressurization loss, CHECK NATOPS, then:

[1] Cabin altitude – Checked (P)

[2] Pressurization controller – Checked (CP)

[3] Bleed Air – Checked (CP)

[4] Press dump test switch – TEST (CP)  (hold for 15 sec)

If test switch restores press, may be necessary to hold switch in TEST until cabin profile at 10,000’ or less.  If press is regained through test switch, pull the PRESS CONTROL CB to maintain pressurization.

If unable to restore pressurization:

[5] Oxygen mask/microphone switch (100%) – AS REQ (P,CP,OBS)

[6] Descend – AS REQ (P)

On descent when cabin alt matches press alt, ensure PRESS CONTROL CB is reset to preclude landing

pressurized.

Explosive decompression: occurs in less than 1 second, causes fog, if high alt – rush of air from lungs, dazed sensation, gas pains, hypoxia if oxygen not used immediately.

[1] Oxygen mask/ MIC switches (100%) – AS REQ

[2] Descend – AS REQ

Supplemental oxygen is required when at cabin altitudes above 10,000’.  The oxygen system capacity is 1850 (+/-50) psig for one 49 cubic foot cylinder.  Between 17,000’ to 20,000’ the diluter demand regulator should supply 100% oxygen while in the normal position.  The oxygen mask should be stored in the 100% position.  The min O2 requirement for local flights is 1000psig.  For cross-country flights: 1500psig.

Smoke and fume elimination (NATOPS 15-5): Locate, isolate, and extinguish fire or source of smoke and fumes prior to initiating smoke removal procedures.  Isolate the engine as a possible source by closing the bleed air valves.  Prior to depressurizing, consider MEA and crew O2 requirements.

Immediate smoke removal:

[1] Oxygen masks/MIC switches (100%) – AS REQ

[2] Pressurization – DUMP

After depressurizing, storm windows may be opened (may draw smoke into flight station).  Consider emergency descent and immediate landing.  Nauseating fumes from desalination solution can enter cockpit via bleed air for air-conditioning/pressurization and is often detected immediately after takeoff.  Providing no secondary indications exist, closing the bleed air valves will significantly reduce the fumes.

Setting nonstandard pressurization: [1] Round off given altimeter setting to nearest tenth (exp. 29.77 = 29.8) [2] Subtract that number from 30.4 (30.4 – 29.8 = 0.6) [3] Multiply that number by 1000 (0.6 X 1000 = 600) [4] Add that number to field elvevation (600 + 19 = 619’)  [5] set that number in latitude knob for cabin altitude.

Emergency descent procedure (FTI 1-16, NATOPS 15-12)  [PP FLAW]

[1] Power levers – IDLE

[2] Props – FULL FWD

[3] Flaps – AS REQ

[4] Landing Gear – AS REQ

[5] Airspeed – AS REQ (Don’t exceed VNE (227K))

[6] Windshield heat – AS REQ

NON-PRECISION APPROACH (VOR, TACAN, NDB):  Procedure Turns (PT): Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN, +/- 5 bearings for NDB).  Continue descent to FAF.  Configure and slow to 120K 3 nm prior to the FAF (unless SSE).  Non-precision SSE approach, don’t configure until landing assured.  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

SSE NON-PRECISION APPROACH (VOR, TACAN, NDB): Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

NO HEADING APPROACH (VOR, TACAN): (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, if his have failed as well: secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat), ask him if he is familiar with how to work the mag compass, declare emergency, squawk 7700, have copilot call out cardinal headings.

Utilize either radar vectors to final or execute a procedure turn.  Compass card is frozen but radial/bearing info from needles is still correct.  1st, determine current radial (cross reference with DME).  2nd, take radial & DME and figure out current position on an IAP or chart.  If required, make a timed turn to the VOR inbound course and fly direct to the station (using the VOR needles & the CDI).  At station passage, make a timed turn to the outbound intercept heading (Use 45 degrees is turn is > 90 degrees.  Use 30 degrees offset if turn is < 90 degrees), rotate the course select knob to the Final Approach Course & track outbound using heading info from the copilot.  Reverse course (45/180) using time turns: 15 sec for 45 degrees & 1 minute for 180 degrees.  When CDI starts to center, commence another 15 second turn to final.  To track inbound, either visualize with radials/DME or set CDI and heading bug.  Configure at normal point (3nm from FAF, unless SSE).

ADF NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

The ADF needle shows relative bearing only.  Mentally place the current heading (given by the CP from the mag compass) under the upper lubber line.  Count the number of degrees the head of the single needle deflected from the lubber line to find the relative bearing to the station.  Add (+ to the right) or subtract (- to the left) from the current mag heading to get the magnetic bearing to the station.  Mag bearing from the station found by using the tail of the needle when heading away from the station.  (Exp. If the current heading is 135, and the needle points 45 degrees to the left of the lubber line, subtract (because the needle is left) 45 from 135 to get a magnetic bearing of 090 to the station.  Unless radar vectors are given, request the 45/180 course reversal.  Turn to place the head of the needle on the nose and fly direct to the station.  Utilized known winds to make a wind-corrected heading to the station.  Continue to ask the CP for heading calls and set the heading bug on the corresponding heading to proceed direct.  Compute a heading to intercept the outbound course (the outbound course + 30 degrees [extra 10 seconds]) and determine the seconds required for a timed turn.  Immediately after station passage, commence a timed turn to intercept the outbound course.  Track outbound 1 minute using heading calls from the CP, then start a 45 degree [15 second] turn to the outbound PT heading.  After 1 minute, commence a 180 degree [1 minute] course reversal.  Inbound on the procedure turn, determine my position for situational awareness.  When the needle reaches the 45 degree benchmark, turn inbound so the head of the needle is at the top of the RMI.  Continue to ask the CP for headings.  Configure at the normal point for a non-precision approach.  Passing over the IAF and the FAF, execute the 6Ts (time, turn, time, transition, twist, talk).  If missed approach, generally requires turning to the head of the needle and flying direct back to the station.

RADIO INSTRUMENT MALFUNCTION (VOR, TACAN, NDB): (FTI 4-49) (CDI failure for VOR/TACAN): Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

VOR & TACAN TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.  VOR Station passage: TO/FROM flag changes to FROM.  TACAN Station Passage: minimum DME.

NDB COURSE INTERCEPTS/TRACKING: (RMI only): (FTI 4-19) Inbound: Course to Bearing + 30 = Intercept Bearing. Outbound: Tail to Course + 45 = Intercept Bearing. NDB Station Passage: bearing pointer passes through wingtip position.  In NDB holding, station passage is the first definite move by the bearing pointer through the 45 degree index on the RMI. Radial to Arc – lead by 0.8 nm for 90 degree turn at 150K. Arc to Radial – lead a 90 degree turn by 5 radials at 10 DME at 150K.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.