Review Stage Debrief Notes (as of 20 Jul 02)

*** Restudy these notes before every review fight.

ALWAYS CONTACT MY INSTRUCTOR THE NIGHT BEFORE A REVIEW (AND RI-28X) FLIGHT!!!

RI-24:

[1a] RI-24 through RI-28 will be flown out of the local area!!!  Common areas to work are the Valley, San Antonio/Austin, Palacios/Victoria, or Laredo.

[1b] For the “Mucho 5” departure (which must be filed with Base Ops on the phone or in person), set departure radios NAV 1 & NAV 2 up 115.5 (Corpus).  Leave Navy TACAN 87X up for emergency return to Navy Corpus.  If I need to divert back to NAVY, just flip up the TACAN and follow the magenta needle.

[2] Have the Mucho 5 and the Mucho 5 Return routing written out at the top of my notepad so I don’t have to look it up on the departure.  (If going to San Antonio, have the San Antonio out and back routing written out at the top of my notepad).  Study the routing on the chart to the Valley and San Antonio.

[3] On the way down to the valley: [A] Tell Corpus Approach that I am going to switch over to UHF freq.  Recheck in with Corpus Approach on UHF.  [B] Have the copilot get the ATIS information for all 3 airports in the Valley: Brownsville, Harlingen, and McAllen (all freqs listed in the blue brains (p.7) or on the L-16 Chart (blue freq under airport name).  Have him do the same for approaches in San Antonio (Stinson Muni(under San Antonio in IAP), San Antonio Intl, San Marcos, New Braunfels, Lackland AFB (Kelly Annex),and Randolph).  [C] Have the copilot program all the VOR and LOC freqs into the NCS-31.  Start from left to right on the chart and program the freqs left to right on the NCS-31 (top row are VOR freqs, bottom row are LOC freqs).

[4] When cleared to proceed direct to first IAF, put the radial for the IAF in the CDI (shoot a good point to point to get there!).  A good example is for the McAllen VOR 13.  The IAF is the MFE 029/13.  The Mucho 5 will take me down V20 airway on the 029 radial (just have to dial it up in my CDI and fly it down!).

[5] If shooting a ‘VOR or TACAN’ approach (such as the Brownsville VOR or TACAN approach(p. 62)) and the VOR Needle dies: [a] switch up TACAN [b] if needle working (pointing to the station), ID it [c] if good ident, continue the approach using the TACAN!

[6] Review all Valley and San Antonio approaches.  Look at each from a no-heading & SSE perspective.

[7] Don’t forget to put the ‘FRONT COURSE’ in the CDI (found in the feather) for a LOC BC approach (such as BRO LOC BC 31L, MFE LOC BC 31, or HRL LOC BC 35L).

[8] Utilize Flight Sim 2002 to fly NDB No Heading approaches to every airport in Valley and in San Antonio.

[9] Don’t forget to put “Props full forward, review me complete” when on final, landing assured, and in a safe position to land.

[10] Make sure that I have the proper frequencies in NAV 1 and NAV 2 for the approach I am shooting (at McAllen I had the VOR freq in both navaids when I was shooting an ILS approach!).

[11] Review the NCS-31 set up in the AIGT and my systems notes.

[12] If I get a flameout on short final, say “We will handle the emergency on the deck, you have the coms, this will be a full stop. Tell my wife I love her.”

[13] For any precautionary shutdown (chip light, oil press undesirable, etc...), must Preload the Starter Assisted Checklist up to #12.

[14] Review PAR/ASR (normal, SSE, and no-gyro) approaches

[15] Must do 6Ts over every fix!!!!  Time, Turn, Time, Transition, Twist, Talk.  Talk to tower on final if I haven’t already been cleared to land/touch and go/option.

[16] If I get a generator out light: “Off, Reset Momentarily, Then On.”  If it comes back on line, then continue the mission.

[17] Always be aware of my position on the chart in case I need to divert!

[18] If a prop maintains the its last setting when I move it or goes to 2200RPM, it is a Prop Linkage Malfunction.  If it is within limits (1800 – 2000 RPM), then adjust the other prop to match it and say “If we have to shut this engine down, we will use the Alternate Prop Feathering Checklist.”  Continue with the mission and land as soon as practicable.  To feather a prop with a prop linkage failure, use the Alternate Prop Feathering Checklist.

[19] Make sure to practice ILS/LOC approaches where I will have to DUD and USE, USE!

[20] When briefing an approach, always say what I will back it up with!

[21] For ‘Instruments/Nacelles’ check, don’t forget to actually check the engine instruments!!!

[22] If using an ADF for an approach, always get the observer to continuously monitor it!

[23] If shooting an approach and it changes (ILS to a LOC, VOR to a TACAN, VOR/TAC/NDB to a CIRCLE, etc...) rebrief the altitudes & courses!

[24] For a visual approach: brief the CP on what type of approach I am going to back it up with and when I am going to dirty up.

[25] Do an in-depth study of how to transition from the BRO ILS 13 to the HRL VOR-A (full PT).  It is the quickest turn around in the Valley!

RI-25:

[1] For normal PAR, configure on dogleg.  If PAR is first approach, check fuel to see if the aircraft is light enough for a touch and go.  Normally, I have to do a low approach because the plane is too heavy (too much fuel).  Write down the min fuel amount needed to make a touch and go (put it on my palm).

[2] If I get the ‘CABIN ALT WARN’ light: (1st) Check the cabin altitude!  If it’s 10,000 or below, don’t have to do anything!  If above 10,000’, ask the CP to give me the loss of pressurization checklist.  Options: descend & restore pressurization, or don oxygen masks and continue mission (have to get into checklist on p. 2-49 to see how much O2 is available).

[3] If I get a ‘CABIN DOOR WARN’ light: (1) Have everyone stay seated with seatbelts fastened (2) Depressurize (3) Land as soon as practicable and have door locks checked.

[4] If I get a ‘CROSSFEED’ light: (1) Crossfeed – CLOSE (2) Determine which boost pump has failed by the illuminated ‘FUEL PRESSURE’ light (3) Failed Boost Pump – OFF (4) Crossfeed – OPEN (5) Determine range from fuel left in side crossfeeding from (6) Land as soon as practicable.

[5] If ‘MOVEOFF’: Mechanical malfunction, Overheat, Vibration, Explosion, 0 N1, Fire, Fuel fumes: don’t attempt an airstart.

[6] Review IFR Weather Filing Criteria in Blue Brains.

[7] ***  When flying an approach, MUST do 2 essential things that I keep forgetting: [1st] Do TIMSS: Tune, Identify, Monitor, Select (FD Select Switch either VOR/ILS or TACAN), Set (CDI to inbound course & needles to ADF or VOR)!!!  [2nd] Make sure all radios/navaids are set.  To ensure I have set ALL the radios/navaids I might need, GO ACROSS THE TOP ROW OF NAVAID SWITCHES (just like when setting Radios/Navaids in Takeoff checklist) AND VERIFY I GET ALL AVAILABLE NAVAIDS!!!  These 2 items are crucial for IFR!  Make them my mantra!!!

[8] If no DME on a non-precision approach, configure as soon as I turn inbound on the final approach course!

[9] Don’t ever forget to put “Props full forward, 3 down and locked” before landing again!

[10] If in a holding pattern and need more time to descend to the approach altitude, ask ATC for one more turn in holding!

[11] Port Lavaca NDB – must descend as quickly as possible to MDA!

[12] If on visual and VERY HIGH on approach: (1st) Pull power levers to IDLE (2nd) Pull up enough to slow below 174K and put FLAPS- APPR (3rd) Once the plane slows to below 155K, extend the gear (4th) Once below 140K, Flaps – FULL (5th) Dive towards the deck and get back on glide slope path (3 degrees) (6th) Once on glide path, reestablish normal approach, putting Flaps – APPR.  Don’t forget “Props full forward, Review me complete!”

RI-26:

[1] TERPS establishes 152’ for obstacle clearance on takeoff climb.

[2] ***  If on a non-precision approach and lose an engine, must CLEAN UP.  Don’t just go through the motions on the “Power – UP, Rudder – UP, CLEAN – UP, Speed – UP, Checklist.”  Really think about each step!  Non-precision -  when I get to CLEAN – UP, ask myself what type of approach am I shooting and whether I should raise the flaps and gear.

***Good gouge on CLEAN – UP:

Precision – ILS: 0.5 dots = configure.  PAR: 10 second warning = configure (treat ASR the same as PAR).

Non-precision – Landing assured and in a safe position to land = configure.

Circling – 180 position = configure.

If I am full power on one engine and unable to hold altitude and speed, good indication I should raise the flaps and gear!!!

[3] If I get a Chip Light, do a precautionary shutdown, then preload the Starter Assisted Checklist up to #12.

[4] *** For “Instruments/Nacelles” in the Climb Checklist, MUST also CHECK THE ENGINE INSTRUMENTS!!!

[5] If instructor says “I smell smoke, but I can’t tell where it’s coming from,” (1st) Troubleshoot to find the source of the smoke (2nd) Initiate the Smoke/Fire of Unknown Origin Checklist (on the Lamenated Checklist) (3rd) Initiate the Smoke and Fume Elimination Checklist [1] Oxygen masks/ Mic switch (100%) – AS REQ [2] Pressurization – Dump.

[6] Review procedures for selecting Flaps – APPR and having the flaps remain in UP or having a faulty flap gauge.

[7] If rudder is sticking on takeoff roll, or anything doesn’t feel right, troubleshoot immediately.  Look for any switches in the wrong position (for rudder stick, make sure the Yaw Damp switch is OFF), check the CBs and Godbox.

[8] Always double check my heading on the runway to ensure it’s aligned with runway heading!

[9] For a PAR approach at Navy, back it up with the ILS (114.0 D-U-D (for DME), 110.3 Use, Use (for LOC))!

Pocket Notes:

[1] To shoot a LOC with TACAN channel (without a paired freq listed), such as the approach into Orange Grove, CP has to shoot approach.  Why will it work on CP’s side and not mine?  Because, DME is tuned only through the TACAN or NAV 1.  NAV 1 can receive LOC and DME.  NAV 2 CAN’T receive DME!  The P CDI works off NAV 1 freq & the CP CDI works off NAV 2 freq.  Therefore, as P, I can’t tune up LOC in NAV 1 and put the paired freq in NAV 2 (NAV 2 won’t accept a DME paired freq).  So, to shoot a LOC approach with a paired freq, I must put the DME paired freq in NAV 1 (only radio which will accept a paired freq) and the LOC freq in NAV 2 (NAV 2 works the CP CDI) which allows the CP to shoot the approach.  Very important to remember for Orange Grove ILS/DME 13 and Randolph ILS Z 14L.

[2] Jammed Power Lever: [a] On Deck: Check Yaw Damp and Autopilot – OFF, leave controls in jammed position, call maintenance for immediate inspection. [b] In Flight: Check engines instruments and nacelles for secondary indications.  If no abnormal secondary indications, consider keeping the ENGINE RUNNING unless controllability is a factor.  Land as soon as practicable.  Prior to landing: (1) Condition Lever (failed engine) – FUEL CUTOFF (2) Emergency Shutdown Checklist – EXECUTE.

[3] If I select Flaps – APPROACH & the Flap Indicator shows the Flaps are still up: (A) Visually check the position of the flaps on my side and have the CP check the position of the flaps on his side (B1) If the flaps are actually in the Approach position, ask the CP to pull the FLAP MOTOR CB and land to a full stop. (B2) If the flaps are actually in the UP position, reset the Flap Lever in the UP position, ask the CP to pull the FLAP MOTOR CB, rebrief the new landing speeds (110K Final & 105K over the Threshold), and land to a full stop.