Review Stage Gouge Answers (18 Jul 02)

# *** MUST CALL IP THE NIGHT BEFORE REVIEW STAGE FLIGHT TO GET HOMEWORK!!!

# GENERAL KNOWLEDGE

1. 3710 filing requirements for Alternates and Departure alternates:

## IFR FILING CRITERIA

DESTINATION (+/- 1 HR ETA)			ALTERNATE (+/- 1 HR ETA)

0-0 up to BNI pub mins				3000 – 3 or better

Pub mins up to BNI 3000 – 3				NON = Pub mins + 300-1

PRE  = Pub mins + 200-1/2

3000 – 3 or better					N/A

2. Letter of Agreement and FIH procedures for Lost Comms:

LOA LOST COMM:

VFR: [1] Squawk 7600 (lost comm) [2] Get ATIS (CH 1).  ATC may contact me on ATIS freq [3] If unable to get ATIS (and wx permits), course rules back to Navy Corpus > 3,500’MSL to determine duty runway [4] Make all radio calls “in the blind.”  On approach, rock wings inbound for the break and check interval. [5] On short final, if no aldis lamp signal observed, waveoff.  On 2nd pass, in no lamp signal and runway clear, land.

IFR: Immediately after takeoff: [1] Climb on last assigned heading/route assigned by ATC to 1,500’MSL [2] Wait 3 minutes while trying to contact Corpus Departure.  If can’t establish comm in 3 minutes, climb and maintain 2,500’MSL [3] Reaching 2,500’MSL proceed directly to an approach fix for NAS Corpus and execute an approach.

In the GCA pattern: no comm for 1 minute, proceed direct to approach fix for Navy Corpus and execute an approach.

On Final: no comm for 15 sec (ASR) or 5 sec (PAR), alter course to intercept the VORTAC final approach course for runway in use and, if practicable, execute a TACAN/VOR approach; otherwise, climb and maintain 1,500’MSL, proceed direct to an approach fix for NAS Corpus and execute an approach.

# FIH LOST COMM: (FIH A-6 to A-7)

# In lost comm situation, can shoot any approach.

# VFR:  [1] Troubleshoot radios, headsets, and mike switches (check circuit breakers, transmit on UHF or VHF, change frequencies, transmit on Guard) [2] Squawk 7600 [3] Make radio calls in the blind [4] Tune up Guard and nearest VOR with voice capabilities (if applicable) ***[5] If able to remain VMC, continue flight under VFR, land as soon as PRACTICABLE (not possible), and notify ATC.

# IFR: [1] Troubleshoot radios, headsets, and mike switches (check circuit breakers, transmit on UHF or VHF, change frequencies, transmit on Guard) [2] Squawk 7600 [3] Make radio calls in the blind [4] Tune up Guard and nearest VOR with voice capabilities (if applicable) [5] Route: [AVE-F] In priority order: Assigned, Vectored, Expected Further Clearance, Flight Planned. [6] Altitude: [AME] The highest of the following for the route segment flown: Assigned, MEA, Expected Further Clearance.  If an EFC is given at a point where an approach begins and then comm is lost, start the approach at EFC.  If comm is lost at a point where an approach is not begun, leave the point at the EFC (if given).  If an EFC is not given, continue to approach point and start approach as close as possible to the time filed or amended with ATC enroute.

# 3. Know the meaning of the different colors and symbols on high and low charts:

# 4. Know the differences between the high and low charts:

# Low Charts: [1] If an approach plate lists DME/ADF required, 80% of the time that means DME or ADF is required to shoot the missed approach [2] The 10 mile circle indicates that everything within the circle is to scale [3] Airports shown in blue and green have an approved Low Alt Instrument Approach Procedure.  Airports in blue have an approved DOD Low Alt Instrument Approach Procedure.

# 5. Know where to find Flight Watch freqs on high charts:

6. Know about icing conditions and what we can fly in (we cannot fly into Moderate Icing):

3710: Circumvent forecast icing and thunderstorms whenever practicable.

NATOPS: Flights through icing conditions should be avoided if possible.  If severe icing encountered, climb or descend to altitudes where these conditions do not prevail.

VT-31 SOP: Pilots shall not fly through areas of know or suspected moderate or severe icing.  Areas with forecast or light icing shall be circumnavigated whenever possible.  Flights shall not be filed into airfields with present or forecast (including RONs) snow or ice covered runways.  Ground ops on snow or ice are prohibited.

7. Know about fuel reserves:

FUEL PLANNING: (FTI 4-7)  If alternate not required, must have fuel to fly to destination, plus a 10% reserve (max endurance at 10,000’).  If alternate required, must have fuel to fly to destination IAF and then to alternate airfield, plus a 10% reserve (NATOPS C. 25).  In no case shall reserve be less than 20 minutes.  Minimum on-deck fuel is 265 lbs per side.  Minimum fuel is advisory only (emergency could occur if any undue delay occur).  Min fuel is normally 265 lbs per side (yellow arc).  If min fuel, consider requesting short vectors to final if IMC or visual landing.  If fuel state requires traffic emergency, declare an emergency due to low fuel and report fuel remaining in minutes.  Emergency fuel typically called when 30 minutes of fuel remain (no set amount, can vary).

8. Weight and Balance/ Flight planning: [1] Weight and Balance: refer to “The Definitive Weight and Balance Gouge” in notebook [2] Flight Planning: refer to FLIP, General Planning (Chap. 4)

9. Know what a Diverse Departure is:  (FTI 4-14)Aircraft departing an airfield can ensure obstacle clearance.  To fly it: fly runway heading until 400’ above field elevation before executing any turns while maintaining a minimum climb gradient of 200 ft/nm (unless higher is published) until reaching minimum IFR altitude.

10. Know what the “Trouble T” means: (FTI 4-14)The Trouble T indicates the airport has NONSTANDARD takeoff weather minimums as well as an IFR Departure Procedure.  This information is located in the front of DOD Approach Plates in the section titled “IFR Takeoff Minimums and (Obstacle) Departure Procedures.”  These minimums DO NOT apply to USN aircraft.  The 3710 establishes USN weather minimums!

11. Study the annunciator panel lights and what brings them on: (NATOPS 2-36 – 2-39)

Powered by the No. 1 subpanel bus.  Protected by a circuit breaker placarded ANN LIGHTS and located in the INDICATOR section of the copilot outboard subpanel.

(1) STALL WARN (Red) – approaching stall condition.  Reduce angle of attack

(2) WHEELS UP (Red) – landing gear not extended.  Lower gear.

(3) FAULT WARNING (Red) – fault condition requiring corrective action.  Press light to reset.

(4) NO SMOKING FASTEN SEATBELTS . Pilot operated.  Shows switch is on.

(5) LH & RH GEN OUT (Red) – affected generator offline.  Turn affected gen switch ‘off’, reset it, and turn it ‘on.’  If the gen won’t reset, turn the gen ‘off.’  Monitor the load (don’t let it exceed 1.0).  Normal voltage in the ‘reset’ position indicates failure of the generator control box rather than the generator.  Land as soon as practicable.

(6) LH & RH FUEL PRESSURE (Red) – affected side has a failed boost pump.  A boost pump failure (w/ crossfeed in ‘auto’) will be noted by illum of yellow xfeed annunc lt.  ID the failed boost pump by momentarily closing the crossfeed switch in ‘closed.’  The fuel pressure light which illuminates is the failed boost pump.  Switch the failed boost pump ‘off,’ place the crossfeed switch to ‘open,’ and determine if the flight can be continued by referencing Section V of Fuel System Failure.

(7) LH & RH NO FUEL TRANSFER (Red) – affected side has a failed fuel transfer pump or all usable fuel has been transferred from the wing tanks.  Ensure all fuel was transferred from the wing tanks by checking fuel quantity (check total and nacelle readings – if the same, no fuel in the wing).  If fuel remains in the wing tank, try fuel transfer ‘override’ position (transfer pump continuously energized, float switches bypassed, and 3psi pressure switch reset).  If light remains on, place transfer pump switch to ‘off.’  Loss of transfer pump – fuel will gravity feed when nacelle tanks 3/8 full and 28 gallons of fuel will be trapped in the center section wing tank.  Consider altering flight plan due to unavailable fuel trapped in wing.  Monitor nacelle fuel quantity and land as soon as practicable.

(8) #1 & #2 INVERTER OUT (Red) – affected inverter inop.  Place affected inverter switch off.  Check AC bus switchover.  Consider landing as soon as practicable.

(9) AUTO PILOT TRIM FAIL (Red) – trim servo will not operate or operates in opposite direction commanded.  Depress the AP/YD disengage switch and retrim the aircraft.  Reengage the autopilot.

(10) INST INV OUT (Red) – loss of 26 VAC Stepdown Transformer. Light only illums if both 26 VAC stepdown transformers fail. Torquemeters inop.  Utilize ITT and N1 RPM for power changes.

(11) PROP SYNC ON (Yellow) – landing gear has been extended with prop sync still on.  Turn prop sync ‘off’ before landing.

(12) CABIN DOOR OPEN (Red) – cabin door not secure.  Don’ attempt to close the door or check security of lock.  Insure all passengers and crew members are seated with seatbelts fastened.  Depressurize the cabin.  Land and determine the door pins are properly locked into the door frame.

(13) FUEL CROSSFEED (Yellow) – electronically operated crossfeed valve open.  Determine if boost pump failed by placing the xfeed valve switch to ‘closed.’ If a boost pump failed, follow the procedure for boost pump failure: [1] Failed boost pump – off [2] crossfeed – open [3] land as soon as practicable.   If not, reset the crossfeed valve to the ‘auto’ position.

(14) BAGGAGE DOOR OPEN (Red) – baggage door (in front avionics compartment on pilot side) open or not secure.  On ground, secure the door.  Inflight, land and secure the door.

(15) LH & RH FIRE (Red) – engine compartment fire on affected engine.  Confirm fire actually exists by checking engine instruments and nacelle.  Even if no secondary indications observed, shutdown the engine.  If fire confirmed, perform EMERGENCY SHUTDOWN CHECKLIST: [1] Power lever – Idle [2] Propeller – Feather [3] Condition lever – Fuel cutoff [4] Firewall valve – closed [5] Fire extinguisher – as required [6] Bleed air – closed [7] Dead engine checklist – as required.

(16) LH & RH IGN IND (Yellow) – confirms igniter system receiving electrical power.  In Auto Ignition mode, light extinguishes when torque < 410 +/- 50 ft-lbs or switch placed in ‘off.’  In normal start mode, light extinguishes only when IGN & IGN START switch placed ‘off.’

(17) LH & RH AUTO FEATHER ARMED (Green) – autofeather system armed.  If system is armed power levers > 90 N1 (+/-2)%, props feather. Light extinguishes if either prop autofeathers or if system is disarmed by retarding one power lever (opposite side extinguishes at 410 +/- 50 ft-lbs and same side extinguishes at 260 +/- 50 ft-lbs) or autofeather switch is placed ‘off.’

(18) LH & RH CHIP DETECT (Red) – contamination of affected side’s engine oil.  Metal particles may be present in the prop reduction gearbox.  Perform the EMERGENCY SHUTDOWN CHECKLIST: [1] Power lever – Idle [2] Propeller – Feather [3] Condition lever – Fuel cutoff.  Proceed to the DEAD ENGINE CHECKLIST.

(19) LH AUTO IGNITION ARMED (Green) – appropriate autoignition system armed.  Light extinguishes when torque reaches < 410 +/-50 ft-lbs or auto ignition switch placed ‘off.’

(20) ALT WARN (Yellow) – cabin altitude exceeds 9,500 to 10,000 feet.  Descend to lower altitude.  Restore cabin pressurization.  Check pressurization control and/or don oxygen masks.

(21) PROP REV NOT READY (Yellow) – Props not in high RPM with landing gear handle down.  Place prop levers full forward.

***12. Know all EP procedures.

13. Review FTI thoroughly (particularly RI stage/section 4).

***14. Study every approach from a no heading/ SSE perspective!