RI-16S Study Guide (02 Jul 02)

ILS/LOC CHARACTERISTICS: (FTI 4-42, AIGT 2-2, 2-17, 8-18, AIM 1-1-6) In the U.S., the required components for an ILS approach: [1] glide slope [2] localizer [3] outer marker.  If no outer marker, it can be replaced by DME, radar, another navaid, or a crossing radial.  If VOR 2 is used to identify intermediate fixes and/or the FAF, tune it to the localizer freq no later than the FAF, unless used to define subsequent stepdown fixes or the MAP.  If the glide slope fails, transition to a non-precision localizer approach (SSE, raise the flaps and gear and treat like a non-precision SSE approach (not lowering the flaps or gear until landing assured)).  If the localizer fails, can’t fly the approach.  If the OM (or one of its substitutes) isn’t available, can’t fly the approach.  Intercepting an ILS/LOC approach is either by radar vectors on the published approach procedure.  While being radar vectored, use any available navaid (exp. TACAN, VOR/DME) to aid in position orientation.  The TACAN select switch must be set in “OPER” to receive DME!  The glide slope’s usable range is 10 miles (however, it can be picked up much further out).  The localizer’s usable range is at least 18 miles within 10 degrees of centerline (35 degrees from centerline out to 10 miles).  The localizer may be used beyond 18 miles, but only if under ATC radar guidance. On a normal front course localizer, the localizer antenna is usually located beyond the departure end of the runway, therefore, on a LOC BC approach, it is located prior to the approach end of the runway, making a LOC BC approach much more sensitive than a normal LOC approach.  False glide slope signals may exist in the area of a LOC BC causing the G/S warning flag to dissapear.  Disregard all G/S indications on a LOC BC approach unless a G/S is specified on the IAP.  At FAF, always hack the clock – timing is always a good back up!   Localizer Backcourse – ALWAYS set in the FRONT COURSE on the CDI  (the ‘front course’ refers to the inbound course on the ILS/localizer approach for the OPPOSITE runway.  On the back course approach plate, the published front course is depicted in the feather as an outbound localizer course).  Avoid reverse sensing on a LOC BC by twisting in the front course.  Approaching the final course (on base or arc) perform the ‘LIDS’ check – [1] Localizer freq set in (VOR/ILS selected on FD SEL switch, monitor the ident) [2] Inbound front course set in the CDI [3] DME (tune the TACAN to the localizer associated with the DME freq) [4] Speed and Configuration (slow to 120K and configure when: [1] 1.5 dots of intercepting glide slope on ILS [2] 3 nm prior to FAF on localizer).  Intercepting the localizer – keep heading changes to 5 degrees or less.  Discontinue the approach when localizer course is unreliable or full scale deflection on CDI on final.  Descent – Maintain glide slope intercept altitude (published or assigned) until intercepting the glide slope (identified by a lighting bolt).  On glide slope, crosscheck aircraft altitude with published “Glide slope Altitude at OM/FAF” to ensure established on correct glide slope.  Don’t descend below descent restrictive altitude if CDI indicates full scale.  The glide slope isn’t the primary aid for vertical flight path until the lightening bolt on the IAP.  However, if established on localizer course and within usable range (10 miles) of glide slope, it can be used in descent to glide slope descent altitude.  Glide slope Indicator (GSI) – intercept the glide slope as the GSI moves downward from its upper limits.  Slightly before the GSI reaches the center position, coordinate pitch and power to maintain proper rate of descent (normally 600 feet on a 3 degree glide slope, set power at 350ft-lbs at 120K).  On the indicator, the glide slope is the green pointer & the center line is the aircraft).  Make pitch adjustments in 2 degree or less increments.  On the final portion of the approach, make 1 degree pitch corrections and heading corrections of 5 degrees or less.  Decision Height (DH) is determined by the barometric altimeter.  Don’t descend below localizer minimums if the aircraft is more than one dot (half scale) below or two dots (full scale) above the glide slope.  If glideslope is corrected back to within tolerances, descent can be continued to DH.  At decision height, must immediately decide whether to continue approach or go missed approach.

Flight Director: The Pilot’s CDI can show either VOR 1 (including LOC) or TACAN.  The copilot’s CDI shows only VOR 2 (including LOC).  When tuned to localizer, the TO/FROM flag doesn’t operate & the course arrow setting doesn’t affect the deviation bar.  Marker Beacon comes on when the VOR is turned ON (put P to sensitivity to ‘LOW’ and CP to ‘HIGH’ and turn MB audio switches ON).  LOC/ILS sensitivity on CDI = 2.5 degrees for full scale deflection (5 degrees full deflection for VOR & TACAN).  “HEADING” Warning Flag indicates failure of compass system.  “NAV” Warning Flag indicates NAVAID signal is unreliable.  The center of the glideslope scale (the line) represents the aircraft and the green pointer represents the glide slope.  (For exp., if the pointer is above the center line, the aircraft is below the glide slope, and vice-versa).

***NCS-31:  If need DME from a NAVAID other than the LOC on an ILS/LOC approach: [1] Enter the DME freq and D-U-D [2] Enter the LOC freq and press the USE button twice (USE, USE).

Marker Beacons: Outer, Middle, and Inner.  MB lights begin to flash when in almost directly over a MB.  The lights are accompanied by a series of aural tones (- - - for Outer [Blue]/ - . - . – for Middle[Amber]/ . . . for Inner[White]).  Proper configuration for MB is P turn on audio for MB 1 and set sensitivity to LOW, CP turn on audio for MB 2 and set sensitivity to HIGH.  MB are not tuned (always transmit on 75 MHz).

ILS approach: First portion of ILS approach generally has a large turn (procedure turn or teardrop approach, off a compass locator, or radar vectors to final).  The actual ILS portion begins when joining the final approach fix inbound.  Must be configured prior to reaching glide slope intercept point (or FAF).  At DH, must either go missed approach or continue landing.  DH based on altitude, not position.  Timing is not used on an ILS approach.  However, if glideslope fails, must revert to LOC appraoch (which may use timing).  So, always back up approach by starting clock at glideslope intercept!  If “COURSE” Warning flag is displayed on final approach, initiate a missed approach!  If “GLIDE SLOPE” Warning Flag is displayed on final approach, don’t fly lower than localizer minimums, or (if no localizer mins published) no lower than circling mins.  SSE precision approach (ILS), Flaps – FULL and Gear Down Landing Checklist 0.5 dots below glidepath intercept.  SSE non-precision approach (LOC), Flaps – FULL and Gear Down Landing Checklist when landing assured and in a safe position to land.  When ceiling < 800’ and/or visibility < 2 miles (and landing aircraft is between ILS FAF and airfield) departing aircraft must hold short of the ILS CRITICAL AREA!  When ceiling < 800’ and/or visibility < 2 miles, aircraft can’t hold below 5,000’ between the outer marker and the airfield.

Compass Locators: (CLs)  Often located at OM and MM.  Range of at least 15 miles.  At high powered locations, CLs also have TWEB info.  OM CLs transmit first 2 letters of localizer ident.  MM CLs transmit last 2 letters of localizer ident.

STARS: (AIM 5-4-1)  Standard Terminal Arrival (STAR).  Civil STARS are in the SIDs/STARs manual.  Military STARS are in the FLIP Terminal (high & low altitude).  Civilian STARS are published in the Terminal Procedures Publication (TPP).  Before departing an airfield, determine if a STAR is published or a Departure Procedure (DP) is available for obstacle clearance.  If I don’t want to fly a STAR, notify ATC by writing “NO STAR” in the remarks section of my flight plan or (the less preferred way)by verbally stating it to ATC.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

VOR & TACAN TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.  VOR Station passage: TO/FROM flag changes to FROM.  TACAN Station Passage: minimum DME.

HOLDING:  When ATC issues holding clearance, the direction they give points TO the holding fix (exp.  hold west of the NGP VORTAC means the arrow of the inbound course is FROM the west).  Always get an Expected Further Clearance Time (EFC), it determines course of action in a lost comm situation.  Standard holding means all turns to right.  Non-standard holding means all turns to left.  Make all turns either standard rate or max of 30 deg AOB.  If must tack 10 deg into wind on inbound leg, must tack 30 deg (3 times as large a correction) in opposite direction on outbound leg.  CAN’T hold over a TACAN station. However, can hold on DME & radial off of a TACAN station.  If extended holding, consult NATOPS for max endurance speed.  Request extended legs if delay exceeds 20 minutes.  ATC should issue holding instructions 5 minutes prior to reaching clearance limit fix.  Always request an EFC prior to accepting a holding clearance if one was not given!

Timing:  Begin outbound timing over/abeam the fix, whichever occurs later.  If abeam position can’t be determined, start timing when turn outbound is complete.  < 14,000’ MSL, time 1 minute. > 14,000’ MSL, time for 1 ½ minutes.  Commence inbound timing at wings level.

Speed: Recommended T-44 holding speed is 150KIAS.  Max holding speed is 175KIAS for turboprop aircraft.  Expect ATC holding clearance 5 minutes prior to arriving at holding fix.  Start a speed reduction to 150K (holding airspeed) 3 minutes or less from the holding fix.

Holding Entry:  Reaching the fix, execute the 6 T’s: Time, Turn, Time, Transition, Twist, Talk

Time: Note the time of station passage (VOR: reversal of TO to FROM)

Turn: Parallel, Teardrop, or Direct: determined using the outbound course on the RMI card.  Use pencil method.  If standard, put pencil on 90deg mark on RMI with tip pointed to right & turn 20 deg UP.  Relative position of 360 to 070 = Teardrop.  Relative position 360 to 250 = parallel.  All headings below the pencil are direct.  If nonstandard, put pencil on 90 deg mark on RMI with tip pointed to left & turn 20 deg UP.  Relative position 290 to 360 = teardrop.  Relative position from 360 to 110 = parallel.  All headings below the pencil are direct.

Parallel:  maneuver to outbound course (non-holding side) for one minute, then turn toward holding

pattern inbound course.

Teardrop: Subtract 30 deg from outbound course for standard & add 30 deg for nonstandard.

Direct: Turn in direction of outbound course.

Time: Time first leg outbound (1 min or 1 ½ min)

Transition: slow to 150KIAS

Twist: put in inbound holding course

Talk: Tell ATC established in holding (not required to inform ATC of holding when in a training environment at a military facility).

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, unable to make safe landing, directed to do so by controlling agency, or instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.