RI-1S Study Guide (28 May 02)

__ IFR Communications (Appx. A)

__ IFR Briefs (Appx. A & AIGT, Chap. 8)

INSTRUMENT/ GEOGRAPHIC ORIENTATION: ???

EMERGENCY/ MINIMUM FUEL STATE: (FTI 4-7)

Minimum Fuel: advisory only – indicates that emergency possible if any undue delay should occur (indicated by needles within yellow arc).  If need traffic priority, declare emergency due to low fuel and report fuel remaining in minutes.  Emergency Fuel: varies, declared with approx. 30 minutes of fuel remaining.

FLIGHT INFORMATION HANDBOOK (FIH): Review Table of Contents/Index to know what information is provided in FIH.

LOST COMMUNICATIONS: Refer to FIH (Section A), FAR 91.185, & Letter of Agreement (Copy of Procedures in ‘Blue Brains’)

CLEARANCE COMPLIANCE: Comply with last verbal instructions given by ATC (read back all instructions: headings, altitudes, and altimeter settings).

RADIO COMMUNICATIONS: Refer to RI-0  Brief (pages 5-7)

# PAR APPROACH (NORMAL): (NATOPS 18-4, FTI 4-41) Refer to Normal PAR Approach notes (27 May 02)

PAR APPROACH (PARTIAL PANEL or RADIO INSTRUMENT MALFUNCTION): (FTI 4-46) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.  *Request no-gyro vectors to a no-gyro PAR or ASR!  Turns up to final = SRT (not to exceed 30 AOB).  On final (after controller calls for half SRT), all turns half standard rate (not to exceed 15 AOB).

PAR APPROACH (SINGLE ENGINE): (NATOPS 16-3, FTI 4-41)  Initiated from an Engine Fire in Flight.

ENGINE FIRE IN FLIGHT (NATOPS 15-4): Illum of FIRE warning light.  Confirm by checking eng instruments and nacelles.  Even if no secondary indications, consider shutdown by EMERGENCY ENGINE SHUTDOWN checklist.

(POWER UP, RUDDER UP, CLEAN UP, SPEED UP, Checklist)

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

“Sir, you have the comms, declare an emergency, this will be a full stop landing, give me the Dead Engine Checklist.”

[1] Handle emergency [2] Declare emergency [3] Squawk 7700 [4] If fire extinguishes, turn towards nearest suitable landing field and land.  [5] If fire will not extinguish, perform immediate forced landing or power-on ditch..

SINGLE ENGINE APPROACHES: (NATOPS 16-3, FTI 4-39)  After engine failure, determine if windmilling airstart should be attempted (inadvertent shutdown) or ‘pre-loading’ engine for starter-assisted airstart (don’t perform if failure from MOVEOFF).

Engine failure on climbout: Remain VMC if possible, declare emergency, state intentions.  If IMC, request PAR or ILS.  Provide souls on board, fuel remaining (time), and nature of emergency.

SSE PAR: Request “Ten second gear warning prior to descent” from final controller to aid in configuring aircraft.

Recovery: (Power UP, Rudder UP, Clean UP, Speed UP [102K or 110K], Checklist)

1. Power – AS REQ (max on operating engine)

2. Gear – UP

3. Airspeed – AS REQ (at 102K, raise nose to stop altitude loss & accelerate to 110K if possible)

ID failed engine using the engine instruments (torque, ITT, N1,  fuel flow) and rudder.  The foot working hard to maintain hdg is on the same side as the operating engine (Dead Foot (foot back) = Dead Engine).Don’t look at the power levers to determine failed engine (won’t have this luxury in an actual emergency)!

4. Emergency Shutdown Checklist – Execute

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

(On Climbout): After the first 3 memory items:

(1) Props – 1900RPM

(2) Power – Max allowable (2nd redline torque)

(3) If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

“Sir, give me the dead engine checklist, you have the comms, declare an emergency, this will be a full stop.”

ASR APPROACH (NORMAL): (NATOPS 18-5, FTI 4-42) Request controller provide recommended altitudes on final.  Controller will tell pilot approach to fly, MDA, MAP location, and advance notice of when descent to MDA begin.  Optimum final descent gradient = 600fpm (3 degrees at 120K) on VSI.

Single Engine ASR: treat as SSE precision approach for configuration.  If get well below recommended altitudes, revert to non-precision configuration, retract flaps and gear to get at or above MDA.  If not going to use recommended altitudes on final, treat as non-precision approach.  Slow to 120K after configuration point, but before descent to MDA, gear should be left in UP.  Continue as per FTI-4-39 ‘Single Engine Non-Precision Approach.’

ASR APPROACH (NO HEADING/ RADIO INSTRUMENT MALFUNCTION): (FTI 4-42) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.  *Request no-gyro vectors to a no-gyro PAR or ASR!  Turns up to final = SRT (not to exceed 30 AOB).  On final (after controller calls for half SRT), all turns half standard rate (not to exceed 15 AOB).

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.

IFR DEPARTURES: (FTI 4-13 to 4-17)  Refer to FTI highlight notes.