RI-11 Study Guide (20 Jun 02)

__ IFR COMM (KNEEBOARD)

__ IFR BRIEFS (APPX. A & AIGT CHAP. 8)

__ LOCAL DEPARTURE INSTRUCTIONS (BLUE BRAINS)

__ CHECKLIST PROCEDURES (PARTICULARLY ENGINE RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES GUIDE (DEPARTURES & ARRIVALS TO NAVY CORPUS)

__ EMERGENCY PROCEDURES

__ POCKET NOTES

TACAN NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

Utilize either radar vectors to final or execute a procedure turn.  Compass card is frozen but radial/bearing info from needles is still correct.  1st, determine current radial (cross reference with DME).  2nd, take radial & DME and figure out current position on an IAP or chart.  If required, make a timed turn to the VOR inbound course and fly direct to the station (using the VOR needles & the CDI).  At station passage, make a timed turn to the outbound intercept heading, rotate the course select knob to the Final Approach Course & track outbound using heading info from the copilot.  Reverse course (45/180) using time turns: 15 sec for 45 degrees & 1 minute for 180 degrees.  When CDI starts to center, commence another 15 second turn to final.  To track inbound, either visualize with radials/DME or set CDI and heading bug.  Configure at normal point.

LOST COMMUNICATIONS: TERMINAL: (FIH A-11)

Observe the tower for light signals.  Acknowledge signals in daytime by movement of ailerons or rudder on the ground and by rocking wings in the air.  Acknowledge signals at night by flashing airplane lights.  Signals

COLOR & TYPE SIGNAL		ON THE GROUND		IN THE AIR

Steady Green			Cleared for take-off			Cleared to land

Flashing Green			Cleared to taxi			Return for landing

Steady Red			Stop				Give way to other aircraft and circle

Flashing Red			Taxi clear of landing area		Airport unsafe – don’t land

Flashing White			Return to starting point		N/A in U.S.

Alternating Red & Green		Extreme Caution			Extreme Caution

Red Flare				N.A.				Don’t land for time being

GPS CHARACTERISTICS: (AIM 1-1-33)  U.S. satellite-based radio nav positioning and time transfer system run by the D.O.D.  Two levels of service: [1] Standard Positioning Service (SPS): accurate to 100 meters or less (95%) and 300 meters or less (99.99%). [2] Precise Positioning Service (PPS): more accurate than SPS, limited to authorized U.S. and allied military, gov, and civil users.  GPS receiver measures distance from a satellite through a radio signal, creates a ‘pseudo-range’ (which is subject to isopheric and troposhperic delays and multipath).  Receiver needs at least 4 satellites to yield a 3-D position (lat, long, and alt) and time.  GPS constellation of 24 satellites set up so 5 are observable anywhere on the Earth.  GPS receiver verifies integrity of signals from satellites through receiver autonomous integrity monitoring (RAIM).  RAIM needs at least 5 satellites in view to detect an anomaly, or 4 satellites and a barometric altimeter.  RAIM needs at least 6 satellites in view (or 5 with a baro-aid) to isolate corrupt satellite signal and remove it from nav solution.  Don’t rely on GPS altitude, vertical error can be very large.  To ensure baro-aid available, current altimeter setting must be set into receiver.  Without RAIM, no assurance of accuracy of GPS position.

GPS APPROACH PROCEDURES: (FTI 4-59)

*** Note: T-44 self-taught GPS training program is located in TWG-4 building in course rules computer room (1st floor on right side [if entering from rear]).  Course takes several hours to complete.

Study sources: [1]FMS/GPS computer software (in TWG-4 bldg.) [2]IEC Model 9002M FMS Operator’s Manual (where can I find this pub?) [3]NATOPS [4]AIM [5]AFMAN 11-217 V2.

Navigation Database: All named waypoints identified with 5-letter alpha character name.  Unnamed waypoints, such as a DME fix, have a 5 letter alphanumeric coded name in the database (exp. D234T represents a point on the 234 radial from XYZ vortac at 20DME [the letter T is the 20th letter of the alphabet and indicates 20nm]).  The T-44 GPS uses the Jeppesen NavData Database.

Approaches: 2 types: [1] Stand-Alone: constructed for GPS, absence of other navaids in approach title.  Straight-in minimums correspond to LNAV Minima on RNAV charts [2] Overlay: permit GPS under IFR to fly existing instrument approaches.  2 types: [A] GPS Not in the Title [B] GPS in the Title

Course Sensativity: CDI sensitivity varies with mode of operation.  GPS normally linear (straight line distance from point to point) regardless of distance from waypoint.  [1] Enroute Mode: prior to approach, full scale deflection = 5nm from centerline [2] Terminal Mode: within 30nm of destination airport on charted SID/DP/STAR or between last enroute waypoint and initial waypoint (IAF).  CDI full scale deflection = 1nm from centerline. [3] Approach Mode: sensitivity transitions from 5nm full deflection to 1nm full deflection within 30nm of destination airport.  Must be active to proceed past FAF on non-precision approach.  [4] Final Approach Mode: 2nm to the FAF waypoint, sensitivity transitions to CDI full scale deflection of 0.3nm either side centerline.  [5] Missed Approach Mode: nav to missed approach holding point, sensitivity 1nm of centerline.

RAIM: When GPS integrity doesn’t meet RAIM standards, integrity warning, signals GPS nav info may no longer be reliable (equivalent to ‘off flag’ on HSI.

Approach Restrictions: Instrument Approaches must be: [1]accomplished in accordance with FMS database using current update cycle [2]conducted in FMS Approach Mode.  RAIM must be available at FAF (INTG annunciator extinguished).  Aircraft must have other approved nav equipment.  GPS approaches can be flown without tuning, identifying, or monitoring other navaids (highly discouraged).

Preflight Actions: [1] Review GPS NOTAMS for destination and any alternates.  GPS satellite outages issued under pseudo random noise (PRN), number or satellite vehicle number (SVN).  Accessed using “KGPS” airport identifier.  Important to deselect any affected satellites using SV CONTROL function (on RAIM 2-2 page) of IEC 9002 FMS. [2] NavData NOTAMS: issued every 2 months.  Found at Jeppesen website (http://www.jeppesen.com/onlinepubs/navdatanot.phtml).  Check before flight. [3] If using GPS, aircraft is considered RNAV for flight planning purposes.  Use BE-9L/G in aircraft identification block.  [4] Check GPS ground equipment by start-up and self-test procedures as outlined in NATOPS.  Check currency of database, and predicted RAIM available status for approach plan to fly at ETA.

CRM Procedures: On deck, right-seat pilot enters flight plan into FMS.  In flight, pilot not flying (PNF) changes flight plan data.  PNF verbalizes changes.  Pilot flying (PF) verifies modification.  Both pilots review modified or previously loaded flight plan prior to execution.  Either pilot verbalize need to enter a discontinuity.  PNF will enter discontinuity and adivise PF when entry complete.  Either pilot verbalize need to close up discontinuity.  PNF receive concurrence from PF prior to closing it up.  Either pilot verbalize need to enter or edit a waypoint.  PNF enters the waypoint and executes it after receiving concurrence from PF.  PNF monitors flight progress on LEGS page.  Either pilot verbalizes if annunciator illums.  PNF looks up amplifying information.  In response to CDU APP annunciatior (APPR annunciator on instrument panel), either pilot verbalize approach mode still active, and other pilot acknowledge the call.  After missed approach, PF requests approach reselection.  PNF will acknowledge and comply, then inform PF when approach reselection is complete.  Prior to manually changing CDI sensitivity, PNF obtains concurrence from PF.  PF will verbalize disengaging FMS.  PNF will acknowledge.

Terminal Area Ops – Departure: Load SID/DP (from nav database).  If can’t be retreived, may not use RNAV to fly prior to SID/DP termination point.

Enroute Operations: Use of Predictive Integrity.  Check RAIM availability for planned approach.  If check inicates integrity may not be available, develop alternate plane for landing or proceed to alternate.

Prior to Descent: Compare approach retrieved from GPS nav database to GPS published approach on IAP.  If differences arise, the published approach chart, supplemented by NOTAMs, takes precedence.  Develop a back-up plan.  Consider the possibility of equipment failure past the FAF.  Load the STAR (if applicable).  If can’t be retreived, can’t use RNAV to fly procedure.

Terminal Area Ops – Arrival: Monitor all ground-based navaids since GPS approaches are flown point-to-point.  Bearing pointer on HSI and DME will be to next waypoint, not necessarily the field.  Be prepared to use traditional navaids.  Be wary of having head down in cockpit.

Approach Procedures: Reference NATOPS and IEC 9002M FMS Operator’s Manual.  Inbound Course: course displayed on FMS between FAF and MAP may be slightly different than printed on approach plate; should not affect approach performance.  GPS Integrity Warning Prior to FAF: don’t descent to MDA.  Instead, proceed to MAP via FAF, perform missed approach, and notify ATC.  Alternatively, may continue approach provided backup is available.  Stepdown Waypoints: nav database doesn’t provide stepdown waypoints after FAF.  In all cases, next waypoint is MAP.  Determine stepdown fixes and VDP by refering to GPS distance from MAP as depicted on approach plate.  GPS Integrity Warning After FAF: very serious, immediate action required.  Transition to back-up approach if available or proceed to MAP along FAC and execute missed approach.

Performing Missed Approach Procedures: MAP labeled with named waypoint.  Select Missed Approach Mode manually by using LEGS screen.  Clear MAP discontinuity by pressing Go Around Button on power lever within 2nm of FAF.  If Go Around Button used, EXEC key still needs to be pressed to execute missed approach.  Performing the missed approach: If initiated prior to MAP, proceed to MAP along FAC.  If missed approach includes a turn, don’t begin turn prior to MAP.  Fully review procedure before flying it!  Climb gradient: min of 200 ft/min unless higher published.

FUEL SYSTEM: (NATOPS 2-11 to 2-17)

1. Total fuel system capacity 387.6 G

2. Total usable fuel 384 G

3. Crossfeed allows use of all tanks to either engine.

4. Fuel is transferred from the wing tanks to the center section tank by gravity.

5. Fuel is transferred to the nacelle tank by the transfer pump.

6. Nacelle float switches: 	High level (59G)– turns transfer pump off if 51 Gal switch fails.

Mid level  (51G)—turns transfer pump off

Low level (42G)—turns transfer pump on

7. In override, the transfer pump runs continuously, the float switches are bypassed, & the 3psi press switch reset

8. With a failed transfer pump, the fuel gets to the nacelle tank by gravity feed.

9. There are 10 fuel drains on the aircraft.

10. A fuel capacitance gauging system is used in the T-44.

11. The only diff between the left and right fuel system is the left has crossfeed.

12. A minimum of 42 gallons of fuel is req in the Nacelle tank to obtain a good test of the transfer pump.

13. The boost pump provides 30 psi of pressure to the engine driven fuel pump.

14. It takes 5psi of pressure to activate the FUEL PRESS light.

15. It takes a 30 second pressure delay to activate the NO FUEL TRANSFER light.

16. The engine driven fuel pump supplies 800 psi pressure to the engine.

17. The indications of a boost pump failure: with crossfeed in ‘auto’: [1] red ‘FUEL PRESS’ light illums and goes out very quickly [2] yellow ‘FUEL CROSSFEED’ light illums [3] master caution light flashes [4] gradual loss of power on affected engine beginning at 13,000’.

18. The TBO for an engine driven fuel pump operating on suction lift is 10 hours.

19. The TBO for an eng running on AVGAS is 150 hours.

20. The crossfeed is not left open with the boost pumps operating because fuel may be inadvertently crossfed from either fuel system to both engines because of variances in pump pressure

21. Since the crossfeed valve is electrically opened, if it losses total electric power (i/e battery drained and generators out), it will be closed.

22. The firewall valve is not used to secure an engine because severe damage could occur to the engine-driven high pressure fuel pump (while cavitating) because it obtains essential lubrication from fuel flow.

23. Total fuel quantity error in total can be as high as ______ % and in nacelle is ____ %

24. Fuel quantity error can in total can by as high as 100 lbs, while nacelle can only by 50 lbs.

25. The syphon break is located on the outer wing and opens to prevent negative pressure build up in the fuel system (normally caused by thermal expansion).

26. The purpose of the fuel vents are to provide ram air pressure to prevent tanks from collapsing as they lose fuel to the engine.

27. If either fuel bus fails, the boost pump will still work because it’s wired to the hot battery bus.

28. If the transfer pump CB popped, I would get the following indications: red ‘NO FUEL TRANSFER’ light WON’T ILLUM!

29. The min distance the fuel truck can come to the airplane while refueling it is 10 ft.  For the refueling point 20 ft.

30. Fuel flow from outboard tanks to the engine: gravity feeds to the center inboard tank, through transfer pump to the nacelle tank, through the boost pump, by the firewall shutoff valve, through the fuel filter, through the fuel flow transmitter, through the fuel heater, through the engine driven fuel pump, controlled by FCU, controlled by start control unit, to the engine fuel nozzles, into the combustion chamber.

TACAN APPROACH (WITH PROCEDURE TURN):  Procedure Turns (PT): 3 acceptable methods: [1] 45/180 Technique [2] Holding Technique [3] Teardrop Technique

[1] 45/180 Technique:  Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN).  Continue descent to FAF.  Configure and slow to 120K prior to the FAF (unless SSE).  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

[2] Holding Technique:  If heading within 90 degrees of outbound PT Course, use normal lead points (includes all of teardrop entry and most of parallel entry).  If use teardrop entry, course must be within 30 degrees of PT outbound course.  If intercept the PT outbound course, maintain course for remainder of outbound leg, then turn toward the maneuvering side to reverse course.

Timing: begin timing outbound abeam procedure turn fix.  If can’t determine abeam, start timing at completion of outbound turn.  Stay within the remain within distance.  At the completion of the outbound leg, turn to intercept the procedure turn course inbound.  A 1 ½ minute outbound leg normally is sufficient (2 minutes or more can be used in instances such as an approach with no FAF or an approach with a large descent on the inbound course).

Descent: Don’t descend form fix altitude until abeam PT fix outbound.  Don’t descend from completion turn altitude until established on inbound segment.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

RADIO INSTRUMENT MALFUNCTION: (FTI 4-49) CDI failure: Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.  DME failure:  Don’t shoot approach that requires DME.  Request a non-DME approach (PAR, ASR, ILS, LOC, etc...)  If inside FAF and lose DME (timing NOT available), execute a missed approach unless field is in sight.

TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.

SSE TACAN APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

# FUEL SYSTEM EMERGENCIES (NATOPS 15-4, 15-6 to 15-7)

1. NO FUEL TRANSFER LIGHT

[1] Check total and nacelle fuel quantity

If no fuel in wing tanks:

[2] Transfer pump – OFF

If fuel in wing tanks:

[3] Transfer pump – OVERRIDE

If light remains on:

[4] Transfer pump – OFF (lose 28 Gallons)

[5] Land as soon as practicable

2. CROSSFEED light illumination: illuminates when boost pump fails or engine fails.  If boost pump failure: [1] close crossfeed switch.  If boost pump failed: [1] Failed boost pump – OFF [2] crossfeed – OPEN [3] Land as soon as practicable.  If not a boost pump failure, reset the crossfeed valve to ‘AUTO.’  If engine failure, accomplish EMERGENCY ENGINE SHUTDOWN CHECKLIST.

3. Failure of the engine driven fuel pump – Engine WILL FLAMEOUT.  Accomplish EMERGENCY ENG SHUTDOWN CHECKLIST.

4.  If fuel leak – consider securing electric systems which may contribute to wing fire.  Secure nav lights, strobe lights, fuel vent heaters, and AOA.  Secure inboard wing items with gang bar.  If destination can be reached without dual engines, consider shutting down engine on side of fuel leak.  Secure engine as follows:

[1] Condition Lever – FUEL CUTOFF
[2] Emergency Shutdown Checklist – EXECUTE

5. If fuel syphoning – [1] Airspeed – 140K [2] Land as soon as practicable.  Avoid extreme nose low attitudes.

6. Single engine crossfeed:

[1] Boost pumps – ON

[2] Transfer pumps – AUTO

[3] Crossfeed – OPEN (check crossfeed light on)

[4] Boost pump (nonfeeding tank, tank w/o FUEL PRESS light ON) – OFF

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.