RI-19S Study Guide (03 Jul 02)

ILS/LOC CHARACTERISTICS: (FTI 4-42, AIGT 2-2, 2-17, 8-18, AIM 1-1-6)

ILS: In the U.S., the required components for an ILS approach: [1] glide slope [2] localizer [3] outer marker.  If no outer marker, it can be replaced by DME, radar, another navaid, or a crossing radial.  If VOR 2 is used to identify intermediate fixes and/or the FAF, tune it to the localizer freq no later than the FAF, unless used to define subsequent stepdown fixes or the MAP.  If the glide slope fails, transition to a non-precision localizer approach (SSE, raise the flaps and gear and treat like a non-precision SSE approach (not lowering the flaps or gear until landing assured)).  If the OM (or one of its substitutes) isn’t available, can’t fly the approach.  Intercepting an ILS/LOC approach is either by radar vectors on the published approach procedure.  While being radar vectored, use any available navaid (exp. TACAN, VOR/DME) to aid in position orientation.  The TACAN select switch must be set in “OPER” to receive DME!  The glide slope’s usable range is 10 miles (however, it can be picked up much further out).

Localizer: The localizer’s usable range is at least 18 miles within 10 degrees of centerline (35 degrees from centerline out to 10 miles).  The localizer may be used beyond 18 miles, but only if under ATC radar guidance. If the localizer fails, can’t fly the approach. On a normal front course localizer, the localizer antenna is usually located beyond the departure end of the runway.

Localizer Back Course (LOC BC): On a LOC BC approach, the localizer antenna is located prior to the approach end of the runway, making a LOC BC approach much more sensitive than a normal LOC approach.  False glide slope signals may exist in the area of a LOC BC causing the G/S warning flag to disappear.  Disregard all G/S indications on a LOC BC approach unless a G/S is specified on the IAP.  At FAF, always hack the clock – timing is always a good back up!   Localizer Backcourse – ALWAYS set in the FRONT COURSE on the CDI  (the ‘front course’ refers to the inbound course on the ILS/localizer approach for the OPPOSITE runway.  On the back course approach plate, the published front course is depicted in the feather as an outbound localizer course).  Avoid reverse sensing on a LOC BC by twisting in the front course.  Intercepting the localizer – keep heading changes to 5 degrees or less.  Discontinue the approach when localizer course is unreliable or full scale deflection on CDI on final.

ILS/LOC approach: First portion of ILS approach generally has a large turn (procedure turn or teardrop approach, off a compass locator, or radar vectors to final).  The actual ILS portion begins when joining the final approach fix inbound.  Must be configured prior to reaching glide slope intercept point (or FAF).  At DH, must either go missed approach or continue landing.  DH based on altitude, not position.  Timing is not used on an ILS approach.  However, if glideslope fails, must revert to LOC appraoch (which may use timing).  So, always back up approach by starting clock at glideslope intercept!  If “COURSE” Warning flag is displayed on final approach, initiate a missed approach!  If “GLIDE SLOPE” Warning Flag is displayed on final approach, don’t fly lower than localizer minimums, or (if no localizer mins published) no lower than circling mins.  SSE precision approach (ILS), Flaps – FULL and Gear Down Landing Checklist 1.5 dots below glidepath intercept.  SSE non-precision approach (LOC), Flaps – FULL and Gear Down Landing Checklist when landing assured and in a safe position to land.  When ceiling < 800’ and/or visibility < 2 miles (and landing aircraft is between ILS FAF and airfield) departing aircraft must hold short of the ILS CRITICAL AREA!  When ceiling < 800’ and/or visibility < 2 miles, aircraft can’t hold below 5,000’ between the outer marker and the airfield.  Approaching the final course (on base or arc) perform the ‘LIDS’ check – [1] Localizer freq set in (VOR/ILS selected on FD SEL switch, monitor the ident) [2] Inbound front course set in the CDI [3] DME (tune the TACAN to the localizer associated with the DME freq) [4] Speed and Configuration (slow to 120K and configure when: [1] 1.5 dots of intercepting glide slope on ILS [2] 3 nm prior to FAF on localizer).

Descent – Maintain glide slope intercept altitude (published or assigned) until intercepting the glide slope (identified by a lighting bolt).  On glide slope, crosscheck aircraft altitude with published “Glide slope Altitude at OM/FAF” to ensure established on correct glide slope.  Don’t descend below descent restrictive altitude if CDI indicates full scale!  The glide slope isn’t the primary aid for vertical flight path until the lightening bolt on the IAP.  However, if established on localizer course and within usable range (10 miles) of glide slope, it can be used in descent to glide slope descent altitude.

Glide slope Indicator (GSI): intercept the glide slope as the GSI moves downward from its upper limits.  Slightly before the GSI reaches the center position, coordinate pitch and power to maintain proper rate of descent (normally 600 feet on a 3 degree glide slope, set power at 350ft-lbs at 120K).  On the indicator, the glide slope is the green pointer & the center line is the aircraft).  Make pitch adjustments in 2 degree or less increments.  On the final portion of the approach, make 1 degree pitch corrections and heading corrections of 5 degrees or less.  Decision Height (DH) is determined by the barometric altimeter.  Don’t descend below localizer minimums if the aircraft is more than one dot (half scale) below or two dots (full scale) above the glide slope.  If glideslope is corrected back to within tolerances, descent can be continued to DH.  At decision height, must immediately decide whether to continue approach or go missed approach.

Flight Director: The Pilot’s CDI can show either VOR 1 (including LOC) or TACAN.  The copilot’s CDI shows only VOR 2 (including LOC).  When tuned to localizer, the TO/FROM flag doesn’t operate & the course arrow setting doesn’t affect the deviation bar.  Marker Beacon comes on when the VOR is turned ON (put P to sensitivity to ‘LOW’ and CP to ‘HIGH’ and turn MB audio switches ON).  LOC/ILS sensitivity on CDI = 2.5 degrees for full scale deflection (5 degrees full deflection for VOR & TACAN).  “HEADING” Warning Flag indicates failure of compass system.  “NAV” Warning Flag indicates NAVAID signal is unreliable.  The center of the glideslope scale (the line) represents the aircraft and the green pointer represents the glide slope.  (For exp., if the pointer is above the center line, the aircraft is below the glide slope, and vice-versa).

NCS-31:  ***If need DME from a NAVAID other than the LOC on an ILS/LOC approach: [1st] Enter the DME freq and D-U-D [2nd] Enter the LOC freq and press the USE button twice (USE, USE)!

Marker Beacons: Outer, Middle, and Inner.  MB lights begin to flash when in almost directly over a MB.  The lights are accompanied by a series of aural tones (- - - for Outer [Blue]/ - . - . - for Middle[Amber]/ . . . for Inner[White]).  Proper configuration for MB is P turn on audio for MB 1 and set sensitivity to LOW, CP turn on audio for MB 2 and set sensitivity to HIGH.  MB are not tuned (always transmit on 75 MHz).

Compass Locators: (CLs)  Often located at OM and MM.  Range of at least 15 miles.  At high powered locations, CLs also have TWEB info.  OM CLs transmit first 2 letters of localizer ident.  MM CLs transmit last 2 letters of localizer ident.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

VOR & TACAN TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.  VOR Station passage: TO/FROM flag changes to FROM.  TACAN Station Passage: minimum DME.

SSE LOC NON-PRECISION APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

ILS/LOC NO HEADING APPROACH: (FTI 4-49)  Troubleshoot heading system, check all circuit breakers and God Box, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.  Request radar vectors to final.  If can’t get radar vectors, use a procedure turn (to do a PT, must have a Localizer Outer Marker (LOM) – an NDB aligned to course centerline).  If no radar vectors available: proceed direct to NDB using no heading techniques.  At station passage, time turn to intercept localizer course outbound, rotate course to Final Approach Course and track localizer course outbound.  NDB no heading information used as back up.  When localizer ‘comes alive’ and comes off full scale deflection, time turn to final.  Once established on localizer and tracking inbound, treat as a normal approach.

ADF NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, check all circuit breakers and God Box, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

The ADF needle shows relative bearing only.  Mentally place the current heading (given by the CP from the mag compass) under the upper lubber line.  Count the number of degrees the head of the single needle deflected from the lubber line to find the relative bearing to the station.  Add (+ to the right) or subtract (- to the left) from the current mag heading to get the magnetic bearing to the station.  Mag bearing from the station found by using the tail of the needle when heading away from the station.  (Exp. If the current heading is 135, and the needle points 45 degrees to the left of the lubber line, subtract (because the needle is left) 45 from 135 to get a magnetic bearing of 090 to the station.  Unless radar vectors are given, request the 45/180 course reversal.  Turn to place the head of the needle on the nose and fly direct to the station.  Utilized known winds to make a wind-corrected heading to the station.  Continue to ask the CP for heading calls and set the heading bug on the corresponding heading to proceed direct.  Compute a heading to intercept the outbound course (the outbound course + 30 degrees [extra 10 seconds]) and determine the seconds required for a timed turn.  Immediately after station passage, commence a timed turn to intercept the outbound course.  Track outbound 1 minute using heading calls from the CP, then start a 45 degree [15 second] turn to the outbound PT heading.  After 1 minute, commence a 180 degree [1 minute] course reversal.  Inbound on the procedure turn, determine my position for situational awareness.  When the needle reaches the 45 degree benchmark, turn inbound so the head of the needle is at the top of the RMI.  Continue to ask the CP for headings.  Configure at the normal point for a non-precision approach.  Passing over the IAF and the FAF, execute the 6Ts (time, turn, time, transition, twist, talk).  If missed approach, generally requires turning to the head of the needle and flying direct back to the station.

RADIO INSTRUMENT MALFUNCTION (VOR, TACAN, NDB): (FTI 4-49) (CDI failure for VOR/TACAN): Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.

# PROPELLER MALFUNCTIONS (NATOPS 15-10)

Primary Governor Failure (memory item): feather or overspeed.  If landing with overspeeding prop carry extra airspeed.

If prop rpm out of normal governor range (1800 – 2200RPM):

[1] Propeller RPM  -- Adjust to Normal RPM.

By prop lever. If norm restored, continue ops.  If norm can’t be obtained:

[2] Power Lever – IDLE (concur)

[3] Propeller Lever – FEATHER (concur)

Notes: Prop RPM > 2420 may result in RGB failure and/or N2 turbine damage.  Engine with disabled prop can provide electric power.  Right prop may not full feather with prop sync ON.

[4] Emergency Engine Shutdown Checklist – AS REQ

Prop Linkage Failure: prop gov ctl linkage fails, prop goes to 2200 or maintains last rpm setting.

[1] Manipulate prop lever to determine if cockpit control lost

[2] If cockpit control lost and RPM in safe limits, match opposite prop speed with uncontrollable prop and land as soon as practicable.

Alternate Prop Feathering: Linkage or gov fails and prop doesn’t feather by itself or normal feathering procedures don’t work, use autofeather.

[1] Power Lever – IDLE (failed propeller) (concur)

[2] Condition Lever – FUEL CUTOFF (concur)

[3] Autofeather – ARM

[4] Power Levers -- > 90% N1 *Don’t pull power lever to IDLE during autofeather, may disarm system.

[5] Emergency Shutdown Checklist – EXECUTE

Simulated Dual Engine Failure: initiated above 91K by IP reducing bother power levers to idle.  Sea Level is bottom of deck.  Select appropriate ditch heading.  Simulated restart on both engines simultaneously.

[1] Clean up (Gear – UP, Flaps – UP)

[2] Commence turn towards coastline, desired hdg, IP assigned hdg while transitioning to 130K (max range) or 102 (max endurance) if restart altitude minimal.

[3] Simultaneously commence Windmilling Airstart Checklist.  Simulate both condition levers fuel cutoff by pointing  at both levers.  Autoignition may be ARMED or starters simulated ON at student’s discretion.

[4] IP will state what engines lightoff.  If restart successful, add power and complete the checklist.  If restart fails, complete the Emergency Engine Shutdown checklist and follow ditching procedures.  IP will reduce props to 1900RPM when simulated feather and may add power to 100ft-lbs.  Stop engine restart attempts at some time during the power off ditch.  Secure the engines by doing at least first 3 items of Emergency Engine Shutdown.  Emphasize proper ditch.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, unable to make safe landing, directed to do so by controlling agency, or instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.