RI-15 Study Guide (08 Jul 02)

__ IFR BRIEF (BE HRC)

__ RUNWAY LENGTH REQUIREMENTS

__ LOST COMM (LOCAL & FIH: IFR & VFR)

__ CHECKLIST PROCEDURES (PARTICULARLY RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (NATOPS (C.4),T-44 IMPORTANT NUMBERS, & CONDENSED NATOPS)

__ CODED DEPARTURES

__ SYSTEM OF THE DAY (PLUS AN ADDITIONAL SYSTEM IF TIME PERMITS)

__ EMERGENCY PROCEDURES (NATOPS: GROUND, TAKEOFF, IN-FLIGHT, LANDING)

__ AIGT (HOLDING, TERMINAL PROCEDURES, IAPs)

__ FLIGHT SIMULATOR 2002 (LOCAL APPROACHES (NORMAL & NO HEADING))

# ELECTRIC SYSTEM

1. Sources of DC power: 1 24V/42A battery, 2 28V/250A starter-generators

2. Sources of AC power: DC power routed through 2 750V-A single-phase inverters.

3. The generator control box: overvoltage protection (31V), undervoltage protection (18V), reverse current protection, & automatic paralleling.

4. The generator control boxes are under the cabin center aisle aft of the main spar.

5. The inverters are located ____________________________________________.

6. The current limiters: tie the main buses & provide fault protection.

7. To get around a faulty master avionics switch, pull the master power CB.

8. Limitations of the generators: 28.25 (+/- 0.8)V.

9. Limitations of the inverters: 114 (+/-7)V, 400 (+/-6)Hz.

10. For APU start, the unit must be able to provide 1000 Amps for 0.1sec and 300Amp continuous load thereafter.

11. A failed Inverter Select Relay Fuse will cause failure of respective AC bus without illum of inverter light (switch over impossible).

12. Illum of INST INV OUT light indicates loss of 26VAC inverter select relay & torquemeters.

13. Procedures for INVERTER failure: [1] Fail Inverter – OFF [2] AC Bus switchover [3] Land ASA practicable

Total ac failure, still have: fuel flow, oil press, oil temp, turn & bank, pitot static, N1, ITT, prop RPM, clocks, NCS-31A, AOA indexers, RADALT, wet compass14. Procedures for GENERATOR failure: OFF, reset momentarily, ON.

15. If generator fails to reset, yet registers normal volts in reset position, indicates failure of generator control box.

16. Total loss of power, still have: copilot turn and slip, VSI, altimeters, airspeed indicators, wet compass, N1 and N2, ITT, & clocks.

17.  Dual inverter failure, still have DME, radar, VOR (audio), ADF-LF (audio), xponder, MKR BCNs.  For comm still have, UHF & VHF.

18. Excessive load indications (>1.0): [1] Battery/ammeter – Note.  If charge rate in excess of 30A: [2] Battery – OFF. If loadmeter still over 1.0: [3] Non-essential loads – OFF.  Splits of >0.1 indicate abnormal generator paralleling.

19. If get a GEN OUT light, check Current Limiters: [1] Battery- no discharge, no other equip failures = CL intact. [2] Battery – discharge, no other equip failures = opposite CL failed. [3] Battery – no discharge, other equip failures = same side CL failed.

ELECTRICAL SYSTEM MALFUNCTIONS: (NATOPS 15-7 to 15-10)

# GENERATOR FAILURE

1. Generator – OFF, RESET MOMENTARILY, THEN ON (Memory Item)

Note: Normal generator voltage in reset position indicates failure of generator control box, not the generator.

If the generator will NOT RESET:

2. Generator – OFF

3. Operating Generator – Load < 1.0

4. Land as soon as practicable

# DUAL GENERATOR FAILURE

1. Cabin temp mode, electric heater, anti-ice/deice, lights –

OFF

2. Emergency call

3. Gang bar – OFF (unless in IMC)

Note: Loss of electric power will depressurize the cabin.  If cabin altitude exceeds 10,000 ft, supplemental O2 for all occupants should be considered

4. Boost pumps – OFF

5. Both invertors – OFF (unless IMC)

6. Pull the following circuit breakers:

a. LT & RT feeder bus circuit breaker for NO. 2

subpanel bus

b. LT & RT fuel panel bus

c. No.1 avionics (unless IMC)

d. No.1 RMI, radar, CP FDI, No. 2 FD

e. LH fuel flow, LH oil temp

f. PR bleed air control, prop sync, annunc power,

flap motor, & flap indicator

7. Switch CP speaker/phone switch to speaker

8. Battery – ON

Note: (1) pilot turn and slip, UHF, landing gear annunc lights still powered (2) With total loss of power, following items work: [1] CP turn and slip [2] P & CP VSI [3] altimeters [4] airspeed [5] wet compass [6] N1 & N2 tachs [7] ITT [8] clocks (3) If attitude gyro & NAVAIDS required for flight, pull all C.B.s stated above except No. 1 avionic CB and leave No. 1 inverter and battery switches ON. (4) Anticipate a no-flap landing and manual gear extension.

EXCESSIVE LOADMETER INDICATIONS (> 1.0)

1. Battery/ammeter – NOTE

If charge rate > 30 amps:

2. Battery – OFF

If loadmeter still > 1.0:

3. Nonessential loads – OFF

Note: Loadmeter splits > 0.1 indicate abnormal generator paralleling.  With air conditioner or electric heater activated, an excessive loadmeter indication for LEFT generator may indicate a current limiter failure.

# INVERTER FAILURE

Indicated by illum of failed INVERTER OUT annunc light.

1. Failed inverter – OFF

2. Check AC bus switchover

3. Land as soon as practicable

Note: After total AC power failure, only following work: [1] fuel flow [2] oil press [3] oil temp [4] P & CP turn & bank [5] VSI [6] airspeed [7] altimeters [8] N1 [9] ITT [10] prop RPM [11] clocks [12] NCS-31A [13] AOA indexers [14] RADALT [15] wet compass

Note: After dual-inverter failure, only following work: [1] DME [2] radar [3] VOR (audio only) [4] ADF-LF (audio only) [5] transponder (mode C) [6] marker beacons [7] VHF & UHF available

# INSTRUMENT INVERTER FAILURE

Illum of INST INV annunc light.  Indicates loss of both 26VAC stepdown transformers.  Torquemeter readings erroneous (AC lies)

# CIRCUIT BREAKER TRIPPED

1. Nonessential CBs – DON’T reset in flight

2. Essential CBs – only reset ONCE in flight

Note: Subpanel feeder CBs tripped indicates a short.  DON’T RESET!

# ENGINE SYSTEM MALFUNCTIONS

# [1] OIL SYSTEM FAILURE

Oil press < 85psi undesirable – tolerated only to complete flight. Monitor engine instruments and nacelles for secondary indications.  Consider shutting down engine and landing as soon as POSSIBLE; otherwise, reduce power on engine and landing as soon as PRACTICABLE.  Oil pressure < 40psi & oil temp >99C unsafe requiring engine shutdown or landing made as soon as possible using min power to sustain flight.  Note both discrepancies on maintenance forms.

# [2] CHIP DETECTOR LIGHT ILLUMINATION

Indicates metal particles in prop reduction gearbox.  Perform EMERGENCY SHUTDOWN CHECKLIST.

# [3] EMERGENCY ENGINE SHUTDOWN ON DECK (for FIRE, FIRE WARN LIGHT, CHIP LIGHT)

1. Condition Levers – FUEL CUTOFF

2. Firewall Valves – CLOSED

3. Boost Pumps – OFF

4. Fire Extinguisher – AS REQUIRED

5. Gang Bar – OFF

6. Evacuate aircraft

[4] ABNORMAL START (for HOT START & NO LIGHT OFF)

If ITT likely to exceed 925C or no rise in ITT within 10 sec after condition levers LOW IDLE:

1. Condition Lever – FUEL CUTOFF (ITT < 790C)

2. Starter – OFF

Caution: Starter time limited to 40 sec ON, 60 sec OFF, 40 sec ON, 60 sec OFF, 40 sec ON, 30 minutes OFF.

Note: If start attempt discontinued and another start attempted, allow 60 sec delay to drain fuel and cool starter, then motor up starter for 15 sec minimum.  Allow eng to completely stop before attempting another start.

# [5] ABORT & ENG FAILURE DURING TAKEOFF

1. Announce -- “ABORT” (Initiate < 91KIAS)

2. Power Levers – IDLE

3. Reverse – AS REQUIRED (then beta range)

4. Brakes – AS REQUIRED (smooth, even, don’t lock)

Prior to departing prepared surface:

(verbally recite remaining items)

5. Condition Levers – FUEL CUTOFF

As soon as practicable:

6. Firewall valves – CLOSED

7. Boost Pumps – OFF

8. Fire Extinguishers – AS REQUIRED

9. Gang bar – OFF

10. Evacuate aircraft

Warning: If direction control problems occur while reversing, advance both power levers to FLIGHT IDLE.  Maintain directional control with rudder, nosewheel steering, and by pumping brakes.

Warning: Accelerate-stop distances increased 900 ft with condition levers HIGH IDLE and no reverse utilized.

# [6] ENGINE FAILURE AFTER TAKEOFF

If sufficient runway to stop on ground, ABORT!

If insufficient runway to abort takeoff:

(POWER UP, RUDDER UP, CLEAN UP)

1. Power – AS REQUIRED

2. Gear – UP

3. Airspeed – (102 (obstacles) or 110)

4. EMERGENCY SHUTDOWN CHECKLIST

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

Note: address first 2 items of dead engine checklist, transfer comm to CP and have him declare an emergency.

Note: Dynamic Engine Cut complete when trimmed at 110KIAS (min 102KIAS), on takeoff heading, and Emergency Shutdown Checklist complete.

## [7] INFLIGHT EMERGENCIES

Note: 1. Rudder trim sufficient above 100KIAS.  Below 100KIAS, supplement full rudder trim with constant rudder pressure.

2. SSE waveoff with full flaps, result in loss of 200 feet before positive rate of climb established.

3. Impending engine failure or flameout preceded by:

[1] unstable engine operation

[2] fluctuating turbine RPM

[3] torque and ITT temp changes

[4] illumination of fuel warning lights

[5] drop in oil pressure

[6] loss of thrust

4. DON’T attempt an airstart if: [MOVEOFF]

Mechanical malfunction

Overheating

Vibration

Explosion

O N1

Fire

Fuel Fumes

NON-PRECISION (VOR, TACAN, NDB) HOLDING:  When ATC issues holding clearance, the direction they give points TO the holding fix (exp.  hold west of the NGP VORTAC means the arrow of the inbound course is FROM the west).  Always get an Expected Further Clearance Time (EFC), it determines course of action in a lost comm situation.  Standard holding means all turns to right.  Non-standard holding means all turns to left.  Make all turns either standard rate or max of 30 deg AOB.  If must tack 10 deg into wind on inbound leg, must tack 30 deg (3 times as large a correction) in opposite direction on outbound leg.  If extended holding, consult NATOPS for max endurance speed.  Request extended legs if delay exceeds 20 minutes.  ATC should issue holding instructions 5 minutes prior to reaching clearance limit fix.  Always request an EFC prior to accepting a holding clearance if one was not given.

Timing:  Begin outbound timing when abeam fix.  If abeam position can’t be determined, start timing when turn outbound is complete.  < 14,000’ MSL, time 1 minute. > 14,000’ MSL, time for 1 ½ minutes.  Commence inbound timing at wings level.  Station passage: [1] VOR = TO/FROM flag to FROM [2] TACAN = minimum DME. [3] NDB = needle passes wingtip position.

Speed: Recommended T-44 holding speed is 150KIAS.  Max holding speed is 175KIAS for turboprop aircraft.  Expect ATC holding clearance 5 minutes prior to arriving at holding fix.  Start a speed reduction to 150K (holding airspeed) 3 minutes or less from the holding fix.

Holding Entry:  Reaching the fix, execute the 6 T’s: Time, Turn, Time, Transition, Twist, Talk

Time: Note the time of station passage (NDB: needle passes 45 degree benchmark on RMI, cleared to turn)

Turn: Parallel, Teardrop, or Direct: determined using the outbound course on the RMI card.  Use pencil method.  If standard, put pencil on 90deg mark on RMI with tip pointed to right & turn 20 deg UP.  Relative position of 360 to 070 = Teardrop.  Relative position 360 to 250 = parallel.  All headings below the pencil are direct.  If nonstandard, put pencil on 90 deg mark on RMI with tip pointed to left & turn 20 deg UP.  Relative position 290 to 360 = teardrop.  Relative position from 360 to 110 = parallel.  All headings below the pencil are direct.

Parallel:  maneuver to outbound course (non-holding side) for one minute, then turn toward holding

pattern inbound course.

Teardrop: Subtract 30 deg from outbound course for standard & add 30 deg for nonstandard.

Direct: Turn in direction of outbound course.

Time: Time first leg outbound (1 min or 1 ½ min)

Transition: slow to 150KIAS

Twist: put in inbound holding course

Talk: Tell ATC established in holding

NON-PRECISION APPROACH (VOR, TACAN, NDB):  Procedure Turns (PT): Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN, +/- 5 bearings for NDB).  Continue descent to FAF.  Configure and slow to 120K 3 nm prior to the FAF (unless SSE).  Non-precision SSE approach, don’t configure until landing assured.  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

SSE NON-PRECISION APPROACH (VOR, TACAN, NDB): Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

NO HEADING APPROACH (VOR, TACAN): (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, if his have failed as well: secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat), ask him if he is familiar with how to work the mag compass, declare emergency, squawk 7700, have copilot call out cardinal headings.

Utilize either radar vectors to final or execute a procedure turn.  Compass card is frozen but radial/bearing info from needles is still correct.  1st, determine current radial (cross reference with DME).  2nd, take radial & DME and figure out current position on an IAP or chart.  If required, make a timed turn to the VOR inbound course and fly direct to the station (using the VOR needles & the CDI).  At station passage, make a timed turn to the outbound intercept heading, rotate the course select knob to the Final Approach Course & track outbound using heading info from the copilot.  Reverse course (45/180) using time turns: 15 sec for 45 degrees & 1 minute for 180 degrees.  When CDI starts to center, commence another 15 second turn to final.  To track inbound, either visualize with radials/DME or set CDI and heading bug.  Configure at normal point (3nm from FAF, unless SSE).

ADF NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

The ADF needle shows relative bearing only.  Mentally place the current heading (given by the CP from the mag compass) under the upper lubber line.  Count the number of degrees the head of the single needle deflected from the lubber line to find the relative bearing to the station.  Add (+ to the right) or subtract (- to the left) from the current mag heading to get the magnetic bearing to the station.  Mag bearing from the station found by using the tail of the needle when heading away from the station.  (Exp. If the current heading is 135, and the needle points 45 degrees to the left of the lubber line, subtract (because the needle is left) 45 from 135 to get a magnetic bearing of 090 to the station.  Unless radar vectors are given, request the 45/180 course reversal.  Turn to place the head of the needle on the nose and fly direct to the station.  Utilized known winds to make a wind-corrected heading to the station.  Continue to ask the CP for heading calls and set the heading bug on the corresponding heading to proceed direct.  Compute a heading to intercept the outbound course (the outbound course + 30 degrees [extra 10 seconds]) and determine the seconds required for a timed turn.  Immediately after station passage, commence a timed turn to intercept the outbound course.  Track outbound 1 minute using heading calls from the CP, then start a 45 degree [15 second] turn to the outbound PT heading.  After 1 minute, commence a 180 degree [1 minute] course reversal.  Inbound on the procedure turn, determine my position for situational awareness.  When the needle reaches the 45 degree benchmark, turn inbound so the head of the needle is at the top of the RMI.  Continue to ask the CP for headings.  Configure at the normal point for a non-precision approach.  Passing over the IAF and the FAF, execute the 6Ts (time, turn, time, transition, twist, talk).  If missed approach, generally requires turning to the head of the needle and flying direct back to the station.

RADIO INSTRUMENT MALFUNCTION (VOR, TACAN, NDB): (FTI 4-49) (CDI failure for VOR/TACAN): Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

VOR & TACAN TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.  VOR Station passage: TO/FROM flag changes to FROM.  TACAN Station Passage: minimum DME.

NDB COURSE INTERCEPTS/TRACKING: (RMI only): (FTI 4-19) Inbound: Course to Bearing + 30 = Intercept Bearing. Outbound: Tail to Course + 45 = Intercept Bearing. NDB Station Passage: bearing pointer passes through wingtip position.  In NDB holding, station passage is the first definite move by the bearing pointer through the 45 degree index on the RMI. Radial to Arc – lead by 0.8 nm for 90 degree turn at 150K. Arc to Radial – lead a 90 degree turn by 5 radials at 10 DME at 150K.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.