RI-21 Study Guide (11 Jul 02)

__ IFR BRIEF (BE HRC)

__ RUNWAY LENGTH REQUIREMENTS

__ LOST COMM (LOCAL & FIH: IFR & VFR)

__ CHECKLIST PROCEDURES (PARTICULARLY RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (NATOPS (C.4),T-44 IMPORTANT NUMBERS, & CONDENSED NATOPS)

__ CODED DEPARTURES

__ SYSTEM OF THE DAY (PLUS AN ADDITIONAL SYSTEM IF TIME PERMITS)

__ EMERGENCY PROCEDURES (NATOPS: GROUND, TAKEOFF, IN-FLIGHT, LANDING)

__ AIGT (HOLDING, TERMINAL PROCEDURES, IAPs)

__ FLIGHT SIMULATOR 2002 (LOCAL APPROACHES (NORMAL & NO HEADING))

NAVIGATION BAG: Refer to Cross-Country Checklist in Blue Brains (p. 71).

PASSENGER BRIEF: Is there standard gouge for pax brief???

FUEL PACKET/SERVICING:  Ensure that a fuel packet is checked out from maintenance control (must have credit cards).  Servicing: Ensure that the field we land at has the right type of fuel (Jet A, Jet A1, Jet B, JP-4, JP-5, PWA.522).  Land at a military airfield (with the correct JP fuel) or a civilian field with a government contract (contract list is in the Duty Office).  Ensure the lineman knows how to service a T-44 (nacelle tanks are filled before the wing tanks).

UNFAMILIAR FIELD OPERATIONS: Upon landing, flip IAP to airport diagram.  If not familiar with taxi instructions, request “progressive taxi.”  Ensure that the lineman are familiar with T-44 servicing procedures (fill the nacelle tanks before the wing tanks!).  Ensure rudder lock is removed if there is any chance the aircraft will be towed.  If no lineman is present, call “clear right/left” loudly out the window prior to starting each engine.  At civilian fields, preferable to use VHF.  At military fields, preferable to use UHF.

ARRIVAL TRANSITION: Prior to arriving in terminal area, do the ABC check.  [A] ATIS and Wx [B] Brief Approach [C] Checklists/Charts/Clearance for approach.  Checking in with approach in terminal area, inform them of the current ATIS letter, destination, desired approach, how it will terminate (full stop, touch and go, option, low approach, etc...). and/or the next approach.

OPARS: Military computer generated flight plan that takes into account winds, weather, fuel, and weight to generate the best possible route to reach a destination.  Gives headings, altitudes, fuel requirements, and times.  OPARS are requested at weather (961-2124).

ENROUTE WEATHER CHECK: (AIM 7-1-5 to 7-1-12/ FIH Section C)

[1] En Route Flight Advisory Service [EFAS] “Flight Watch”: live, use common freq 122.0 from 5,000 AGL – 17,500 MSL.  Discrete freq for 18,000 – 45,000 MSL.  Contact ‘flight watch’ by using name of local ARTCC, aircraft identification, and nearest VOR.  List of flight watches on enroute charts and in Enroute Supp.

[2] Transcribed Weather Broadcast (TWEB): taped, broadcast continuously over navaids and/or VORs.  Contains forecasts, Inflight Advisories, winds aloft, NOTAMS, METAR/TAFs.

[3] Hazardous InFlight Weather Advisory Service (HIWAS): taped, continuous broadcast of inflight hazardous weather.  Listed on Enroute Charts with an “H” in navaid boxes, VFR sectionals, and in Enroute Supp.

[4] Automated Weather Observing System (AWOS): taped, automated.  Visibility averaged for last 10 minutes.  Ceilings averaged from last 30 minutes.  Transmitted over discrete VHF or local navaid.  Receivable max 25nm from site and max altitude of 10,000’ AGL.  Updated each minute.  No two way comm..  Most have telephone access.  Freqs published in Enroute Supp and on IAPs.

[5] Automated Surface Observing System (ASOS): taped, automated.  Primary surface observation system in U.S.  Continuous minute-by-minute observations.  Generate METARs.  Transmitted over discrete VHF freq or at local navaid.  Receivable max 25nm from site and max altitude of 10,000’ AGL.

[6] Flight Service Station (FSS): live, freq listed on chart or in Enroute Supp.  Give METAR/TAFs for requested airport.

[7] Pilot-to-Metro Service (PMSV) “Metro”: Live, direct contact with military observers or forecasters.  Observers only give current wx info.  Forecasters can give wx, update DD175-1, and receive PIREPS.  Freqs listed in Section C of FIH.

[8] Automated Terminal Information System (ATIS): taped, broadcast over navaids or ATIS freq.  Give pertinent weather, airfield, and runway information for aerodrome ATIS services.  Freqs listed on charts, in enroute supp, and on IAPs.

[9] VOLMET Voice Weather Broadcasts: taped, continuous weather information for flights overseas.  Given over VHF and HF freqs which are listed in Section C of the FIH.

[10] Air Route Traffic Coordination Center (ARTCC): live by air traffic controllers.  Given on work load basis.  Normally only for hazardous weather.  Also give TAFs/METARS.

# LOST COMM ENROUTE: (FIH A-6 to A-7)

# In lost comm situation, can shoot any approach.

# VFR:  [1] Troubleshoot radios, headsets, and mike switches (check circuit breakers, transmit on UHF or VHF, change frequencies, transmit on Guard) [2] Squawk 7600 [3] Make radio calls in the blind [4] Tune up Guard and nearest VOR with voice capabilities (if applicable) [5] If able to remain VMC, continue flight under VFR, land as soon as PRACTICABLE (not possible), and notify ATC.

# IFR: [1] Troubleshoot radios, headsets, and mike switches (check circuit breakers, transmit on UHF or VHF, change frequencies, transmit on Guard) [2] Squawk 7600 [3] Make radio calls in the blind [4] Tune up Guard and nearest VOR with voice capabilities (if applicable) [5] Route: [AVE-F] In priority order: Assigned, Vectored, Expected Further Clearance, Flight Planned. [6] Altitude: The highest of the following for the route segment flown: Assigned, MEA, Expected Further Clearance.  If an EFC is given at a point where an approach begins and then comm is lost, start the approach at EFC.  If comm is lost at a point where an approach is not begun, leave the point at the EFC (if given).  If an EFC is not given, continue to approach point and start approach as close as possible to the time filed or amended with ATC enroute.

# Radar Approaches: If comm lost for [A] 1 minute on vectors to final [B] 15 seconds on ASR [C] 5 seconds on PAR, then attempt contact on secondary freq and, if still no contact, fly a non-radar approach.  Squawk 7600.  Maintain last assigned altitude, min safe altitude, or emergency safe altitude (if more than 25nm from field), whichever is higher.

# SEVERE WEATHER AVOIDANCE:  Utilize weather radar (read NATOPS 20-61 to 20-73).  ICING: During icing flight, min speed 140K to minimize accumulation.  Continuous flight in severe icing should be avoided!  Extend ice vanes when ambient temp is 5 C (41 F) or below in visible moisture (torque will drop 40 to 60 ft-lbs and range will decrease 10-12%).  Arm autoignition when when ambient temp is 5 C (41 F) or below in visible moisture.  BEFORE flying into icing, turn ON: [1] Prop Deice [2] Pitot Heat [3] Engine Lip Boot Heat [4] Fuel Vent Heat.  In icing, turn windshield anti-ice to BOTH.  When ice is ½ to 1 inch thick, activate single surface deice cycle.  If a thunderstorm or severe turbulence is encountered, fly at 153K (Why?  The aircraft will stall before its over-Ged).

COPILOT RESPONSIBILITIES: (NATOPS 22-1 to 22-5)  Refer to NATOPS highlighted sections in Chap. 22.

ANTI-ICE/DEICE SYSTEM & EMERGENCIES (NATOPS 2-50 to 2-53, 15-11)

Windshield Wipers: Don’t operate on dry glass.

Windshield anti-ice: Temp sensing unit maintains 95F at windshield surface.  Electric heater lockout.  Mag compass will be unreliable when windshield anti-ice in use.  May blur objects.

Powerplant Ice Protection: Mechanically operated ice vanes or electrical lip boot heat.  With ice vanes extended slight decr in engine torque.

Fuel System Anti-icing: Oil-to-fuel heat exchanger; operates continuously; no controls; one external fuel vent in each wing for both nacelle and wing tanks (protected from icing by electric heat elements).

Fuel Control Heat: Each FCU pneumatic line protected from ice by electrically heated jackets.  To prevent overheat, FUEL CONTROL HEAT should not be turned on unless cooling air will soon pass over the jackets.

Propeller Electrothermal Deicer System: Deicing boots on each prop blade; outboard and inboard element that receive electrical power through the deicer timer, slip ring, and brush block.; current measured by the prop ammeter.  Sequence: RH OUT, RH IN, LH OUT, LH IN (RORI LOLI); each segment heats for 34+/-6 sec; req 2 minutes for entire cycle; old mechanical timers sequence to next segment (show amperage fluctuation at timer switches to next segment); electrical timers always reset to RH OUT when power applied (no flux in amperage); each blade draws 5 to 6 amps (3 blades working correctly read between 14 to 18 amps).

Surface Deicer System: Ice removed from outboard wing leading edge, horizontal stab, and vertical stab by flexing pneumatic deicer boots.  Engine bleed air from the engine compressor supplies press to inflate boots and vacuum to hold down boots; press regulator protects system from overinflation;  when not operating, a distributor valve applies vacuum to hold boots down; when the DEICE CYCLE switch is toggled to SINGLE or MANUAL, a servo valve changes the inlet boots from vacuum to pressure that allows the boots to inflate; when the solenoid is deenergized, airflow through the valve is cutoff and it discharges out the boots through the intregal check valve until press reaches approx 1-inch Hg, at which time the boots port to vacuum and the remaining air is evacuated; the boots are again held down by vacuum; either engine can provide the bleed air for the system; bleed air passes through an 18-psi regulator and then enters the deicer and vacuum systems; vacuum press is created by the distribution valve and is proportional to pneumatic pressure by the deicer press regulator valve.  Stall speeds significantly increase with the wing deice boots inflated; never cycle the system rapidly, may cause ice to accumulate outside the contour of the inflated boots and prevent ice removal.  In SINGLE, boot inflate for 7 to 8 seconds, then deflate.  In MANUAL, all boots inflate and stay inflated until switch is released.  The boots can’t be overinflated.

Pitot & AOA Heat Systems: On the ground, AOA heating elements pass through a current limiter lowering voltage to 14 volts DC (low heat).  At lift off , a squat switch on the left main gear causes the current limiter to be bypassed and heating elements receive 28 Volts DC (high heat); except for takeoff roll, PITOT heat should not be used on the ground (might overheat because of lack of cooling airflow over heating elements).

Surface Deice Emergency Procedure: If boots fail to inflate, operate in MANUAL.  If boots fail to deflate, pull the SUR DE-ICE CB and reset as necessary to operate boots.

WEIGHT AND BALANCE: Refer to NATOPS (C. 21) and “The Definitive Weight & Balance Gouge” (from the Duty Office) 08 Jul 02.

DD-175/DD-175-1 (DASH ONE):  A DD-175 flight plan should be filled out anytime leaving the local area.  Refer to GP Chapter 4 for the correct procedures to fill it out.  A Dash One should be requested (min of 2 hours prior to the flight) anytime filing IFR.

OPNAV WEATHER FILING CRITERIA: Weather for filing based on: [1] Actual weather at departure at time of clearance [2] Existing and forecast weather for entire route of flight [3] Destination and alternate forecasts for period 1 hour before and after ETA.

## IFR FILING CRITERIA

DESTINATION (+/- 1 HR ETA)			ALTERNATE (+/- 1 HR ETA)

0-0 up to BNI pub mins				3000 – 3 or better

Pub mins up to BNI 3000 – 3				NON = Pub mins + 300-1

PRE  = Pub mins + 200-1/2

3000 – 3 or better					N/A

VISUAL APPROACH: (FTI 4-49)  Conducted on IFR flight plan.  Proceed visually to airport by having either the airport or the preceding aircraft in sight.  Must be authorized by ATC.  Always back it up with available navaids.  To fly visual approach: [1] Reported weather must be 1000 – 3 or greater [2] Authorized by ATC [3] Must be able to proceed visually clear of clouds [4] Must keep safe approach interval and adequate wake turbulence separation.  Don’t cancel IFR clearance.  Airspeed and configuration is at pilot’s discretion.  Complete the Landing Checklist no later than one mile from the runway.  Clearance for visual approach DOESN’T clear me for overhead/VFR traffic pattern!  Doesn’t have a missed approach segment.  At tower controlled airports, if have to go-around, tower will issue a clearance.  At uncontrolled airports, expected to remain clear of clouds and land as soon as possible.

PREFLIGHT PLANNING: (FTI 4-4)  Check weather, NOTAMs, fuel requirements, terminal approaches, alternates, anticipated traffic delays.  Tasks at Base Ops [PWANTS]:  Publications, Weather, Activate flight plan, TOLD data, SIDS/DPs/STARs/IAPs. [1] Publications- check FLIP, Enroute Supp, FIH, Charts, IAPs, TCNs, STARs.  Ensure all items are CURRENT!  [2] Weather – formal briefing and DD 175-1 for all flights in IMC.  Review IFR weather mins.  [3] Fuel Planning – fuel log for all cross country flights.  If alternate not required, must have fuel to fly to destination, plus a 10% reserve (max endurance at 10,000’).  If alternate required, must have fuel to fly to destination IAF and then to alternate airfield, plus a 10% reserve.  In no case shall reserve be less than 20 minutes.  Minimum on-deck fuel is 265 lbs per side.  Minimum fuel is advisory only (emergency could occur if any undue delay occur).  Min fuel is normally 265 lbs per side (yellow arc).  If min fuel, consider requesting short vectors to final if IMC or visual landing.  If fuel state requires traffic emergency, declare an emergency due to low fuel and report fuel remaining in minutes.  Emergency fuel typically called when 30 minutes of fuel remain (no set amount, can vary).  [4] Flight Plans: DD-175 info is in FLIP (chap. 4).  Avoid prohibited areas, restricted areas, and MOAs by minimum of 3nm (unless permission to cross is given by controlling agency).  Change of flight plan enroute: done with ATC (use back cover of enroute supp for proper flow- don’t use block headings, only information required).  If complicated, file with FSS (255.4 or charted VHF freq).  On initial call to FSS, identify freq on and VOR close to.  After filing, request weather update.  After FSS enters flight plan, allow ARTCC time to process request.  Close flight plans: Functioning control tower: will close IFR flight plans automatically.  No functioning control tower:  call ATC or FSS and close.  At military fields: verbally or physically with tower or base ops.  [5] NOTAMs: check on internet. 1. FDC (important info) 2. Attention Notices (military info) 3. Civilian D (Distant) aerodromes 4. Civilian L (Local): airport advisory, doesn’t prevent use of runways, found by calling local FSS for field 5.Notices to Airmen (NTAP) 6. Military Summary Reports: info on individual military airfields 7. GPS.  FSS has NOTAMs for all fields within 400 miles of their station.  Local NOTAMs must be requested.  [6] TOLD: Reducing gross weight is easiest way to improve takeoff and climb performance.  On cross country, don’t have tanks filled until determine gross weight limitations.  Accelerate-stop distance must be equal to or less than runway length (factor on wet runways).  One engine inop climb gradient must be 200’/nm or published gradient for departure procedure.  Restrictions increase at high density altitudes and in mountainous terrain.  Accelerate-Go Distance is a factor in bad weather from an airport with a runway end crossing height.  Enroute: one engine inop may be affected by MEAs.  Weight & Balance: full form must be filled out if four or more souls onboard for cross country.  [7] SIDs/DPs/STARs/IAPs: always check to see if airport flying into utilizes any of these procedures. [8] Civilian Airfield Ops: Get weather, NOTAMs, and file flight plan with FSS (1-800-Wx-Brief).  Ensure that the lineman are familiar with T-44 servicing procedures (fill the nacelle tanks before the wing tanks!).  Ensure rudder lock is removed if there is any chance the aircraft will be towed.  If no lineman is present, call “clear right/left” loudly out the window prior to starting each engine.  At civilian fields, preferable to use VHF.  At military fields, preferable to use UHF.  [9] Departure Message: Flight following primarily provided by ATC (also by military destination Base Ops dispatcher).  Departing a civilian field, must ensure actual departure time is passed to FSS serving departure field (very important when arriving at military fields).

FUEL PLANNING: (FTI 4-7)  Use a fuel log for all cross country flights.  If alternate not required, must have fuel to fly to destination, plus a 10% reserve (max endurance at 10,000’).  If alternate required, must have fuel to fly to destination IAF and then to alternate airfield, plus a 10% reserve.  In no case shall reserve be less than 20 minutes.  Minimum on-deck fuel is 265 lbs per side.  Minimum fuel is advisory only (emergency could occur if any undue delay occur).  Min fuel is normally 265 lbs per side (yellow arc).  If min fuel, consider requesting short vectors to final if IMC or visual landing.  If fuel state requires traffic emergency, declare an emergency due to low fuel and report fuel remaining in minutes.  Emergency fuel typically called when 30 minutes of fuel remain (no set amount, can vary).

FILING IN FLIGHT: (FTI 4-7) Change of flight plan enroute: done with ATC (use back cover of enroute supp for proper flow- don’t use block headings, only information required).  If complicated, file with FSS (255.4 or charted VHF freq).  On initial call to FSS, identify freq on and VOR close to.  After filing, request weather update.  After FSS enters flight plan, allow ARTCC time to process request.

GPS CHARACTERISTICS: (AIM 1-1-33)  U.S. satellite-based radio nav positioning and time transfer system run by the D.O.D.  Two levels of service: [1] Standard Positioning Service (SPS): accurate to 100 meters or less (95%) and 300 meters or less (99.99%). [2] Precise Positioning Service (PPS): more accurate than SPS, limited to authorized U.S. and allied military, gov, and civil users.  GPS receiver measures distance from a satellite through a radio signal, creates a ‘pseudo-range’ (which is subject to isopheric and troposhperic delays and multipath).  Receiver needs at least 4 satellites to yield a 3-D position (lat, long, and alt) and time.  GPS constellation of 24 satellites set up so 5 are observable anywhere on the Earth.  GPS receiver verifies integrity of signals from satellites through receiver autonomous integrity monitoring (RAIM).  RAIM needs at least 5 satellites in view to detect an anomaly, or 4 satellites and a barometric altimeter.  RAIM needs at least 6 satellites in view (or 5 with a baro-aid) to isolate corrupt satellite signal and remove it from nav solution.  Don’t rely on GPS altitude, vertical error can be very large.  To ensure baro-aid available, current altimeter setting must be set into receiver.  Without RAIM, no assurance of accuracy of GPS position.

GPS APPROACH PROCEDURES: (FTI 4-59 & GPS/FMS Notes 20 Jun 02)

*** Note: T-44 self-taught GPS training program is located in TWG-4 building in course rules computer room (1st floor on right side [if entering from rear]).  Course takes several hours to complete.

Study sources: [1]FMS/GPS computer software (in TWG-4 bldg.) [2]IEC Model 9002M FMS Operator’s Manual (where can I find this pub?) [3]NATOPS [4]AIM [5]AFMAN 11-217 V2.

Navigation Database: All named waypoints identified with 5-letter alpha character name.  Unnamed waypoints, such as a DME fix, have a 5 letter alphanumeric coded name in the database (exp. D234T represents a point on the 234 radial from XYZ vortac at 20DME [the letter T is the 20th letter of the alphabet and indicates 20nm]).  The T-44 GPS uses the Jeppesen NavData Database.

Approaches: 2 types: [1] Stand-Alone: constructed for GPS, absence of other navaids in approach title.  Straight-in minimums correspond to LNAV Minima on RNAV charts [2] Overlay: permit GPS under IFR to fly existing instrument approaches.  2 types: [A] GPS Not in the Title [B] GPS in the Title

Course Sensativity: CDI sensitivity varies with mode of operation.  GPS normally linear (straight line distance from point to point) regardless of distance from waypoint.  [1] Enroute Mode: prior to approach, full scale deflection = 5nm from centerline [2] Terminal Mode: within 30nm of destination airport on charted SID/DP/STAR or between last enroute waypoint and initial waypoint (IAF).  CDI full scale deflection = 1nm from centerline. [3] Approach Mode: sensitivity transitions from 5nm full deflection to 1nm full deflection within 30nm of destination airport.  Must be active to proceed past FAF on non-precision approach.  [4] Final Approach Mode: 2nm to the FAF waypoint, sensitivity transitions to CDI full scale deflection of 0.3nm either side centerline.  [5] Missed Approach Mode: nav to missed approach holding point, sensitivity 1nm of centerline.

RAIM: When GPS integrity doesn’t meet RAIM standards, integrity warning, signals GPS nav info may no longer be reliable (equivalent to ‘off flag’ on HSI.

Approach Restrictions: Instrument Approaches must be: [1]accomplished in accordance with FMS database using current update cycle [2]conducted in FMS Approach Mode.  RAIM must be available at FAF (INTG annunciator extinguished).  Aircraft must have other approved nav equipment.  GPS approaches can be flown without tuning, identifying, or monitoring other navaids (highly discouraged).

Preflight Actions: [1] Review GPS NOTAMS for destination and any alternates.  GPS satellite outages issued under pseudo random noise (PRN), number or satellite vehicle number (SVN).  Accessed using “KGPS” airport identifier.  Important to deselect any affected satellites using SV CONTROL function (on RAIM 2-2 page) of IEC 9002 FMS. [2] NavData NOTAMS: issued every 2 months.  Found at Jeppesen website (http://www.jeppesen.com/onlinepubs/navdatanot.phtml).  Check before flight. [3] If using GPS, aircraft is considered RNAV for flight planning purposes.  Use BE-9L/G in aircraft identification block.  [4] Check GPS ground equipment by start-up and self-test procedures as outlined in NATOPS.  Check currency of database, and predicted RAIM available status for approach plan to fly at ETA.

CRM Procedures: On deck, right-seat pilot enters flight plan into FMS.  In flight, pilot not flying (PNF) changes flight plan data.  PNF verbalizes changes.  Pilot flying (PF) verifies modification.  Both pilots review modified or previously loaded flight plan prior to execution.  Either pilot verbalize need to enter a discontinuity.  PNF will enter discontinuity and adivise PF when entry complete.  Either pilot verbalize need to close up discontinuity.  PNF receive concurrence from PF prior to closing it up.  Either pilot verbalize need to enter or edit a waypoint.  PNF enters the waypoint and executes it after receiving concurrence from PF.  PNF monitors flight progress on LEGS page.  Either pilot verbalizes if annunciator illums.  PNF looks up amplifying information.  In response to CDU APP annunciatior (APPR annunciator on instrument panel), either pilot verbalize approach mode still active, and other pilot acknowledge the call.  After missed approach, PF requests approach reselection.  PNF will acknowledge and comply, then inform PF when approach reselection is complete.  Prior to manually changing CDI sensitivity, PNF obtains concurrence from PF.  PF will verbalize disengaging FMS.  PNF will acknowledge.

Terminal Area Ops – Departure: Load SID/DP (from nav database).  If can’t be retreived, may not use RNAV to fly prior to SID/DP termination point.

Enroute Operations: Use of Predictive Integrity.  Check RAIM availability for planned approach.  If check inicates integrity may not be available, develop alternate plane for landing or proceed to alternate.

Prior to Descent: Compare approach retrieved from GPS nav database to GPS published approach on IAP.  If differences arise, the published approach chart, supplemented by NOTAMs, takes precedence.  Develop a back-up plan.  Consider the possibility of equipment failure past the FAF.  Load the STAR (if applicable).  If can’t be retreived, can’t use RNAV to fly procedure.

Terminal Area Ops – Arrival: Monitor all ground-based navaids since GPS approaches are flown point-to-point.  Bearing pointer on HSI and DME will be to next waypoint, not necessarily the field.  Be prepared to use traditional navaids.  Be wary of having head down in cockpit.

Approach Procedures: Reference NATOPS and IEC 9002M FMS Operator’s Manual.  Inbound Course: course displayed on FMS between FAF and MAP may be slightly different than printed on approach plate; should not affect approach performance.  GPS Integrity Warning Prior to FAF: don’t descent to MDA.  Instead, proceed to MAP via FAF, perform missed approach, and notify ATC.  Alternatively, may continue approach provided backup is available.  Stepdown Waypoints: nav database doesn’t provide stepdown waypoints after FAF.  In all cases, next waypoint is MAP.  Determine stepdown fixes and VDP by refering to GPS distance from MAP as depicted on approach plate.  GPS Integrity Warning After FAF: very serious, immediate action required.  Transition to back-up approach if available or proceed to MAP along FAC and execute missed approach.

Performing Missed Approach Procedures: MAP labeled with named waypoint.  Select Missed Approach Mode manually by using LEGS screen.  Clear MAP discontinuity by pressing Go Around Button on power lever within 2nm of FAF.  If Go Around Button used, EXEC key still needs to be pressed to execute missed approach.  Performing the missed approach: If initiated prior to MAP, proceed to MAP along FAC.  If missed approach includes a turn, don’t begin turn prior to MAP.  Fully review procedure before flying it!  Climb gradient: min of 200 ft/min unless higher published.

FUEL LOG: Refer to FTI Appx. C.

RNAV PROCEDURES: Refer to NATOPS 20-1 to 20-13 & RNAV System Notes 25 Mar 01 (p. 20-22)