MR. BAZEMORE’S GOUGE FOR

THE MILITARY COMPETENCE EXAM

COMPLIMENTS OF AVIATION ACADEMY

Only 20 minutes from the Harbor Bridge

2487 South Main Street

Ingleside, TX  78362

## PHONE OR FAX (361) 776-3011

e-mail:  bazeman@2fords.net

Every effort is made to provide you with current information. The FAA makes periodic changes to its tests. Please dispose of this testing guide after use. Current guides may be obtained from Aviation Academy via e-mail or fax, NAVALTX.NAVY.MIL/CTW4/, Mr. Bazemore, or the VT-31 student lounge. Please understand that this is a "guide" to the military competence test and that the FAA does not supply the correct answers to questions. In order to maintain the integrity of each test, the FAA may shuffle the answer stems and make modifications to the wording of the questions.

The military competence exam contains 50 multiple-choice questions. You will have 2 hours to complete the exam. The passing score is 70%. You may take the exam 2 years prior to your winging date.  After you take the test, you will meet with Mr. Bazemore to obtain your certificate.  You will need to bring him your:

- NATOPS jacket including your instrument rating

- Designation/Completion letter which states that you are a winged aviator

- Logbook showing a Military PIC check out or 10 hours PIC in the category and class of aircraft sought within the preceding 12 calendar months

- Results of the FAA Military Competence Test

- A completed Form FAA-8710-1 application (available at LaserGrade or at www.afs600.faa.gov )

You will be eligible to obtain your COMMERCIAL PILOT CERTIFICATE with AIRPLANE MULTIENGINE LAND and INSTRUMENT AIRPLANE ratings.  If you have received a military checkout as PIC or have flown 10 hours in a military single engine aircraft as PIC within the preceding 12 calendar months, you will also be eligible for your SINGLE ENGINE LAND CLASS rating.  Take note of the amount of time that you have before this will expire. If 12 months have passed and you desire the single engine rating, it will be necessary to do a single engine add-on, which requires a check ride with an examiner. Feel free to call Aviation Academy if you have any additional questions. Please do not try to contact Mr. Bazemore at the simulator building!

Call Kelley at Aviation Academy, 361-688-3011 (toll free from Corpus Christi) or 1-361-776-3011, with any questions and to schedule your test.  To take your military competence test and receive a certificate you will need a Military ID, Driver's License, and $110.00 cash or check.  This fee covers both the military competence exam and the processing fee for the certificate.

### DENNIS A. BAZEMORE, FAA PILOT EXAMINER

Thank you for using LaserGrade, offered by Aviation Academy.

Directions to LaserGrade Testing Center

Our testing center is located only 20 minutes from the Harbor Bridge.

Cross the Harbor Bridge and take US-181N through Portland.  Take the exit for TX-35 N, towards Aransas Pass/Rockport.  Take the South-TX 361 exit towards Ingleside/Port Aransas.

In Ingleside, turn right onto MAIN STREET (FM 1069), towards Naval Station Ingleside.  Veer left at the first light (the road splits).  Continue on Main Street for 1.0 miles.  The office is a small tan building on your right between Remy’s Garage (there is a large stop sign advertising the repair shop) and a rock house with a green metal roof.

If you have additional questions, please phone (361) 688-3011, 1-361-776- 3011 or 1-361-776-2955.

1.5.0.0.1.A.1               G10

Notification to the NTSB is required when there has been substantial damage

A.   which adversely affects structural strength or flight characteristics.

1.5.0.0.2.A.1               G11

NTSB Part 830 requires an immediate notification as a result of which incident?

A.   Any required flight crewmember being unable to perform flight duties because of illness.

1.5.0.0.3.A.1               G11

Which incident would require that the nearest NTSB field office be notified immediately?

A.   In-flight fire.

1.5.0.0.3.A.2               G11

Which airborne incident would require that the nearest NTSB field office be notified immediately?

A.   Flight control system malfunction or failure.

1.5.0.0.4.A.1               G11

While taxiing for takeoff, a small fire burned the insulation from a transceiver wire. What action would be required to comply with NTSB Part 830?

A.   No notification or report is required.

1.5.0.0.4.A.2               G11

While taxiing on the parking ramp, the landing gear, wheel, and tire are damaged by striking ground equipment. What action would be required to comply with NTSB Part 830?

A.   No notification or report is required.

1.5.0.0.5.A.1               G11

During flight a fire, which was extinguished, burned the insulation from a transceiver wire. What action is required by regulations?

A.   An immediate notification by the operator of the aircraft to the nearest NTSB field office.

1.5.0.0.6.A.1               G11

When should notification of an aircraft accident be made to the NTSB if there was substantial damage and no injuries?

A.   Immediately.

1.5.0.0.7.A.1               G13

The operator of an aircraft that has been involved in an incident is required to submit a report to the nearest field office of the NTSB

A.   only if requested to do so.

1.5.0.0.8.A.1               G13

How many days after an accident is a report required to be filed with the nearest NTSB field office?

- A.   10.

1.5.0.0.9.A.1               J08

What designated airspace associated with an airport becomes inactive when the control tower at that airport is not in operation?

A.   Class D, which then becomes Class E.

1.5.0.1.0.A.1               A01

Regulations which refer to "commercial operators" relate to that person who

A.   for compensation or hire, engages in the carriage by aircraft in air commerce of persons or property, other than as an air carrier.

1.5.0.1.1.A.1               A01

Regulations which refer to "operate" relate to that person who

A.   causes the aircraft to be used or authorizes its use.

1.5.0.1.2.A.1               A01

- Regulations which refer to the "operational control" of a flight are in relation to

A.   exercising authority over initiating, conducting, or terminating a flight.

1.5.0.1.3.A.1               A02

Which is the correct symbol for the stalling speed or the minimum steady flight speed in a specified configuration?

- A.   VS1.

1.5.0.1.4.A.1               A02

Which is the correct symbol for the stalling speed or the minimum steady flight speed at which the airplane is controllable?

- A.   VS.

1.5.0.1.5.A.1               A02

14 CFR part 1 defines VF as

A.   design flap speed.

1.5.0.1.6.A.1               A02

14 CFR part 1 defines VLE as

A.   maximum landing gear extended speed.

1.5.0.1.6.A.2               A02

14 CFR part 1 defines VNE as

A.   never-exceed speed.

1.5.0.1.6.A.3               A02

14 CFR part 1 defines VY as

A.   speed for best rate of climb.

1.5.0.1.7.A.1               A150

If an airplane category is listed as utility, it would mean that this airplane could be operated in which of the following maneuvers?

A.   Limited acrobatics, including spins (if approved).

1.5.0.1.8.A.1               A20

Commercial pilots are required to have a valid and appropriate pilot certificate in their personal possession when

A.   acting as pilot in command.

1.5.0.1.9.A.1               A20

Which of the following are considered aircraft class ratings?

A.   Single-engine land, multiengine land, single-engine sea, and multiengine sea.

1.5.0.2.0.A.1               A20

Does a commercial pilot certificate have a specific expiration date?

A.   No, it is issued without a specific expiration date.

1.5.0.2.1.A.1               A20

A second-class medical certificate issued to a commercial pilot on April 10, this year, permits the pilot to exercise which of the following privileges?

A.   Commercial pilot privileges through April 30, next year.

1.5.0.2.2.A.1               A20

When is the pilot in command required to hold a category and class rating appropriate to the aircraft being flown?

A.   On flights when carrying another person.

1.5.0.2.3.A.1               A20

Unless otherwise authorized, the pilot in command is required to hold a type rating when operating any

A.   aircraft of more than 12,500 pounds maximum certificated takeoff weight.

1.5.0.2.4.A.1               A20

To act as pilot in command of an airplane that is equipped with retractable landing gear, flaps, and controllable-pitch propeller, a person is required to

A.   receive and log ground and flight training in such an airplane, and obtain a logbook endorsement certifying proficiency.

1.5.0.2.5.A.1               A20

What flight time may a pilot log as second in command?

A.   All flight time when qualified and occupying a crewmember station in an aircraft that requires more than one pilot.

1.5.0.2.6.A.1               A20

What flight time must be documented and recorded by a pilot exercising the privileges of a commercial certificate?

A.   Flight time showing training and aeronautical experience to meet requirements for a certificate, rating or flight review.

1.5.0.2.7.A.1               A20

If a pilot does not meet the recency of experience requirements for night flight and official sunset is 1900 CST, the latest time passengers should be carried is

- A.   1959 CST.

1.5.0.2.8.A.1               A20

Prior to carrying passengers at night, the pilot in command must have accomplished the required takeoffs and landings in

A.   the same category, class, and type of aircraft (if a type rating is required).

1.5.0.3.0.A.1               A20

No pilot may act as pilot in command of an airship under IFR or in weather conditions less than the minimums prescribed for VFR unless that pilot has, within the past 6 months, performed and logged under actual or simulated instrument conditions, at least

A.   six instrument approaches, holding procedures, intercepting and tracking courses, or passed an instrument proficiency check in an airship.

1.5.0.3.1.A.1               A20

To act as pilot in command of an aircraft operated under 14 CFR part 91, a commercial pilot must have satisfactorily accomplished a flight review or completed a proficiency check within the preceding

A.   24 calendar months.

1.5.0.3.2.A.1               A20

Pilots, who change their permanent mailing address, and fail to notify the FAA Airmen Certification Branch of this change, are entitled to exercise the privileges of their pilot certificate for a period of

A.   30 days.

1.5.0.3.3.B.1               A21

To act as pilot in command of an airplane towing a glider, the tow pilot is required to have

A.   a logbook endorsement from an authorized glider instructor certifying receipt of ground and flight training in gliders, and be proficient with techniques and procedures for safe towing of gliders.

1.5.0.3.4.B.1               A21

To act as pilot in command of an airplane towing a glider, a pilot must have accomplished, within the preceding 12 months, at least

A.   three actual or simulated glider tows while accompanied by a qualified tow pilot.

1.5.0.3.9.B.1               A24

What limitation is imposed on a newly certificated commercial pilot - airplane, if that person does not hold an instrument rating? The carriage of passengers

A.   for hire on cross-country flights in excess of 50 NM or for hire at night is prohibited.

1.5.0.4.1.A.1               A26

What is the maximum amount of flight instruction an authorized instructor may give in any 24 consecutive hours?

A.   8 hours.

1.5.0.4.3.A.1               A66

- Excluding Hawaii, the vertical limits of the Federal Low Altitude airways extend from

A.   1,200 feet AGL up to, but not including, 18,000 feet MSL.

1.5.0.4.4.B.1               B07

What action must be taken when a pilot in command deviates from any rule in 14 CFR part 91?

A.   Upon the request of the Administrator, send a written report of that deviation to the Administrator.

1.5.0.4.5.B.1               B07

Who is responsible for determining if an aircraft is in condition for safe flight?

A.   The pilot in command.

1.5.0.4.6.B.1               B07

When operating a U.S.-registered civil aircraft, which document is required by regulation to be available in the aircraft?

A.   A current, approved Airplane Flight Manual.

1.5.0.4.7.B.1               B07

A pilot in command (PIC) of a civil aircraft may not allow any object to be dropped from that aircraft in flight

A.   if it creates a hazard to persons and property.

1.5.0.4.8.A.1               B08

Which person is directly responsible for the prelaunch briefing of passengers for a balloon flight?

A.   Pilot in command.

1.5.0.4.9.B.1               B08

When is preflight action required, relative to alternatives available, if the planned flight cannot be completed?

A.   Any flight not in the vicinity of an airport.

1.5.0.4.9.B.2               B08

The required preflight action relative to weather reports and fuel requirements is applicable to

A.   Any flight not in the vicinity of an airport.

1.5.0.5.0.B.1               B08

Before beginning any flight under IFR, the pilot in command must become familiar with all available information concerning that flight. In addition, the pilot must

A.   be familiar with the runway lengths at airports of intended use, and the alternatives available, if the flight cannot be completed.

1.5.0.5.0.B.2               B08

Before beginning any flight under IFR, the pilot in command must become familiar with all available information concerning that flight. In addition, the pilot must

A.   be familiar with the runway lengths at airports of intended use, weather reports, fuel requirements, and alternatives available, if the planned flight cannot be completed.

1.5.0.5.1.A.1               B08

Required flight crewmembers' seatbelts must be fastened

A.   while the crewmembers are at their stations.

1.5.0.5.1.A.2               B08

Each required flight crewmember is required to keep his or her shoulder harness fastened

A.   during takeoff and landing, unless he or she is unable to perform required duties.

1.5.0.5.2.A.1               B08

With U.S.-registered civil airplanes, the use of seatbelts is required during movement on the surface, takeoffs, and landings for

A.   each person over 2 years of age on board.

1.5.0.5.5.A.1               B12

Which is required to operate an aircraft towing an advertising banner?

A.   A certificate of waiver issued by the Administrator.

1.5.0.5.6.B.1               B07

Portable electronic devices which may cause interference with the navigation or communication system may not be operated on a  U.S.- registered civil aircraft being flown

A.   in air carrier operations.

1.5.0.5.6.B.2               B07

Portable electronic devices which may cause interference with the navigation or communication system may not be operated on U.S.-registered civil aircraft being operated

A.   under IFR.

1.5.0.5.9.A.1               B10

If weather conditions are such that it is required to designate an alternate airport on your IFR flight plan, you should plan to carry enough fuel to arrive at the first airport of intended landing, fly from that airport to the alternate airport, and fly thereafter for

A.   45 minutes at normal cruising speed.

1.5.0.6.0.A.1               B11

- A coded transponder equipped with altitude reporting equipment is required for

A.   Class A, Class B, and Class C airspace areas.

1.5.0.6.1.A.1               B11

In the contiguous U.S., excluding the airspace at and below 2,500 feet AGL, an operable coded transponder equipped with Mode C capability is required in all airspace above

A.   10,000 feet MSL.

1.5.0.6.2.A.1               B10

What is the maximum tolerance (+ or -) allowed for an operational VOR equipment check when using an FAA-approved ground test signal?

A.   4 degrees.

1.5.0.6.3.A.1               B11

In accordance with14 CFR part 91, supplemental oxygen must be used by the required minimum flightcrew for that time exceeding 30 minutes while at cabin pressure altitudes of

A.   12,500 feet MSL up to and including 14,000 feet MSL.

1.5.0.6.4.A.1               B11

What are the oxygen requirements when operating at cabin pressure altitudes above 15,000 feet MSL?

A.   The flightcrew and passengers must be provided with supplemental  oxygen.

1.5.0.6.5.A.1               B11

Which is required equipment for powered aircraft during VFR night flights?

A.   Anticollision light system.

1.5.0.6.6.A.1               B11

Which is required equipment for powered aircraft during VFR night flights?

A.   An electric landing light, if the flight is for hire.

1.5.0.6.7.A.2               B11

Approved flotation gear, readily available to each occupant, is required on each airplane if it is being flown for hire over water,

A.   beyond power-off gliding distance from shore.

1.5.0.6.8.B.2               B12

Which is true with respect to operating limitations of a "restricted'' category airplane?

A.   No person may operate a "restricted" category airplane carrying  passengers or property for compensation or hire.

1.5.0.6.8.B.3               B12

Which is true with respect to operating limitations of a "primary'' category airplane?

A.   No person may operate a "primary" category airplane carrying  passengers or property for compensation or hire.

1.5.0.6.9.A.1               B12

The carriage of passengers for hire by a commercial pilot is

A.   not authorized in a "limited" category aircraft.

1.5.0.7.0.A.1               B11

The maximum cumulative time that an emergency locator transmitter may be operated before the rechargeable battery must be recharged is

A.   60 minutes.

1.5.0.7.1.A.1               B07

No person may operate a large civil aircraft of U.S. registry which is subject to a lease, unless the lessee has mailed a copy of the lease to the FAA Aircraft Registration Branch, Technical Section, Oklahoma City, OK, within how many hours of its execution?

- A.   24.

1.5.0.7.2.B.2               B11

What transponder equipment is required for airplane operations within Class B airspace?  A transponder

A.   with 4096 code or Mode S, and Mode C capability.

1.5.0.7.3.B.1               B08

- Which is true with respect to formation flights? Formation flights are

A.   not authorized when carrying passengers for hire.

1.5.0.7.3.B.2               B08

- Which is true with respect to operating near other aircraft in flight?  They are

A.   not authorized, when operated so close to another aircraft they can create a collision hazard.

1.5.0.7.3.B.3               B08

- Which is true with respect to formation flights?  Formation flights are

A.   not authorized, except by arrangement with the pilot in command of each aircraft.

1.5.0.7.4.A.1               B08

While in flight a helicopter and an airplane are converging at a 90° angle, and the helicopter is located to the right of the airplane. Which aircraft has the right-of-way, and why?

A.   The helicopter, because it is to the right of the airplane.

1.5.0.7.5.A.1               B08

Two aircraft of the same category are approaching an airport for the purpose of landing. The right-of-way belongs to the aircraft

A.   at the lower altitude, but the pilot shall not take advantage of this rule to cut in front of or to overtake the other aircraft.

1.5.0.7.6.A.3               B08

During a night operation, the pilot of aircraft #1 sees only the green light of aircraft #2. If the aircraft are converging, which pilot has the right-of-way? The pilot of aircraft

A.   #1; aircraft #1 is to the right of aircraft #2.

1.5.0.7.6.A.4               B08

A pilot flying a single-engine airplane observes a multiengine airplane approaching from the left.  Which pilot should give way?

A.   The pilot of the multiengine airplane should give way; the single-engine airplane is to its right.

1.5.0.7.6.B.1               B08

Airplane A is overtaking airplane B. Which airplane has the right-of-way?

A.   Airplane B; the pilot should expect to be passed on the right.

1.5.0.7.6.B.2               B08

An airplane is overtaking a helicopter. Which aircraft has the right-of-way?

A.   Helicopter; the pilot should expect to be passed on the right.

1.5.0.7.7.A.1               B08

What is the maximum indicated airspeed authorized in the airspace underlying Class B airspace?

A.   200 knots.

1.5.0.7.8.A.1               B08

Unless otherwise authorized or required by ATC, the maximum indicated airspeed permitted when at or below 2,500 feet AGL within 4 NM of the primary airport within Class C or D airspace is

- A.   200 knots.

1.5.0.7.9.A.1               B12

What is the minimum altitude and flight visibility required for acrobatic flight?

- B.   1,500 feet AGL and 3 miles.

1.5.0.8.0.A.1               B11

- If not equipped with required position lights, an aircraft must terminate flight

A.   at sunset.

1.5.0.8.0.A.2               B11

If an aircraft is not equipped with an electrical or anticollision light system, no person may operate that aircraft

A.   after sunset to sunrise.

1.5.0.8.0.A.2               B11

If an aircraft is not equipped with an anticollision light system, no person may operate that aircraft

A.   after sunset.

1.5.0.8.2.B.1               B08

Which is true regarding flight operations in Class B airspace?

A.   The pilot must receive an ATC clearance before operating an aircraft in that area.

1.5.0.8.2.B.2               B08

Which is true regarding pilot certification requirements for operations in Class B airspace?

A.   The pilot in command must hold at least a private pilot certificate.

1.5.0.8.2.B.3               B08

Which is true regarding flight operations in Class B airspace?

A.   The aircraft must be equipped with an ATC transponder and altitude reporting equipment.

1.5.0.8.3.A.1               B08

The minimum flight visibility for VFR flight increases to 5 statute miles beginning at an altitude of

A.   10,000 feet MSL if above 1,200 feet AGL.

1.5.0.8.5.A.1               B08

What is the minimum flight visibility and proximity to cloud requirements for VFR flight, at 6,500 feet MSL, in Class C, D, and E airspace?

A.   3 miles visibility; 1,000 feet above and 500 feet below.

1.5.0.8.8.A.1               B09

When operating an airplane for the purpose of takeoff or landing within Class D airspace under special VFR, what minimum distance from clouds and what visibility are required?

A.   Remain clear of clouds, and the ground visibility must be at least 1 SM.

1.5.0.8.9.A.1               B09

At some airports located in Class D airspace where ground visibility is not reported, takeoffs and landings under special VFR are

A.   authorized by ATC if the flight visibility is at least 1 SM.

1.5.0.9.0.A.1               B09

To operate an airplane under SPECIAL VFR (SVFR) within Class D airspace at night, which is required?

A.   The pilot must hold an instrument rating, and the airplane must be equipped for instrument flight.

1.5.0.9.1.A.1               B09

- VFR cruising altitudes are required to be maintained when flying

A.   more than 3,000 feet AGL, based on magnetic course.

1.5.0.9.2.A.1               B10

Except when necessary for takeoff or landing or unless otherwise authorized by the Administrator, the minimum altitude for IFR flight is

A.   2,000 feet above the highest obstacle over designated mountainous terrain; 1,000 feet above the highest obstacle over terrain elsewhere.

1.5.0.9.3.A.1               B13

Who is primarily responsible for maintaining an aircraft in an airworthy condition?

A.   Owner or operator of the aircraft.

1.5.0.9.4.A.1               B13

- Assuring compliance with an Airworthiness Directive is the responsibility of the

A.   owner or operator of that aircraft.

1.5.0.9.5.A.1               B13

After an annual inspection has been completed and the aircraft has been returned to service, an appropriate notation should be made

A.   in the aircraft maintenance records.

1.5.0.9.6.B.1               B13

- A standard airworthiness certificate remains in effect as long as the aircraft receives

A.   required maintenance and inspections.

1.5.0.9.7.A.1               B13

If an aircraft's operation in flight was substantially affected by an alteration or repair, the aircraft documents must show that it was test flown and approved for return to service by an appropriately-rated pilot prior to being operated

A.   with passengers aboard.

1.5.0.9.8.A.1               B13

Which is correct concerning preventive maintenance, when accomplished by a pilot?

A.   A record of preventive maintenance must be entered in the maintenance records.

1.5.0.9.9.A.1               B13

An aircraft carrying passengers for hire has been on a schedule of inspection every 100 hours of time in service. Under which condition, if any, may that aircraft be operated beyond 100 hours without a new inspection?

A.   The 100-hour limitation may be exceeded by not more than 10 hours if necessary to reach a place at which the inspection can be done.

1.5.1.0.0.A.1               B13

Which is true concerning required maintenance inspections?

A.   An annual inspection may be substituted for a 100-hour inspection.

1.5.1.0.1.A.1               B13

An ATC transponder is not to be used unless it has been tested, inspected, and found to comply with regulations within the preceding

A.   24 calendar months.

1.5.1.0.2.A.1               B13

- Aircraft maintenance records must include the current status of the

A.   life-limited parts of each airframe, engine, propeller, rotor, and appliance.

1.5.1.0.3.A.1               B13

Which is true relating to Airworthiness Directives (AD's) ?

A.   Noncompliance with AD's renders an aircraft unairworthy.

1.5.1.0.4.A.1               B13

A new maintenance record being used for an aircraft engine rebuilt by the manufacturer must include previous

A.   changes as required by Airworthiness Directives.

1.5.1.0.5.A.1               B13

If an ATC transponder installed in an aircraft has not been tested, inspected, and found to comply with regulations within a specified period, what is the limitation on its use?

A.   Its use is not permitted.

1.5.1.0.6.B.1               A20

To act as pilot-in-command of an airplane with more than 200 horsepower, a person is required to

A.   receive and log ground and flight training from an authorized instructor in such an airplane.

1.5.1.0.7.B.1               A20

To serve as pilot in command of an airplane that is certified for more than one pilot crewmember, and operated under part 91, a person must

A.   complete a pilot-in-command proficiency check within the preceding 12 calendar months in an airplane that is type certificated for more than one pilot.

1.5.1.0.8.B.1               A20

To serve as second in command of an airplane that is certificated for more than one pilot crewmember, and operated under part 91, a person must

A.   within the last 12 months become familiar with the required information, and perform and log pilot time in the type of  airplane for which privileges are requested.

1.5.1.0.9.B.1               B07

What person is directly responsible for the final authority as to the operation of the airplane?

A.   Pilot in command.

1.5.1.1.0.B.1               B08

Operating regulations for U.S.-registered civil airplanes require that during movement on the surface, takeoffs, and landings, a seat belt and shoulder harness (if installed) must be properly secured about each

A.   person on board.

1.5.1.1.1.B.1               B08

- No person may operate an aircraft in simulated instrument flight conditions unless the

A.   other control seat is occupied by a safety pilot, who holds at least a private pilot certificate and is appropriately rated.

1.5.1.1.2.B.1               B08

If the minimum safe speed for any particular operation is greater than the maximum speed prescribed in 14 CFR part 91, the

A.   aircraft may be operated at that speed.

1.5.1.1.4.B.1               B08

What altimeter setting is required when operating an aircraft at 18,000 feet MSL?

A.   29.92" Hg.

1.5.1.1.5.B.1               B08

After an ATC clearance has been obtained, a pilot may not deviate from that clearance, unless the pilot

A.   recieves an amended clearance or has an emergency.

1.5.1.1.6.B.1               B08

When approaching to land at an airport, without an operating control tower, in Class G airspace, the pilot should

A.   make all turns to the left, unless otherwise indicated.

1.5.1.1.7.B.1               B08

When operating an aircraft in the vicinity of an airport with an operating control tower, in Class E airspace, a pilot must establish communications prior to

A.   4 NM, and up to and including 2,500 feet AGL.

1.5.1.1.8.B.1               B08

When approaching to land at an airport with an ATC facility, in Class D airspace, the pilot must establish communications prior to

A.   4 NM, up to and including 2,500 feet AGL.

1.5.1.1.9.B.1               B08

Which is true regarding flight operations to or from a satellite airport, without an operating control tower, within the Class C airspace area?

A.   Aircraft must be equipped with an ATC transponder and altitude reporting equipment.

1.5.1.1.9.B.2               B08

Which is true regarding flight operations to or from a satellite airport, without an operating control tower, within the Class C airspace area?

A.   Prior to entering that airspace, a pilot must establish and maintain communication with the ATC serving facility.

1.5.1.2.0.B.1               B08

Which is true regarding flight operations in Class A airspace?

A.   Must conduct operations under instrument flight rules.

1.5.1.2.0.B.2               B08

Which is true regarding flight operations in Class A airspace?

A.   Aircraft must be equipped with an ATC transponder and altitude reporting equipment.

1.5.1.2.1.B.1               B08

When weather information indicates that abnormally high barometric pressure exists, or will be above _____ inches of mercury, flight operations will not be authorized contrary to the requirements published in NOTAMs.

- A.   31.00

1.5.1.2.2.B.1               B10

When must an operational check on the aircraft VOR equipment be accomplished to operate under IFR?  Within the preceding

A.   30 days.

1.5.1.2.2.B.2               B10

Which data must be recorded in the aircraft logbook or other record by a pilot making a VOR operational check for IFR operations?

A.   Date of check, place of operational check, bearing error, and signature.

1.5.1.2.3.B.1               B10

For an airport with an approved instrument approach procedure to be listed as an alternate airport on an IFR flight plan, the forecasted weather conditions at the time of arrival must be at or above the following weather minimums.

A.   Ceiling 800 feet and visibility 2 SM for nonprecision.

1.5.1.2.3.B.2               B10

For an airport without an approved instrument approach procedure to be listed as an alternate airport on an IFR flight plan, the forecasted weather conditions at the time of arrival must have at least a

A.   ceiling and visibility that allows for a descent, approach, and landing under basic VFR.

1.5.1.2.4.A.2               B10

- Pilots are not authorized to land an aircraft from an instrument approach unless the

A.   flight visibility is at, or exceeds the visibility prescribed in the approach procedure being used.

1.5.1.2.4.B.1               B10

On an instrument approach where a DH or MDA is applicable, the pilot may not operate below, or continue the approach unless the

A.   aircraft is continuously in a position from which a descent to a normal landing, on the intended runway, can be made.

1.5.1.2.4.B.3               B10

A pilot performing a published instrument approach is not authorized to perform a procedure turn when

A.   receiving a radar vector to a final approach course or fix.

1.5.1.2.5.B.1               B10

The pilot in command of an aircraft operated under IFR, in controlled airspace, not in radar contact, shall report by radio as soon as possible when

A.   passing each designated reporting point, to include time and altitude.

1.5.1.2.5.B.2               B10

The pilot in command of an aircraft operated under IFR, in controlled airspace, shall report as soon as practical to ATC when

A.   experiencing any malfunctions of navigational, approach, or communications equipment, occurring in flight.

1.5.1.2.6.B.1               A24

A person with a Commercial Pilot certificate may act as pilot in command of an aircraft for compensation or hire, if that person

A.   is qualified in accordance with 14 CFR part 61 and with the applicable parts that apply to the operation.

1.5.1.2.6.B.2               A24

A person with a commercial pilot certificate may act as pilot in command of an aircraft carrying persons or property for compensation or hire, if that person

A.   is qualified in accordance with 14 CFR part 61 and with the applicable parts that apply to the operation.

1.5.1.2.8.B.1               A20

To act as pilot in command of a tailwheel airplane, without prior experience, a pilot must

A.   receive and log flight training from and authorized instructor

1.5.1.2.9.B.1               B12

- No person may operate an aircraft that has an experimental airworthiness certificate

- when carrying persons or property for hire.

1.5.1.3.0.B.1               H564

- For night flying operations, the best night vision is achieved when the

A.   rods in the eyes have become adjusted to the darkness in approximately 30 minutes.

1.5.1.3.3.B.1               H568

- When planning a night cross-country flight, a pilot should check for

A.   availability and status of en route and destination airport lighting systems.

1.5.1.3.4.B.1               H568

Light beacons producing red flashes indicate

A.   obstructions or areas considered hazardous to aerial navigation.

1.5.1.3.5.B.1               H572

What is the first indication of flying into restricted visibility conditions when operating VFR at night?

A.   A gradual disappearance of lights on the ground.

1.5.1.3.6.B.1               H574

After experiencing a powerplant failure at night, one of the primary considerations should include

A.   planning the emergency approach and landing to an unlighted portion of an area.

1.5.1.3.7.B.1               H574

When planning for an emergency landing at night, one of the primary considerations should include

A.   selecting a landing area close to public access, if possible.

1.5.1.3.8.B.1               J13

Who has the final authority to accept or decline any "land and hold short" (LAHSO) clearance?

A.   Pilot-in-Command.

1.5.1.3.9.B.1               J13

When should pilots decline a "land and hold short" (LAHSO) clearance?

A.   When it will compromise safety.

1.5.1.4.0.B.1               J13

What is the minimum visibility and ceiling required for a pilot to receive a "land and hold short" clearance?

A.   3 statute miles and 1,000 feet.

1.5.1.4.1.B.1               A20

A pilot convicted of operating a motor vehicle while either intoxicated by, impaired by, or under the influence of alcohol or a drug is required to provide a

A.   written report to the FAA Civil Aviation Security Division (AMC-700) not later than 60 days after the conviction.

1.5.1.4.2.B.1               A20

A pilot convicted of a motor vehicle offense involving alcohol or drugs is required to provide a written report to the

A.   FAA Civil Aviation Security Division (AMC-700) within 60 days after such action.

1.5.1.4.3.B.1               A20

A pilot convicted for the violation of any Federal or State statute relating to the process, manufacture, transportation, distribution, or sale of narcotic drugs is grounds for

A   suspension or revocation of any certificate, rating, or authorization issued under 14 CFR part 61.

1.5.1.4.4.B.1               A20

A pilot convicted of operating an aircraft as a crewmember under the influence of alcohol, or using drugs that affect the person's faculties, is grounds for a

A.   denial of an application for an FAA certificate or rating.

- HELICOPTER SUPPLEMENT

1.5.0.2.9.A.1               A20

To act as pilot in command of a gyroplane carrying passengers, what must the pilot accomplish in that gyroplane to meet recent daytime flight experience requirements?

A.   Make three takeoffs and landings within the preceding 90 days.

1.5.0.5.8.A.1               B09

To begin a flight in a rotorcraft under VFR, there must be enough fuel to fly to the first point of intended landing and, assuming normal cruise speed, to fly thereafter for at least

- 20 minutes.

1.5.0.6.7.A.1               B11

Approved flotation gear, readily available to each occupant, is required on each helicopter if it is being flown for hire over water,

A.   beyond power-off gliding distance from shore.

1.5.0.6.8.B.1               B12

Which is true with respect to operating limitations of a "restricted'' category helicopter?

A.   No person may operate a "restricted" category helicopter carrying  passengers or property for compensation or hire.

1.5.0.7.2.B.1               B11

What transponder equipment is required for helicopter operations within Class B airspace? A transponder

A.   with 4096 code and Mode C capability.

1.5.0.8.6.A.1               B08

Which minimum flight visibility and distance from clouds is required for a day VFR helicopter flight in Class G airspace at 3,500 feet MSL over terrain with an elevation of 1,900 feet MSL?

A.   Visibility-1 mile; distance from clouds-500 feet below, 1,000 feet above, and 2,000 feet horizontally.

1.5.0.8.7.A.1               B08

Basic VFR weather minimums require at least what visibility for operating a helicopter within Class D airspace?

A.   3 miles.

1.5.1.1.0.B.2               B08

Operating regulations for U.S.-registered civil helicopters require that during movement on the surface, takeoffs, and landings, a seat belt and shoulder harness (if installed) must be properly secured about each

A.   person on board.

1.5.1.1.3.B.1               B08

Minimum safe altitude rules require that helicopter pilots

A.   comply with routes and altitudes prescribed by the FAA.

1.5.1.1.3.B.2               B08

Minimum safe altitude rules authorize helicopter pilots to

- fly at less than 500 feet if they do not create a hazard to persons or property on the surface.

1.5.1.1.6.B.2               B08

When approaching to land at an airport, without an operating control tower, in Class G airspace, a helicopter pilot should

- avoid the flow of fixed-wing aircraft.