RI-2 Study Guide (08 Jun 02)

__ IFR COMM (APPX. A)

__ IFR BRIEFS (APPX. A & AIGT, CHAP. 8)

__ IFR LOST COMM PROCEDURES (COURSE RULES – APPX B5)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES (DISREGARD SEAGULL)

__ CHECKLIST PROCEDURES (REFER TO POCKET CHECKLIST & FTI APPENDIX E)

__ EMERGENCY PROCEDURES

__ RADIO CALLS

__ START CHECKLIST

__ SSE PATTERN

__ LANDING PATTERN (REVIEW CABANISS PATTERN NOTES – 16 APR 02)

__ NCS-31 (RADIO/NAVAID CHECK/SET)

*** All training events with Student Naval Aviator (SNA) shall observe single-piloted approach criteria and minimums unless he is RI-18 complete.  SMA’s are ALWAYS limited to 200-1/2 when at the controls executing an actual instrument approach.  The IP can allow the student to execute approaches to 100-1/4 in VMC.

OBSERVERS DUTIES: (FTI 4-65)  Clear for conflicting traffic, ensure gear is down for landings, back up pilot on radios.  Post on same side as student flying.  Note: Foggles can be used when an observer is on the flight.

IFR LOST COMMUNICATIONS: Refer to Course Rules Booklet (Appx B5)

[A] AFTER TAKEOFF: [1] Climb on last hdg or route assigned [2] Maintain 1,500’ [3] If can’t estab comm w/ Corpus Christi Dept within 3 minutes, climb & maintain 2,500’.  [3] After reaching 2,500’, proceed direct to approach fix for NAS Corpus and execute any approach.

[B] PRIOR TO VFR ON Sagebrush, Surf, or Rooke flight plans, if comm w/ Corpus Departure is lost & cleared > 2,500’, [1] Climb on last assigned hdg/route until reaching assigned altitude [2] Reaching assigned altitude, proceed direct to approach fix for NAS Corpus and execute any approach.

[C] GCA PATTERN: If no transmission received within 1 min in GCA pattern, proceed direct to approach fix for NAS Corpus and execute approach.

[D] ON FINAL: [1] If no transmission received within 15 sec (ASR) or 5 sec (PAR), alter course to intercept VORTAC final approach course for runway in use & execute a TACAN/VOR approach [TACAN Z APPROACH].  [2] Otherwise, climb and maintain 1,500’, proceed direct to an approach fix and execute any approach.

CLEARANCE COMPLIANCE: Comply with last verbal instructions given by ATC (read back all instructions: headings, altitudes, and altimeter settings).

# __ IFR DEPARTURES: (FTI 4-13 to 4-18)  Refer to FTI highlight notes.

# __ PAR APPROACH (NORMAL): (NATOPS 18-4, FTI 4-41) Refer to Normal PAR Approach notes (27 May 02)

ASR APPROACH (NORMAL): (NATOPS 18-5, FTI 4-42) Request controller provide recommended altitudes on final.  Controller will tell pilot approach to fly, MDA, MAP location, and advance notice of when descent to MDA begin.  Optimum final descent gradient = 600fpm (3 degrees at 120K) on VSI.

Single Engine ASR: treat as SSE precision approach for configuration.  If get well below recommended altitudes, revert to non-precision configuration, retract flaps and gear to get at or above MDA.  If not going to use recommended altitudes on final, treat as non-precision approach.  Slow to 120K after configuration point, but before descent to MDA, gear should be left in UP.  Continue as per FTI-4-39 ‘Single Engine Non-Precision Approach.’

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.

FLAP LANDINGS

[a] Approach 	Final = 105K	Threshold = 95K

[b] Full		Final = 105K	Threshold = 95K   (Select after 90, but before rolling onto final) Additional

power required.

[c] No flap	Final = 110K	Threshold = 105K Slowly close power levers while gradually flaring.  Tends to

balloon and then sink rapidly.

# LANDING GEAR SYSTEM (NATOPS 2-27 to 2-32, 16-6 to 16-9)

1. Nosewheel steering actuates nose gear 12 degrees left and 14 degrees right.  (48 degrees either side with braking)

2. The function that up and down limit switches share is deactivates landing gear motor and prevents overtravel of gear.

3. The up limit switch also deactivates the g-meter when the landing gear is extended.

4. The down limit switch also activates the AOA indexer lights.  On the ground, the right main squat switch deactivates the AOA indexer lights.

5. The three up locks all will turn on the red lights in the gear selector handle, illum the wheels up light, and activate the gear warning horn if the gear don’t fully retract.

6. The right up lock also deactivates the prop sync light when the prop sync switch is ON.  If the right up lock has gear fully retracted, the prop sync light will not come on when the prop sync switch in ON.

7. The left up lock also keeps the loading for the electric heat in NORM & GND MAX within slight fluctuation or no significant generator load difference.   If the left up lock has gear fully retracted, switching the electric heat from NORM to GND MAX will yield only a slight or no significant change in generator loading.

8. The nose up lock also deactivates the landing lights.  If up lock has gear fully retracted, no illumination of landing lights and no increase in generator loading when landing lights are turned ON.

9. The right squat switch: [GFISH] on ground

Gear Circuit inop

Flight hour meter inop

Inlet lip heat inop

Stall warning & AOA indexer lights inop

Handle (gear) & hook (downlock) activated

10. The left squat switch: [HHHAP] on ground

Heat for left engine lip boot inop

Heat for AOA sensor – LOW (14V)

Heat – GND MAX

Ambient air solenoid – CLOSED

Pressure control safety valve – OPEN

11. Down lock switches: close when strut fully extends.  When all 3 down lock switches activated (all 3 gear down & locked): [1] WHEELS UP light deactivates [2] Gear horn deactivates [3] 3 green down lights illum

12. The red light in the gear handle will illuminate when: [1] the gear is unlocked, in transit, or gear warning horn activated [2] both power levers retarded below 79%N1 when gear not down and locked [3] pressing the Landing Gear Handle Warning Light Test Button [4] Flaps below APPR & gear is not down and locked.

13. The warning horn silence button and the WHEELS UP light can be turned off in flight (with power retarded, gear up, and flaps up by pressing the WARN HORN SILENCE button.  After silencing the horn, it will remain silent until the flaps are extended or at least one power lever is advanced beyond 79%N1 and then pulled back below 79%N1.  Don’t silence the horn in the traffic pattern or during the final segment of an IAP when potential to land exists.  Only the PIC will cancel the horn.

14. During manual gear extension, the first green down lock indicator to illum will be the nosegear light.

15. The extension time of the gear is 4-6 seconds.  The retraction time of the gear is 5-7 seconds.

16. The prop reverse not ready light will not illum with the landing gear relay CB pulled (during manual emergency extension of gear).

-- The warning horn, wheels up light, and gear handle will all come on when all 3 landing gear struts are not down and locked and: [1] both power levers retarded < 79(+/-2)%N1  OR  [2]Flaps extended beyond approach position (Flaps – FULL)

-- The wheels up light & gear handle light illum when: [1] both power levers retarded < 79(+/-2)%N1 with Gear UP  OR  [2]Flaps extended beyond approach position with Gear UP.

17. It is possible to have a safe gear indication and not have safe gear (such as not having 3 complete tire or wheel assemblies).

18.  The gear are held down and locked by spring loaded locks.

19.  The gear are held up by the jackscrew in the actuator.

20. I can tell the difference between an electrical and mechanical malfunction on an unsafe gear by comparing visual indications with cockpit gear indications and gear circuit breaker positions.

Pull the following circuit breakers: [1] Landing gear relay CB [2] Landing gear indicator CB [3] Landing gear motor CB [4] Cockpit gear position indicators.  If during pulling any of the following , the gear indicates safely down and is visually confirmed safely down, land as soon as practicable.  ***Absence of illuminated AOA indexers could indicate premature landing gear motor cutoff which is indicative of an electrical problem.

__ LANDING GEAR SYSTEM MALFUNCTION & LANDING GEAR EMERGENCY EXTENSION

Refer to NATOPS 16-6 to 16-9

# LANDING WITH GEAR UNSAFE UP INDICATION

If one or more landing gear fail to retract or don’t indicate safe up position:

1. Gear handle – DOWN

2. Gear position – CHECK

3. If a safe down condition obtained and visually confirmed, LAND as soon as practicable.

Note: Before landing, visually check gear down by another aircraft or tower fly-by.

4. If SAFE gear down and locked indication NOT obtained or visual check indicates UNSAFE, proceed to Landing Gear Unsafe Down Indications.

# LANDING GEAR UNSAFE DOWN INDICATIONS

If one or more landing gear fail to extend or fail to indicate a safe condition:

Note: If during any of following steps the problem is isolated and visual check indicates safe gear down, LAND as soon as practicable.

1. Landing gear relay CB, landing gear indicator CB, landing gear motor CB – CHECK

2. Cockpit gear position indicators – CHECK

a. Press-to-test bulbs

b. If unsafe green (for down) light accompanied by

red light in gear handle, momentarily retard

power levers well below 79 (+/-2)% N1.  If no

wheels up lights or horn detected, all 3 gears

most probably in safe position.

c. AOA indexer lights – check if illum.  Absence of

indexers indicate premature landing gear motor

cutoff (electrical problem).  If all gear indicate

unsafe, consider manual extension.

d. For RT main unsafe – Prop Sync – ON.  If prop

sync light illum, right main NOT LOCKED.

e. For nosgear unsafe – Landing/Taxi Lights – ON.

Note gen load.  If gen load increases, nosegear

NOT LOCKED.

Note: Windshield hear, prop deice, and eng lip boot heat have lock out mechanisms that override electric heater.  Ensure these systems are secure before checking left main landing gear.

f. For left main unsafe – select manual heat and position electric heat switch to NORMAL.  Note load.  Then hold electric heat switch in GRD MAX.  If significant gen load increase, left main NOT LOCKED.

5. Gear position (visual) – CHECK

6. If visual check indicates gear down, LAND as soon as practicable.

7. If visual check confirms gear not down and locked, determine if malfunction electrical or mechanical.  If electrical, raise gear handle, attempt manual extension, and obtain visual check prior to landing.

Note: Attempt manual extension only if electrical malfunction.

8. Landing gear handle – SELECT UP

9. Gear position (visual) – CHECK

10. If visual check confirms gear retracted, gear up landing recommended.  If gear are NOT retracted, refer to landing with gear up or unsafe.  Consult home squadron by direct radio comm or by ATC/FSS relay.

Caution: Don’t taxi with unsafe gear indication!

# LANDING GEAR EMERGENCY EXTENSION

Note: If mechanical malfunction, don’t attempt manual gear extension!  Gear can only be lowered manually if electrical gear malfunction.  Gear can’t be raised mechanically.  Gear is easier to manually actuate at lower airspeeds.

Note: Prop reverse not ready light WON’T illum with landing gear relay CB pulled.  Landing gear motor switches may not activate; therefore, AOA indicator light may not be available.

To emergency extend gear:

1. Airspeed – 120KIAS RECOMMENDED (155KIAS max)

2. Autopilot – AS REQUIRED

3. Landing gear relay ‘LDG GEAR’ CB – PULL

4. Landing Gear Handle – DOWN

5. Clutch Disengage lever – LIFT & TURN CLOCKWISE

6. Manual Extension Handle – PUMP until 3 green gear indicator lights illum (approx 50 strokes to fully extend)

Note: Reduce handle stroke length when nosegear indicates safe.  DON’T pump handle after 3 gear down lights illum.  DON’T stow handle, move gear controls, switches, or CBs until on ground and malfunction found.

LANDING WITH ONE MAIN GEAR UP OR UNSAFE

Mechanical failure in drive mechanism.  Land on runway or hard surface.  Touch down well to same side of runway as extended gear.  Roll on the down and locked gear, holding opposite wing up and nose straight as long as possible.  As wing tip strikes ground, apply opposite brake to maintain straight path.  Remove field arresting cables.  If night emergency egress, pre-position threshold, spar, and aft compartment lighting ON.

Before touchdown:

1. Crew and passengers – EMERGENCY BRIEF

2. Seat belts and harnesses – SECURE (passengers assume braced position)

3. Landing checklist – COMPLETE

After touchdown:

4. Power levers – IDLE

5. Condition Levers – FUEL CUTOFF

6. Use ailerons and rudder to maintain directional control and keep wings level as long as possible

7. Brakes – AS REQUIRED

8. Firewall Valves – CLOSED

9. Boost Pumps – OFF

10 . Fire extinguisher  -- AS REQUIRED

11. Gang Bar – OFF

12. Evacuate aircraft

LANDING WITH MAIN GEAR DOWN, NOSE GEAR UP OR UNSAFE

If all attempts to lock nosegear fail, retract main gear, complete gear-up landing procedure, and execute gear-up landing.  If main gear can’t be retracted, make a normal approach with power setting using minimum wing flap to hold nose up as much as possible after touchdown.  DON’T USE BRAKES!  Remove field arresting cables.  If night emergency egress, pre-position threshold, spar, and aft compartment lighting ON.

Before touchdown:

1. Crew and passengers – EMERGENCY BRIEF

2. Seat belts and harnesses – SECURE (passengers assume braced position)

3. Landing checklist – COMPLETE

After touchdown:

4. Power levers – IDLE

5. Condition Levers – FUEL CUTOFF

6. Firewall Valves – CLOSED

7. Boost Pumps – OFF

8 . Fire extinguisher  -- AS REQUIRED

9. Gang Bar – OFF

10. Evacuate aircraft

GEAR UP LANDING

Main landing gear wheels protrude slightly from wheelwell in gear-up position and will roll on landing.  Keep wings level.  Land with approach flaps on a hard surface runway (paved preferred).  Touch down at 85KIAS.  If night emergency egress, pre-position threshold, spar, and aft compartment lighting ON.

Before touchdown:

1. Crew and passengers – EMERGENCY BRIEF

2. Seat belts and harnesses – SECURE (passengers assume braced position)

3. Clutch disengage handle – LIFT & TURN COUNTERCLOCKWISE

4. Manual extension handle – STOWED

5. Landing gear relay ‘LDG GR’ – PULLED

6. Flaps – APPROACH

Note: Tendancy to overshoot due to decreased drag.  If conditions permit, gang bar – OFF prior to touchdown.

Immediately prior to touchdown:

7. Condition levers – FUEL CUTOFF

After touchdown:

8. Power levers – IDLE

9. Condition levers – FUEL CUTOFF (only if power utilized to point of touchdown)

10. Brakes – AS REQUIRED (directional control maintained using brakes and rudder)

11. Firewall Valves – CLOSED

12. Boost Pumps – OFF

13. Fire extinguisher  -- AS REQUIRED

14 Gang Bar – OFF

15. Evacuate aircraft

ENGINE MALFUNCTION ON START:

ABNORMAL START (for HOT START & NO LIGHT OFF)

If ITT likely to exceed 925C or no rise in ITT within 10 sec after condition levers LOW IDLE:

1. Condition Lever – FUEL CUTOFF (ITT < 790C)

2. Starter – OFF

Caution: Starter time limited to 40 sec ON, 60 sec OFF, 40 sec ON, 60 sec OFF, 40 sec ON, 30 minutes OFF.

Note: If start attempt discontinued and another start attempted, allow 60 sec delay to drain fuel and cool starter, then motor up starter for 15 sec minimum.  Allow eng to completely stop before attempting another start.

# EMERGENCY ENGINE SHUTDOWN ON DECK (for FIRE, FIRE WARN LIGHT, CHIP LIGHT)

*** Set the parking brake.

1. Condition Levers – FUEL CUTOFF

2. Firewall Valves – CLOSED

3. Boost Pumps – OFF

4. Fire Extinguisher – AS REQUIRED

*** Make a radio call to alert ground control or tower of shutdown prior to turning off gang bar.

5. Gang Bar – OFF

6. Evacuate aircraft

# FLIGHT DIRECTOR/ AUTOPILOT USAGE (FTI 4-64 to 4-65 & NATOPS 20-29 to 20-40 )

FTI: Flight director (FD) can be used independently or coupled with autopilot (AP).  FD used alone, fly with command bars as guidance.  AP can be used with or without FD.  AP alone, used manual pitch wheel and roll knob.  FD & AP coupled, AP controls aircraft by commands from the FD.  Touch control steering can be used anytime AP is engaged.  Power levers must be adjusted manually.  Disengage AP by pressing the AP/YD yoke switch (1st detent) or the go around button on the power levers.  Min altitude for AP use in 200’AGL.  Confirm all annunc lights are illum when using the AP/FD.  To use AP, P must be seated at the controls with seatbelt fastened.  AP will roll up to 30 degrees AOB.  When AP is coupled with FD, select HDG (with bug on nose) prior to changing NAVAID frequencies.  When flying a coupled approach, advise ATC approach controller no later than FAF so the ILS critical area can be cleared or an advisory issued, such as “localizer/glideslope signal not protected.”  In this case, be alert for unstable ILS.  Boundary for ILS critical area is a double runged ladder marking with a sign saying ‘ILS’ in white on a red background.  Used as runway holding position when ceiling < 800’ and/or vis < 2 miles or when directed by ATC.

AUTOPILOT DISENGAGEMENT (NATOPS 15-12 & 20-40)

Any interruption or failure of power, vertical gyro failure, activation of vertical gyro fast errect, flight control system power or circuit failure, activation of electric elevator trim, or autopilot trim failure may cause the autopilot to automatically disengage.  The autopilot can be intentionally disengaged by the following: [1] actuation of AP/YD disconnect switch (either control wheel) [2] autopilot engage lever moved to ‘DIS’ position [3] actuation of go-around button (left power lever; yaw damp remains on) [4] pulling flight director/ autopilot circuit breaker (turning off BATT/GENS (gang bar) or avionics master switch.  If an engine fails, disengage autopilot, retrim aircraft, reengage autopilot.  If used in conjunction with an instrument approach, maintain 120KIAS for single-engine approach speed until landing assured.