RI-7 Study Guide (15 Jun 02)

__ IFR COMM (KNEEBOARD)

__ IFR BRIEFS (APPX. A & AIGT CHAP. 8)

__ LOCAL DEPARTURE INSTRUCTIONS (BLUE BRAINS)

__ CHECKLIST PROCEDURES (PARTICULARLY ENGINE RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (CONDENSED NATOPS)

__ COURSE RULES GUIDE (DEPARTURES & ARRIVALS TO NAVY CORPUS)

__ EMERGENCY PROCEDURES

ENROUTE WEATHER FACILITIES: (AIM 7-1-5 to 7-1-12/ FIH Section C)

[1] En Route Flight Advisory Service [EFAS] “Flight Watch”: live, use common freq 122.0 from 5,000 AGL – 17,500 MSL.  Discrete freq for 18,000 – 45,000 MSL.  Contact ‘flight watch’ by using name of local ARTCC, aircraft identification, and nearest VOR.  List of flight watches on enroute charts and in Enroute Supp.

[2] Transcribed Weather Broadcast (TWEB): taped, broadcast continuously over navaids and/or VORs.  Contains forecasts, Inflight Advisories, winds aloft, NOTAMS, METAR/TAFs.

[3] Hazardous InFlight Weather Advisory Service (HIWAS): taped, continuous broadcast of inflight hazardous weather.  Listed on Enroute Charts with an “H” in navaid boxes, VFR sectionals, and in Enroute Supp.

[4] Automated Weather Observing System (AWOS): taped, automated.  Visibility averaged for last 10 minutes.  Ceilings averaged from last 30 minutes.  Transmitted over discrete VHF or local navaid.  Receivable max 25nm from site and max altitude of 10,000’ AGL.  Updated each minute.  No two way comm..  Most have telephone access.  Freqs published in Enroute Supp and on IAPs.

[5] Automated Surface Observing System (ASOS): taped, automated.  Primary surface observation system in U.S.  Continuous minute-by-minute observations.  Generate METARs.  Transmitted over discrete VHF freq or at local navaid.  Receivable max 25nm from site and max altitude of 10,000’ AGL.

[6] Flight Service Station (FSS): live, freq listed on chart or in Enroute Supp.  Give METAR/TAFs for requested airport.

[7] Pilot-to-Metro Service (PMSV) “Metro”: Live, direct contact with military observers or forecasters.  Observers only give current wx info.  Forecasters can give wx, update DD175-1, and receive PIREPS.  Freqs listed in Section C of FIH.

[8] Automated Terminal Information System (ATIS): taped, broadcast over navaids or ATIS freq.  Give pertinent weather, airfield, and runway information for aerodrome ATIS services.  Freqs listed on charts, in enroute supp, and on IAPs.

[9] VOLMET Voice Weather Broadcasts: taped, continuous weather information for flights overseas.  Given over VHF and HF freqs which are listed in Section C of the FIH.

[10] Air Route Traffic Coordination Center (ARTCC): live by air traffic controllers.  Given on work load basis.  Normally only for hazardous weather.  Also give TAFs/METARS.

VOR (& TACAN) NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

Utilize either radar vectors to final or execute a procedure turn.  Compass card is frozen but radial/bearing info from needles is still correct.  1st, determine current radial (cross reference with DME).  2nd, take radial & DME and figure out current position on an IAP or chart.  If required, make a timed turn to the VOR inbound course and fly direct to the station (using the VOR needles & the CDI).  At station passage, make a timed turn to the outbound intercept heading, rotate the course select knob to the Final Approach Course & track outbound using heading info from the copilot.  Reverse course (45/180) using time turns: 15 sec for 45 degrees & 1 minute for 180 degrees.  When CDI starts to center, commence another 15 second turn to final.  To track inbound, either visualize with radials/DME or set CDI and heading bug.  Configure at normal point.

SSE NON-PRECISION (VOR) APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

CIRCLING: 120K for dual and single engine.  Circling MDA and wx mins are for runway approach is flown to.  Circling mins are for non-radar non-precision approaches.  Don’t circle from a precision approach.  Report “commencing circle” when initiating any circling maneuver.  T-44 is Category B (approach speeds 91-120K).  If need to maneuver at airspeeds >120K, use Category C mins.  When requesting an ASR circling approach, state aircraft category (B) to controller.  When circling, keep runway environment in sight.  Circle in what ever direction is the shortest to reach landing runway, unless directed otherwise by ATC, IAP, or Enroute Supp.  SSE: If able, fly circling approach above circling MDA (preferably at VFR pattern altitude of 800’).  In training, we fly at circling MDA.  Don’t put Flaps-APPR & Gear Down Landing Checklist until at 180 pos (latest is 90) and in safe position to land.  Circling Obstruction Clearance Area for Category B aircraft is 1.5nm.  If engine fails after configuring, raise flaps and gear immediately.  Don’t put them down until landing assured.  Use 60 rule to offset from runway (if landing on opposite side of runway approach was flown to)– turn 45 degrees offset and fly outbound for 15 seconds.  Missed approach instructions are for runway approach was flown to (not the runway landing)!  Must be familiar with missed approach procedures to know what direction to turn!

HEAT RISE: The convective nature of radiation heating causes heat to rise from the ground.  This affects aircraft performance in a number of ways.  Rise heat increases updrafts and the risk of turbulence.  Hotter than standard days effects performance (measured as density altitude) at all altitudes (less power, slower, longer takeoff roll).  Greater heat rise occurs from plowed fields and manmade objects (cities/buildings).  Earth tends to radiate heat more readily than water.  Therefore, more updrafts/turbulence will be found over ground than water.

ANTI-ICE/DEICE SYSTEM & EMERGENCIES (NATOPS 2-50 to 2-53, 15-11)

Windshield Wipers: Don’t operate on dry glass.

Windshield anti-ice: Temp sensing unit maintains 95F at windshield surface.  Electric heater lockout.  Mag compass will be unreliable when windshield anti-ice in use.  May blur objects.

Powerplant Ice Protection: Mechanically operated ice vanes or electrical lip boot heat.  With ice vanes extended slight decr in engine torque.

Fuel System Anti-icing: Oil-to-fuel heat exchanger; operates continuously; no controls; one external fuel vent in each wing for both nacelle and wing tanks (protected from icing by electric heat elements).

Fuel Control Heat: Each FCU pneumatic line protected from ice by electrically heated jackets.  To prevent overheat, FUEL CONTROL HEAT should not be turned on unless cooling air will soon pass over the jackets.

Propeller Electrothermal Deicer System: Deicing boots on each prop blade; outboard and inboard element that receive electrical power through the deicer timer, slip ring, and brush block.; current measured by the prop ammeter.  Sequence: RH OUT, RH IN, LH OUT, LH IN (RORI LOLI); each segment heats for 34+/-6 sec; req 2 minutes for entire cycle; old mechanical timers sequence to next segment (show amperage fluctuation at timer switches to next segment); electrical timers always reset to RH OUT when power applied (no flux in amperage); each blade draws 5 to 6 amps (3 blades working correctly read between 14 to 18 amps).

Surface Deicer System: Ice removed from outboard wing leading edge, horizontal stab, and vertical stab by flexing pneumatic deicer boots.  Engine bleed air from the engine compressor supplies press to inflate boots and vacuum to hold down boots; press regulator protects system from overinflation;  when not operating, a distributor valve applies vacuum to hold boots down; when the DEICE CYCLE switch is toggled to SINGLE or MANUAL, a servo valve changes the inlet boots from vacuum to pressure that allows the boots to inflate; when the solenoid is deenergized, airflow through the valve is cutoff and it discharges out the boots through the intregal check valve until press reaches approx 1-inch Hg, at which time the boots port to vacuum and the remaining air is evacuated; the boots are again held down by vacuum; either engine can provide the bleed air for the system; bleed air passes through an 18-psi regulator and then enters the deicer and vacuum systems; vacuum press is created by the distribution valve and is proportional to pneumatic pressure by the deicer press regulator valve.  Stall speeds significantly increase with the wing deice boots inflated; never cycle the system rapidly, may cause ice to accumulate outside the contour of the inflated boots and prevent ice removal.  In SINGLE, boot inflate for 7 to 8 seconds, then deflate.  In MANUAL, all boots inflate and stay inflated until switch is released.  The boots can’t be overinflated.

Pitot & AOA Heat Systems: On the ground, AOA heating elements pass through a current limiter lowering voltage to 14 volts DC (low heat).  At lift off , a squat switch on the left main gear causes the current limiter to be bypassed and heating elements receive 28 Volts DC (high heat); except for takeoff roll, PITOT heat should not be used on the ground (might overheat because of lack of cooling airflow over heating elements).

Surface Deice Emergency Procedure: If boots fail to inflate, operate in MANUAL.  If boots fail to deflate, pull the SUR DE-ICE CB and reset as necessary to operate boots.

RADIO INSTRUMENT MALFUNCTION: (FTI 4-49) (CDI failure for VOR/TACAN): Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.

# FUEL SYSTEM EMERGENCIES (NATOPS 15-4, 15-6 to 15-7)

1. NO FUEL TRANSFER LIGHT

[1] Check total and nacelle fuel quantity

If no fuel in wing tanks:

[2] Transfer pump – OFF

If fuel in wing tanks:

[3] Transfer pump – OVERRIDE

If light remains on:

[4] Transfer pump – OFF (lose 28 Gallons)

[5] Land as soon as practicable

2. CROSSFEED light illumination: illuminates when boost pump fails or engine fails.  If boost pump failure: [1] close crossfeed switch.  If boost pump failed: [1] Failed boost pump – OFF [2] crossfeed – OPEN [3] Land as soon as practicable.  If not a boost pump failure, reset the crossfeed valve to ‘AUTO.’  If engine failure, accomplish EMERGENCY ENGINE SHUTDOWN CHECKLIST.

3. Failure of the engine driven fuel pump – Engine WILL FLAMEOUT.  Accomplish EMERGENCY ENG SHUTDOWN CHECKLIST.

4.  If fuel leak – consider securing electric systems which may contribute to wing fire.  Secure nav lights, strobe lights, fuel vent heaters, and AOA.  Secure inboard wing items with gang bar.  If destination can be reached without dual engines, consider shutting down engine on side of fuel leak.  Secure engine as follows:

[1] Condition Lever – FUEL CUTOFF
[2] Emergency Shutdown Checklist – EXECUTE

5. If fuel syphoning – [1] Airspeed – 140K [2] Land as soon as practicable.  Avoid extreme nose low attitudes.

6. Single engine crossfeed:

[1] Boost pumps – ON

[2] Transfer pumps – AUTO

[3] Crossfeed – OPEN (check crossfeed light on)

[4] Boost pump (nonfeeding tank, tank w/o FUEL PRESS light ON) – OFF

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.