RI-12S Study Guide (23 Jun 02)

NAVAID CHARACTERISTICS:

(AIM 1-1-1) NDB = Non-Directional Beacon. It is the station that sends the signal. ADF = Automated Direction Finder. It’s the radio receiver in the airplane that picks up the NDB signal and converts it into bearing information. low or medium freq radio beacon, nondirectional, gives bearings, “home” to the station, freq 190-535 kHz, transmits continuous carrier with either 400 or 1020 Hz modulation. All NDBs except compass locators, transmit a 3-letter ID code, except during voice transmissions. An NDB coupled to ILS markers are a Compass Locator. Voice transmissions are made on all NDBs unless the letter “W” (without voice) is included in the class designator (HW). Subject to disturbances that may cause erroneous bearing information, such as lightning, precipitation, static, etc. . . At night, NDBs are vulnerable to interference from distant stations. Noisy identification usually occurs when the ADF needle is erratic. ADF receivers don’t have flags. The only way to tell if the signal is correct is to CONTINUOUSLY MONITOR the station!!!

(AIGT 1-12) The only information an NDB provides is direction to the station. The LF/MF NDB freq = 190 to 535 KHz. UHF NDB freq = 275 to 287 MHz. Accuracy is = +/- 5 degrees. Pilot must continuously monitor the NDB. Station passage is when the pointer moves through the wingtip position. Due to dip error the ADF bearing pointer gives accurate bearings only in wings-level flight. NDB station generates a NON-DIRECTIONAL nav signal. Compass locators are normally installed with marker beacons. When a compass locator is co-located with a marker beacon, it is a “Locator Marker.” Compass locators freq = 200 – 415 kHz with a reliable range to 15nm, transmit 2-letter identification groups. The outer locator transmits the first 2 letters of the localizer ident group, and the middle locator transmits the last two letters of the localizer identification group. Beacons with code ‘SAB’ indicate continuous weather information. UHF ADF may also be used to provide auxiliary emergency voice reception if the normal radio receiver should fail.

(AUGT 2-28) The T-44 uses the Collins ADF-60. Reception range is based upon power output of the transmitting station and atmospheric conditions (it isn’t line-of-sight). The ADF VOLUME switch is the ON/OFF switch for the ADF! The “R” Range filter filters out all 1020 Hz tone information so that only voice information is heard. The “V”filter only works in conjunction with the ADF. It is used to filter out voice audio signal so that only the ID signal can be heard and continuously monitored. If both switches are on, they cancel each other out. The ADF is only tuned through the NCS-31. ADF information is displayed on the SINGLE needle of the RMI (no ADF information is available on the HSI). On the MSU, there are 3 buttons under “ADF.” The TONE button will cause the received AD carrier signal to be modulated with an audible 1000 Hz tone (the normal position for this button is out). The TEST button causes the single RMI needle, when switched to ADF, to rotate counterclockwise 90 degrees if valid bearing information is received. The ADF button is sort of an ON/OFF switch for the ADF needle (normal position is IN). The ADF signal must be continuosly monitored when used as a navaid (delegate this responsibility to the observer or the copilot). When the ADF receives a weak signal or no signal, the ADF needle will park in a horizontal position (9 to 3 position). The number under the needle’s head is the “course to” the station. The number under the tail is called the “bearing from” the station. If the compass card fails, the ADF needle will continue to point towards the direction of the station, relative to the nose of the aircraft, regardless of the compass card position. To head to the station, put the head of the needle at the top of the RMI card. No-turned headings must be timed, the needle takes a moment to settle down after a turn.

(AIGT 2-30) The T-44 uses the ARA-50 direction finder, in conjunction with teh ARC-159 UHF transceiver, for UHF ADF. Allows the aircraft to home in on UHF NDB stations or other aircraft transmitting on the UHF frequency (very helpful for search and rescue). The double needle will display UHF NDB information in the ‘DF’ position. It homes to the STRONGEST signal on the frequency selected. To make it work: tune the frequency on the UHF radio, select “ADF” on the UHF radio, switch the double needle on the RMI to “DF.” Continuously monitor the aural signal. Azimuth information is relative to the aircraft.

(FTI 4-18) Setting up the ADF: TIMSS [1] Tune the ADF freq in the NCS-31 [2] Identify the station using the ADF switch to get an aural tone (2 or 3 letter Morse Code) [3] Monitor the tone continuously while using the ADF [4] Select FD switch – N/A for the ADF (can only read ADF of the RMI needles) [5] Set the single needle to “ADF” if using LF/MF freq, or set the double needle to “DF” using UHF freq.

NDB COURSE INTERCEPTS/TRACKING: (RMI only): (FTI 4-19) Inbound: Course to Bearing + 30 = Intercept Bearing. Outbound: Tail to Course + 45 = Intercept Bearing. Station Passage: bearing pointer passes to the inbound course. In NDB holding, station passage is the first definite move by the bearing pointer through the 45 degree index on the RMI. Radial to Arc – lead by 0.8 nm for 90 degree turn at 150K. Arc to Radial – lead a 90 degree turn by 5 radials at 10 DME at 150K.

SHUTTLE APPROACH: (FTI 4-38) Used in holding to maneuver an aircraft to a lower altitude to commence a low approach. It is only initiate by ATC, not the pilot! If several aircraft are holding at the same fix, they will be ‘stacked’ with altitude separation (normally 1000’). Shuttles will descend each aircraft to the next lower stack altitude after the bottom stack aircraft has commenced its approach.

ADF HOLDING:  When ATC issues holding clearance, the direction they give points TO the holding fix (exp.  hold west of the NGP VORTAC means the arrow of the inbound course is FROM the west).  Always get an Expected Further Clearance Time (EFC), it determines course of action in a lost comm situation.  Standard holding means all turns to right.  Non-standard holding means all turns to left.  Make all turns either standard rate or max of 30 deg AOB.  If must tack 10 deg into wind on inbound leg, must tack 30 deg (3 times as large a correction) in opposite direction on outbound leg.  If extended holding, consult NATOPS for max endurance speed.  Request extended legs if delay exceeds 20 minutes.  ATC should issue holding instructions 5 minutes prior to reaching clearance limit fix.  Always request an EFC prior to accepting a holding clearance if one was not given.

Timing:  Begin outbound timing when abeam fix.  If abeam position can’t be determined, start timing when turn outbound is complete.  < 14,000’ MSL, time 1 minute. > 14,000’ MSL, time for 1 ½ minutes.  Commence inbound timing at wings level.

Speed: Recommended T-44 holding speed is 150KIAS.  Max holding speed is 175KIAS for turboprop aircraft.  Expect ATC holding clearance 5 minutes prior to arriving at holding fix.  Start a speed reduction to 150K (holding airspeed) 3 minutes or less from the holding fix.

Holding Entry:  Reaching the fix, execute the 6 T’s: Time, Turn, Time, Transition, Twist, Talk

Time: Note the time of station passage (NDB: needle passes 45 degree benchmark on RMI, cleared to turn)

Turn: Parallel, Teardrop, or Direct: determined using the outbound course on the RMI card.  Use pencil method.  If standard, put pencil on 90deg mark on RMI with tip pointed to right & turn 20 deg UP.  Relative position of 360 to 070 = Teardrop.  Relative position 360 to 250 = parallel.  All headings below the pencil are direct.  If nonstandard, put pencil on 90 deg mark on RMI with tip pointed to left & turn 20 deg UP.  Relative position 290 to 360 = teardrop.  Relative position from 360 to 110 = parallel.  All headings below the pencil are direct.

Parallel:  maneuver to outbound course (non-holding side) for one minute, then turn toward holding

pattern inbound course.

Teardrop: Subtract 30 deg from outbound course for standard & add 30 deg for nonstandard.

Direct: Turn in direction of outbound course.

Time: Time first leg outbound (1 min or 1 ½ min)

Transition: slow to 150KIAS

Twist: put in inbound holding course

Talk: Tell ATC established in holding

ADF NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

The ADF needle shows relative bearing only.  Mentally place the current heading (given by the CP from the mag compass) under the upper lubber line.  Count the number of degrees the head of the single needle deflected from the lubber line to find the relative bearing to the station.  Add (+ to the right) or subtract (- to the left) from the current mag heading to get the magnetic bearing to the station.  Mag bearing from the station found by using the tail of the needle when heading away from the station.  (Exp. If the current heading is 135, and the needle points 45 degrees to the left of the lubber line, subtract (because the needle is left) 45 from 135 to get a magnetic bearing of 090 to the station.  Unless radar vectors are given, request the 45/180 course reversal.  Turn to place the head of the needle on the nose and fly direct to the station.  Utilized known winds to make a wind-corrected heading to the station.  Continue to ask the CP for heading calls and set the heading bug on the corresponding heading to proceed direct.  Compute a heading to intercept the outbound course (the outbound course + 30 degrees [extra 10 seconds]) and determine the seconds required for a timed turn.  Immediately after station passage, commence a timed turn to intercept the outbound course.  Track outbound 1 minute using heading calls from the CP, then start a 45 degree [15 second] turn to the outbound PT heading.  After 1 minute, commence a 180 degree [1 minute] course reversal.  Inbound on the procedure turn, determine my position for situational awareness.  When the needle reaches the 45 degree benchmark, turn inbound so the head of the needle is at the top of the RMI.  Continue to ask the CP for headings.  Configure at the normal point for a non-precision approach.  Passing over the IAF and the FAF, execute the 6Ts (time, turn, time, transition, twist, talk).  If missed approach, generally requires turning to the head of the needle and flying direct back to the station.

ADF APPROACH (WITH PROCEDURE TURN):  Procedure Turns (PT): 3 acceptable methods: [1] 45/180 Technique [2] Holding Technique [3] Teardrop Technique

[1] 45/180 Technique:  Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 bearings for ADF).  Continue descent to FAF.  Configure and slow to 120K prior to the FAF (unless SSE).  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

[2] Holding Technique:  If heading within 90 degrees of outbound PT Course, use normal lead points (includes all of teardrop entry and most of parallel entry).  If use teardrop entry, course must be within 30 degrees of PT outbound course.  If intercept the PT outbound course, maintain course for remainder of outbound leg, then turn toward the maneuvering side to reverse course.

Timing: begin timing outbound abeam procedure turn fix.  If can’t determine abeam, start timing at completion of outbound turn.  Stay within the remain within distance.  At the completion of the outbound leg, turn to intercept the procedure turn course inbound.  A 1 ½ minute outbound leg normally is sufficient (2 minutes or more can be used in instances such as an approach with no FAF or an approach with a large descent on the inbound course).

Descent: Don’t descend form fix altitude until abeam PT fix outbound.  Don’t descend from completion turn altitude until established on inbound segment.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

RADIO INSTRUMENT MALFUNCTION: (FTI 4-49) CDI failure: Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.  DME failure:  Don’t shoot approach that requires DME.  Request a non-DME approach (PAR, ASR, ILS, LOC, etc...)  If inside FAF and lose DME (timing NOT available), execute a missed approach unless field is in sight.

# ENGINE SYSTEM MALFUNCTIONS

# [1] OIL SYSTEM FAILURE

Oil press < 85psi undesirable – tolerated only to complete flight. Monitor engine instruments and nacelles for secondary indications.  Consider shutting down engine and landing as soon as POSSIBLE; otherwise, reduce power on engine and landing as soon as PRACTICABLE.  Oil pressure < 40psi & oil temp >99C unsafe requiring engine shutdown or landing made as soon as possible using min power to sustain flight.  Note both discrepancies on maintenance forms.

# [2] CHIP DETECTOR LIGHT ILLUMINATION

Indicates metal particles in prop reduction gearbox.  Perform EMERGENCY SHUTDOWN CHECKLIST.

# [3] EMERGENCY ENGINE SHUTDOWN ON DECK (for FIRE, FIRE WARN LIGHT, CHIP LIGHT)

1. Condition Levers – FUEL CUTOFF

2. Firewall Valves – CLOSED

3. Boost Pumps – OFF

4. Fire Extinguisher – AS REQUIRED

5. Gang Bar – OFF

6. Evacuate aircraft

[4] ABNORMAL START (for HOT START & NO LIGHT OFF)

If ITT likely to exceed 925C or no rise in ITT within 10 sec after condition levers LOW IDLE:

1. Condition Lever – FUEL CUTOFF (ITT < 790C)

2. Starter – OFF

Caution: Starter time limited to 40 sec ON, 60 sec OFF, 40 sec ON, 60 sec OFF, 40 sec ON, 30 minutes OFF.

Note: If start attempt discontinued and another start attempted, allow 60 sec delay to drain fuel and cool starter, then motor up starter for 15 sec minimum.  Allow eng to completely stop before attempting another start.

# [5] ABORT & ENG FAILURE DURING TAKEOFF

1. Announce -- “ABORT” (Initiate < 91KIAS)

2. Power Levers – IDLE

3. Reverse – AS REQUIRED (then beta range)

4. Brakes – AS REQUIRED (smooth, even, don’t lock)

Prior to departing prepared surface:

(verbally recite remaining items)

5. Condition Levers – FUEL CUTOFF

As soon as practicable:

6. Firewall valves – CLOSED

7. Boost Pumps – OFF

8. Fire Extinguishers – AS REQUIRED

9. Gang bar – OFF

10. Evacuate aircraft

Warning: If direction control problems occur while reversing, advance both power levers to FLIGHT IDLE.  Maintain directional control with rudder, nosewheel steering, and by pumping brakes.

Warning: Accelerate-stop distances increased 900 ft with condition levers HIGH IDLE and no reverse utilized.

# [6] ENGINE FAILURE AFTER TAKEOFF

If sufficient runway to stop on ground, ABORT!

If insufficient runway to abort takeoff:

(POWER UP, RUDDER UP, CLEAN UP)

1. Power – AS REQUIRED

2. Gear – UP

3. Airspeed – (102 (obstacles) or 110)

4. EMERGENCY SHUTDOWN CHECKLIST

[1] Power Lever – IDLE

[2] Propeller – FEATHER

[3] Condition Lever – FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve – CLOSED

[5] Fire Extinguisher – AS REQUIRED

[6] Bleed Air – CLOSED

[7] Dead Engine Checklist – AS REQUIRED

Note: address first 2 items of dead engine checklist, transfer comm to CP and have him declare an emergency.

Note: Dynamic Engine Cut complete when trimmed at 110KIAS (min 102KIAS), on takeoff heading, and Emergency Shutdown Checklist complete.

## [7] INFLIGHT EMERGENCIES

Note: 1. Rudder trim sufficient above 100KIAS.  Below 100KIAS, supplement full rudder trim with constant rudder pressure.

2. SSE waveoff with full flaps, result in loss of 200 feet before positive rate of climb established.

3. Impending engine failure or flameout preceded by:

[1] unstable engine operation

[2] fluctuating turbine RPM

[3] torque and ITT temp changes

[4] illumination of fuel warning lights

[5] drop in oil pressure

[6] loss of thrust

4. DON’T attempt an airstart if: [MOVEOFF]

Mechanical malfunction

Overheating

Vibration

Explosion

O N1

Fire

Fuel Fumes

SSE ADF APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.