RI-24 & 25 Study Guide (15 Jul 02)

__ IFR BRIEF (BE HRC)

__ RUNWAY LENGTH REQUIREMENTS

__ LOST COMM (LOCAL & FIH: IFR & VFR)

__ CHECKLIST PROCEDURES (PARTICULARLY RADIO/NAVAIDS & ENGINE RUN-UP)

__ OPERATING LIMITS (NATOPS (C.4),T-44 IMPORTANT NUMBERS, & CONDENSED NATOPS)

__ CODED DEPARTURES

__ SYSTEM OF THE DAY (PLUS AN ADDITIONAL SYSTEM IF TIME PERMITS)

__ EMERGENCY PROCEDURES (NATOPS: GROUND, TAKEOFF, IN-FLIGHT, LANDING)

__ AIGT (HOLDING, TERMINAL PROCEDURES, IAPs)

__ FLIGHT SIMULATOR 2002 (LOCAL APPROACHES (NORMAL & NO HEADING))

FORCED LANDING: (NATOPS 16-10)  If suitable altitude, perform a circular pattern to observe surface terrain and wind conditions.  After terrain and wind noted, set up normal pattern and extend gear when landing assured.  Plan to overshoot.  Use flaps as needed.  If positive gear down and locked indication can’t be determined, do a gear up landing.  Night egress: preposition threshold, spar and aft compartment lighting ON.

1. Configuration:  [A] Don’t extend full flaps until landing assured [B] Gear up landing based on type of touchdown surface

2. Landing Area: Adequate size to land, free of obstacles, smooth.  Cultivated fields are best.  Swamps, bogs, shallow lakes, and forests should be avoided. Land into wind.

3. Landing: Make at slowest speed that retains complete control.  If landing into trees, fly into them; don’t attempt to land on the tops.  (Refer to NATOPS Figure 16-5 on page 16-12 for Emergency Landing Pattern illustration!)

WAKE TURBULENCE: (AIM 7-3-1 to 7-3-7)  Wake turbulence is caused by counter rotating vortices trailing from wingtips.  Usually only a problem from large aircraft.  Greatest strength when generating aircraft is HEAVY, CLEAN, & SLOW!  Vortices are generated whenever aircraft is in the air (moment it leaves the ground).  Vortices sink at several hundred feet per minute.  In flight, avoid area behind and below.  1000’ separation in the air is considered safe.  Vortices tend to sink and move laterally out from the flight path when near the ground and no wind.  With 1K- 5K crosswind, the upwind vortice tends to remain in the touchdown area and the downwind vortice tends to drift at twice the crosswind speed with the wind (possibly to a parallel runway).  The most dangerous is a quartering tailwind (remains on the runway).

1. Landing behind larger aircraft (same runway) – stay above flight path and land beyond touchdown point.

2. Landing behind larger aircraft (crossing runway) – stay above flight path.

3. Landing behind departing larger aircraft (same runway) – land well prior to its rotation point.

4. Landing behind departing larger aircraft (crossing runway) – stay above flight path.

5. Departing behind larger aircraft – rotate well prior to its rotation point and stay above its flight path.

6. Departing or landing behind larger aircraft executing a missed approach, low approach, or touch and go – insure minimum of 2 minute interval before takeoff or landing.

7. En route flight behind larger aircraft –  avoid flying below and behind (adjust position laterally, preferably upwind).

When landing behind larger aircraft fly on or above glide path.  If visual approach initiated with preceding larger aircraft in sight, pick a visual point to land no further than 1000’ from approach end of runway and establish a line of sight to that landing point which is above and in front of preceding larger aircraft.  Land BEYOND the point of landing of the preceding larger aircraft.  2 minutes or 4 to 5 mile radar separation when taking off behind a B757/heavy.  3 minute interval given when taking off from an intersection behind a departing large aircraft (when preceding aircraft is a B757/heavy and ops on either same or parallel runways are separated by less than 2,500’).  Pilots can request additional separation i/e 2 minutes instead of 4 or 5 miles for wake turbulence separation.  This request must be made as soon as practicable on ground control and at least before taxiing onto the runway.

ARRIVAL TRANSITION: Prior to arriving in terminal area, do the ABC check.  [A] ATIS and Wx [B] Brief Approach [C] Checklists/Charts/Clearance for approach.  Checking in with approach in terminal area, inform them of the current ATIS letter, destination, desired approach, how it will terminate (full stop, touch and go, option, low approach, etc...). and/or the next approach.

FUEL PLANNING: (FTI 4-7)  Use a fuel log for all cross country flights.  If alternate not required, must have fuel to fly to destination, plus a 10% reserve (max endurance at 10,000’).  If alternate required, must have fuel to fly to destination IAF and then to alternate airfield, plus a 10% reserve.  In no case shall reserve be less than 20 minutes.  Minimum on-deck fuel is 265 lbs per side.  Minimum fuel is advisory only (emergency could occur if any undue delay occur).  Min fuel is normally 265 lbs per side (yellow arc).  If min fuel, consider requesting short vectors to final if IMC or visual landing.  If fuel state requires traffic emergency, declare an emergency due to low fuel and report fuel remaining in minutes.  Emergency fuel typically called when 30 minutes of fuel remain (no set amount, can vary).

SSE EN ROUTE: Deal with emergency, have copilot take comms, declare an emergency, squawk 7600, run the dead engine checklist.  Land as soon as possible at the nearest suitable airfield.  Pressurization requires (72% N1 dual engine & 85% N1 for single engine).  Check minimum altitudes.  Fuel burn will increase.  Speed will decrease.  Climb rate will decrease.

FLIGHT PLANNING: (FTI 4-4)  Check weather, NOTAMs, fuel requirements, terminal approaches, alternates, anticipated traffic delays.  Tasks at Base Ops [PWANTS]:  Publications, Weather, Activate flight plan, TOLD data, SIDS/DPs/STARs/IAPs. [1] Publications- check FLIP, Enroute Supp, FIH, Charts, IAPs, TCNs, STARs.  Ensure all items are CURRENT!  [2] Weather – formal briefing and DD 175-1 for all flights in IMC.  Review IFR weather mins.  [3] Fuel Planning – fuel log for all cross country flights.  If alternate not required, must have fuel to fly to destination, plus a 10% reserve (max endurance at 10,000’).  If alternate required, must have fuel to fly to destination IAF and then to alternate airfield, plus a 10% reserve.  In no case shall reserve be less than 20 minutes.  Minimum on-deck fuel is 265 lbs per side.  Minimum fuel is advisory only (emergency could occur if any undue delay occur).  Min fuel is normally 265 lbs per side (yellow arc).  If min fuel, consider requesting short vectors to final if IMC or visual landing.  If fuel state requires traffic emergency, declare an emergency due to low fuel and report fuel remaining in minutes.  Emergency fuel typically called when 30 minutes of fuel remain (no set amount, can vary).  [4] Flight Plans: DD-175 info is in FLIP (chap. 4).  Avoid prohibited areas, restricted areas, and MOAs by minimum of 3nm (unless permission to cross is given by controlling agency).  Change of flight plan enroute: done with ATC (use back cover of enroute supp for proper flow- don’t use block headings, only information required).  If complicated, file with FSS (255.4 or charted VHF freq).  On initial call to FSS, identify freq on and VOR close to.  After filing, request weather update.  After FSS enters flight plan, allow ARTCC time to process request.  Close flight plans: Functioning control tower: will close IFR flight plans automatically.  No functioning control tower:  call ATC or FSS and close.  At military fields: verbally or physically with tower or base ops.  [5] NOTAMs: check on internet. 1. FDC (important info) 2. Attention Notices (military info) 3. Civilian D (Distant) aerodromes 4. Civilian L (Local): airport advisory, doesn’t prevent use of runways, found by calling local FSS for field 5.Notices to Airmen (NTAP) 6. Military Summary Reports: info on individual military airfields 7. GPS.  FSS has NOTAMs for all fields within 400 miles of their station.  Local NOTAMs must be requested.  [6] TOLD: Reducing gross weight is easiest way to improve takeoff and climb performance.  On cross country, don’t have tanks filled until determine gross weight limitations.  Accelerate-stop distance must be equal to or less than runway length (factor on wet runways).  One engine inop climb gradient must be 200’/nm or published gradient for departure procedure.  Restrictions increase at high density altitudes and in mountainous terrain.  Accelerate-Go Distance is a factor in bad weather from an airport with a runway end crossing height.  Enroute: one engine inop may be affected by MEAs.  Weight & Balance: full form must be filled out if four or more souls onboard for cross country.  [7] SIDs/DPs/STARs/IAPs: always check to see if airport flying into utilizes any of these procedures. [8] Civilian Airfield Ops: Get weather, NOTAMs, and file flight plan with FSS (1-800-Wx-Brief).  Ensure that the lineman are familiar with T-44 servicing procedures (fill the nacelle tanks before the wing tanks!).  Ensure rudder lock is removed if there is any chance the aircraft will be towed.  If no lineman is present, call “clear right/left” loudly out the window prior to starting each engine.  At civilian fields, preferable to use VHF.  At military fields, preferable to use UHF.  [9] Departure Message: Flight following primarily provided by ATC (also by military destination Base Ops dispatcher).  Departing a civilian field, must ensure actual departure time is passed to FSS serving departure field (very important when arriving at military fields).

NON-PRECISION APPROACH (VOR, TACAN, NDB):  Procedure Turns (PT): Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN, +/- 5 bearings for NDB).  Continue descent to FAF.  Configure and slow to 120K 3 nm prior to the FAF (unless SSE).  Non-precision SSE approach, don’t configure until landing assured.  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

SSE NON-PRECISION APPROACH (VOR, TACAN, NDB): Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist.  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

NO HEADING APPROACH (VOR, TACAN): (FTI 4-48) Troubleshoot heading system, check circuit breakers and the God Box, transfer controls to co-pilot if his system still operational, if his have failed as well: secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat), ask him if he is familiar with how to work the mag compass, declare emergency, squawk 7700, have copilot call out cardinal headings.

Utilize either radar vectors to final or execute a procedure turn.  Compass card is frozen but radial/bearing info from needles is still correct.  1st, determine current radial (cross reference with DME).  2nd, take radial & DME and figure out current position on an IAP or chart.  If required, make a timed turn to the VOR inbound course and fly direct to the station (using the VOR needles & the CDI).  At station passage, make a timed turn to the outbound intercept heading, rotate the course select knob to the Final Approach Course & track outbound using heading info from the copilot.  Reverse course (45/180) using time turns: 15 sec for 45 degrees & 1 minute for 180 degrees.  When CDI starts to center, commence another 15 second turn to final.  To track inbound, either visualize with radials/DME or set CDI and heading bug.  Configure at normal point (3nm from FAF, unless SSE).

ADF NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

The ADF needle shows relative bearing only.  Mentally place the current heading (given by the CP from the mag compass) under the upper lubber line.  Count the number of degrees the head of the single needle deflected from the lubber line to find the relative bearing to the station.  Add (+ to the right) or subtract (- to the left) from the current mag heading to get the magnetic bearing to the station.  Mag bearing from the station found by using the tail of the needle when heading away from the station.  (Exp. If the current heading is 135, and the needle points 45 degrees to the left of the lubber line, subtract (because the needle is left) 45 from 135 to get a magnetic bearing of 090 to the station.  Unless radar vectors are given, request the 45/180 course reversal.  Turn to place the head of the needle on the nose and fly direct to the station.  Utilized known winds to make a wind-corrected heading to the station.  Continue to ask the CP for heading calls and set the heading bug on the corresponding heading to proceed direct.  Compute a heading to intercept the outbound course (the outbound course + 30 degrees [extra 10 seconds]) and determine the seconds required for a timed turn.  Immediately after station passage, commence a timed turn to intercept the outbound course.  Track outbound 1 minute using heading calls from the CP, then start a 45 degree [15 second] turn to the outbound PT heading.  After 1 minute, commence a 180 degree [1 minute] course reversal.  Inbound on the procedure turn, determine my position for situational awareness.  When the needle reaches the 45 degree benchmark, turn inbound so the head of the needle is at the top of the RMI.  Continue to ask the CP for headings.  Configure at the normal point for a non-precision approach.  Passing over the IAF and the FAF, execute the 6Ts (time, turn, time, transition, twist, talk).  If missed approach, generally requires turning to the head of the needle and flying direct back to the station.

RADIO INSTRUMENT MALFUNCTION (VOR, TACAN, NDB): (FTI 4-49) (CDI failure for VOR/TACAN): Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

VOR & TACAN TRACKING & RADIAL INTERCEPTS: To set up NAVAIDS, use TIMSS: Tune, Identify, Monitor, Select (FD SEL & TACAN SEL switches), Set (Needles or CDI).  Inbound Intercept:  Course to Bearing + 30.  Outbound Intercept: Tail to Course + 45.  VOR Station passage: TO/FROM flag changes to FROM.  TACAN Station Passage: minimum DME.

NDB COURSE INTERCEPTS/TRACKING: (RMI only): (FTI 4-19) Inbound: Course to Bearing + 30 = Intercept Bearing. Outbound: Tail to Course + 45 = Intercept Bearing. NDB Station Passage: bearing pointer passes through wingtip position.  In NDB holding, station passage is the first definite move by the bearing pointer through the 45 degree index on the RMI. Radial to Arc – lead by 0.8 nm for 90 degree turn at 150K. Arc to Radial – lead a 90 degree turn by 5 radials at 10 DME at 150K.

ILS/LOC CHARACTERISTICS: (FTI 4-42, AIGT 2-2, 2-17, 8-18, AIM 1-1-6)

ILS: In the U.S., the required components for an ILS approach: [1] glide slope [2] localizer [3] outer marker.  If no outer marker, it can be replaced by DME, radar, another navaid, or a crossing radial.  If VOR 2 is used to identify intermediate fixes and/or the FAF, tune it to the localizer freq no later than the FAF, unless used to define subsequent stepdown fixes or the MAP.  If the glide slope fails, transition to a non-precision localizer approach (SSE, raise the flaps and gear and treat like a non-precision SSE approach (not lowering the flaps or gear until landing assured)).  If the OM (or one of its substitutes) isn’t available, can’t fly the approach.  Intercepting an ILS/LOC approach is either by radar vectors on the published approach procedure.  While being radar vectored, use any available navaid (exp. TACAN, VOR/DME) to aid in position orientation.  The TACAN select switch must be set in “OPER” to receive DME!  The glide slope’s usable range is 10 miles (however, it can be picked up much further out).

Localizer: The localizer’s usable range is at least 18 miles within 10 degrees of centerline (35 degrees from centerline out to 10 miles).  The localizer may be used beyond 18 miles, but only if under ATC radar guidance. If the localizer fails, can’t fly the approach. On a normal front course localizer, the localizer antenna is usually located beyond the departure end of the runway.

Localizer Back Course (LOC BC): On a LOC BC approach, the localizer antenna is located prior to the approach end of the runway, making a LOC BC approach much more sensitive than a normal LOC approach.  False glide slope signals may exist in the area of a LOC BC causing the G/S warning flag to disappear.  Disregard all G/S indications on a LOC BC approach unless a G/S is specified on the IAP.  At FAF, always hack the clock – timing is always a good back up!   Localizer Backcourse – ALWAYS set in the FRONT COURSE on the CDI  (the ‘front course’ refers to the inbound course on the ILS/localizer approach for the OPPOSITE runway.  On the back course approach plate, the published front course is depicted in the feather as an outbound localizer course).  Avoid reverse sensing on a LOC BC by twisting in the front course.  Intercepting the localizer – keep heading changes to 5 degrees or less.  Discontinue the approach when localizer course is unreliable or full scale deflection on CDI on final.

ILS/LOC approach: First portion of ILS approach generally has a large turn (procedure turn or teardrop approach, off a compass locator, or radar vectors to final).  The actual ILS portion begins when joining the final approach fix inbound.  Must be configured prior to reaching glide slope intercept point (or FAF).  At DH, must either go missed approach or continue landing.  DH based on altitude, not position.  Timing is not used on an ILS approach.  However, if glideslope fails, must revert to LOC appraoch (which may use timing).  So, always back up approach by starting clock at glideslope intercept!  If “COURSE” Warning flag is displayed on final approach, initiate a missed approach!  If “GLIDE SLOPE” Warning Flag is displayed on final approach, don’t fly lower than localizer minimums, or (if no localizer mins published) no lower than circling mins.  SSE precision approach (ILS), Flaps – FULL and Gear Down Landing Checklist 1.5 dots below glidepath intercept.  SSE non-precision approach (LOC), Flaps – FULL and Gear Down Landing Checklist when landing assured and in a safe position to land.  When ceiling < 800’ and/or visibility < 2 miles (and landing aircraft is between ILS FAF and airfield) departing aircraft must hold short of the ILS CRITICAL AREA!  When ceiling < 800’ and/or visibility < 2 miles, aircraft can’t hold below 5,000’ between the outer marker and the airfield.  Approaching the final course (on base or arc) perform the ‘LIDS’ check – [1] Localizer freq set in (VOR/ILS selected on FD SEL switch, monitor the ident) [2] Inbound front course set in the CDI [3] DME (tune the TACAN to the localizer associated with the DME freq) [4] Speed and Configuration (slow to 120K and configure when: [1] 1.5 dots of intercepting glide slope on ILS [2] 3 nm prior to FAF on localizer).

Descent – Maintain glide slope intercept altitude (published or assigned) until intercepting the glide slope (identified by a lighting bolt).  On glide slope, crosscheck aircraft altitude with published “Glide slope Altitude at OM/FAF” to ensure established on correct glide slope.  Don’t descend below descent restrictive altitude if CDI indicates full scale!  The glide slope isn’t the primary aid for vertical flight path until the lightening bolt on the IAP.  However, if established on localizer course and within usable range (10 miles) of glide slope, it can be used in descent to glide slope descent altitude.

Glide slope Indicator (GSI): intercept the glide slope as the GSI moves downward from its upper limits.  Slightly before the GSI reaches the center position, coordinate pitch and power to maintain proper rate of descent (normally 600 feet on a 3 degree glide slope, set power at 350ft-lbs at 120K).  On the indicator, the glide slope is the green pointer & the center line is the aircraft).  Make pitch adjustments in 2 degree or less increments.  On the final portion of the approach, make 1 degree pitch corrections and heading corrections of 5 degrees or less.  Decision Height (DH) is determined by the barometric altimeter.  Don’t descend below localizer minimums if the aircraft is more than one dot (half scale) below or two dots (full scale) above the glide slope.  If glideslope is corrected back to within tolerances, descent can be continued to DH.  At decision height, must immediately decide whether to continue approach or go missed approach.

Flight Director: The Pilot’s CDI can show either VOR 1 (including LOC) or TACAN.  The copilot’s CDI shows only VOR 2 (including LOC).  When tuned to localizer, the TO/FROM flag doesn’t operate & the course arrow setting doesn’t affect the deviation bar.  Marker Beacon comes on when the VOR is turned ON (put P to sensitivity to ‘LOW’ and CP to ‘HIGH’ and turn MB audio switches ON).  LOC/ILS sensitivity on CDI = 2.5 degrees for full scale deflection (5 degrees full deflection for VOR & TACAN).  “HEADING” Warning Flag indicates failure of compass system.  “NAV” Warning Flag indicates NAVAID signal is unreliable.  The center of the glideslope scale (the line) represents the aircraft and the green pointer represents the glide slope.  (For exp., if the pointer is above the center line, the aircraft is below the glide slope, and vice-versa).

NCS-31:  ***If need DME from a NAVAID other than the LOC on an ILS/LOC approach: [1st] Enter the DME freq and D-U-D [2nd] Enter the LOC freq and press the USE button twice (USE, USE)!

Marker Beacons: Outer, Middle, and Inner.  MB lights begin to flash when in almost directly over a MB.  The lights are accompanied by a series of aural tones (- - - for Outer [Blue]/ - . - . - for Middle[Amber]/ . . . for Inner[White]).  Proper configuration for MB is P turn on audio for MB 1 and set sensitivity to LOW, CP turn on audio for MB 2 and set sensitivity to HIGH.  MB are not tuned (always transmit on 75 MHz).

Compass Locators: (CLs)  Often located at OM and MM.  Range of at least 15 miles.  At high powered locations, CLs also have TWEB info.  OM CLs transmit first 2 letters of localizer ident.  MM CLs transmit last 2 letters of localizer ident.

SSE LOC APPROACH: Remain clean and at 150K until established on final.  Once on final, slow to min of 120K after normal configuration point but before FAF (leave flaps & gear up).  Remain clean at 120K until landing is assured and in a safe position to land, at which time select Flaps – APPR & accomplish Gear Down Landing Checklist. If on an ILS approach and lose glideslope (GS FLAG), becomes a LOC approach, must clean up aircraft and remain so until landing assured!  Ensure gear down no later than 1 mile from threshold.  If descending on final for a non-precision approach & lose an engine, immediately raise flaps and gear; don’t put flaps and gear down again until landing assured and in safe position to land.

### SSE ILS APPROACH: When 0.5 dots below glide path intercept, FLAPS – FULL & Gear Down Landing Checklist.

ILS/LOC NO HEADING APPROACH: (FTI 4-49)  Troubleshoot heading system, check all circuit breakers and God Box, transfer controls to co-pilot if his system still operational, ask the CP if he is familiar with how the mag compass works, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.  Request radar vectors to final.  If can’t get radar vectors, use a procedure turn (to do a PT, must have a Localizer Outer Marker (LOM) – an NDB aligned to course centerline).  If no radar vectors available: proceed direct to NDB using no heading techniques.  At station passage, time turn to intercept localizer course outbound, rotate course to Final Approach Course and track localizer course outbound.  NDB no heading information used as back up.  When localizer ‘comes alive’ and comes off full scale deflection, time turn to final.  Once established on localizer and tracking inbound, treat as a normal approach.

EMERGENCIES: Review Ground, Takeoff, In-flight, and Landing emergencies in NATOPS!

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, unable to make safe landing, directed to do so by controlling agency, or instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.

*If a precautionary shutdown (chip light), must preload the Starter Assisted Checklist up to #12!  Don’t forget to memorize the Windshear Emergency Checklist (in NATOPS Extreme Weather Chapter): [1] Power – Max Allowable [2] Attitude – 15 degrees nose up [3] Gear – Up [4] Flaps – Maintain current setting.