RI-10S Study Guide (19 Jun 02)

__ PROCURE A TX HIGH ALTITUDE APPROACH PLATE

IFR ENROUTE SUPPLEMENT: Peruse it and know the general contents.

TACAN HIGH ALTITUDE APPROACH: (FTI 4-30, AIGT 8-13)

Before reaching the IAF: recheck weather, review the approach, obtain clearance for the approach, and complete the Approach Checklist.  Calculate descent rates, Attitude Gyro gradients.  Initiate descent when abeam or past IAF with parallel or intercept heading to courseIf high rate of descent needed, perform Emergency decent procedure (PP FLAW)

Emergency Descent:

[1] Power Levers– IDLE

[2] Propellers – FULL FORWARD

[3] Flaps – AS REQ (lowering flaps increases steepness of descent)

[4] Landing Gear – AS REQ

[5] Airspeed – AS REQ (don’t exceed VNE 227KIAS, (-4K per 1,000’ above 15,500’))

[6] Windshield Heat – AS REQ

Observe NATOPS and airspace speed limitations: Max speed in Class C or D within 4nm of primary airport, less than 2,500’ AGL = 200KIAS.  Max speed in Class B is 250KIAS.  Max speed below 10,000 MSL is 250KIAS.

*** TAS [Above 10,000’] = IAS + (FL [in hundreds] /2) or (Add 5K per 1,000 to IAS) (Exp. At FL 200, 175KIAS, TAS = 175 + (200/2) = 275K)

*** Descent gradients: (FTI 4-68)  Descent Gradient = Altitude to lose / Distance to Travel in nm.  (Exp. 15,000’ to lose / 25nm from IAF = 600ft/min (6 degree pitch change)).

*** VSI needed for pitch change = (___ ft/min X ___ nm/min) (Exp. 600ft/min X 4.25nm/min = 2,5550ft/min on VSI).  The 4.25 is computed by dividing TAS (255K in this case) by 60.

Radial Approach: Approach track formed by one or more radials.  If hdg within 90 degrees of approach course, not required to overfly IAF; use normal lead points.  Start descent abeam or past IAF on parallel or intercept heading to approach course.  For DME approaches, crossing arc is considered abeam IAF.  Slow to 120K and configure prior to FAF.

Radial & Arc Combo Approach: Start descent on intercept to arc and abeam or past IAF.  Slow to 120K and configure prior to FAF.

HIGH ALTITUDE CONSIDERATIONS: (NATOPS 7-14)  If doing high altitude approaches, accomplish the full (not abbreviated) CLIMB and APPROACH checklists.

Cruise Climb Speeds:	Sea Level to 10,000’	150K

10K – 20K		130K

20K – 25K		120K

>25K			110K

Watch ITT closely in climb, as altitude increases, must transition to max ITT limits instead of torque.

Cruise Climb ITT limits: 765 C

Cruise ITT Limit: 740C

*As altitude increases, engines are limited by ITT, not torque!

Pressurization: [1] Bleed Air Valves - OPEN [2] Pressurization Switch - PRESS

CRUISE checklist is only performed when extended cruise ops anticipated (> 1 hour).  Not normally utilized below 10,000’.  If CRUISE checklist is accomplished, must do DESCENT checklist.  Approximately 75% N1 (83% N1 for single engine) required to maintain pressurization schedule during descent.

GPS CHARACTERISTICS: (AIM 1-1-33)  U.S. satellite-based radio nav positioning and time transfer system run by the D.O.D.  Two levels of service: [1] Standard Positioning Service (SPS): accurate to 100 meters or less (95%) and 300 meters or less (99.99%). [2] Precise Positioning Service (PPS): more accurate than SPS, limited to authorized U.S. and allied military, gov, and civil users.  GPS receiver measures distance from a satellite through a radio signal, creates a ‘pseudo-range’ (which is subject to isopheric and troposhperic delays and multipath).  Receiver needs at least 4 satellites to yield a 3-D position (lat, long, and alt) and time.  GPS constellation of 24 satellites set up so 5 are observable anywhere on the Earth.  GPS receiver verifies integrity of signals from satellites through receiver autonomous integrity monitoring (RAIM).  RAIM needs at least 5 satellites in view to detect an anomaly, or 4 satellites and a barometric altimeter.  RAIM needs at least 6 satellites in view (or 5 with a baro-aid) to isolate corrupt satellite signal and remove it from nav solution.  Don’t rely on GPS altitude, vertical error can be very large.  To ensure baro-aid available, current altimeter setting must be set into receiver.  Without RAIM, no assurance of accuracy of GPS position.

GPS APPROACH PROCEDURES: (FTI 4-59)

*** Note: T-44 self-taught GPS training program is located in TWG-4 building in course rules computer room (1st floor on right side [if entering from rear]).  Course takes several hours to complete.

Study sources: [1]FMS/GPS computer software (in TWG-4 bldg.) [2]IEC Model 9002M FMS Operator’s Manual (where can I find this pub?) [3]NATOPS [4]AIM [5]AFMAN 11-217 V2.

Navigation Database: All named waypoints identified with 5-letter alpha character name.  Unnamed waypoints, such as a DME fix, have a 5 letter alphanumeric coded name in the database (exp. D234T represents a point on the 234 radial from XYZ vortac at 20DME [the letter T is the 20th letter of the alphabet and indicates 20nm]).  The T-44 GPS uses the Jeppesen NavData Database.

Approaches: 2 types: [1] Stand-Alone: constructed for GPS, absence of other navaids in approach title.  Straight-in minimums correspond to LNAV Minima on RNAV charts [2] Overlay: permit GPS under IFR to fly existing instrument approaches.  2 types: [A] GPS Not in the Title [B] GPS in the Title

Course Sensativity: CDI sensitivity varies with mode of operation.  GPS normally linear (straight line distance from point to point) regardless of distance from waypoint.  [1] Enroute Mode: prior to approach, full scale deflection = 5nm from centerline [2] Terminal Mode: within 30nm of destination airport on charted SID/DP/STAR or between last enroute waypoint and initial waypoint (IAF).  CDI full scale deflection = 1nm from centerline. [3] Approach Mode: sensitivity transitions from 5nm full deflection to 1nm full deflection within 30nm of destination airport.  Must be active to proceed past FAF on non-precision approach.  [4] Final Approach Mode: 2nm to the FAF waypoint, sensitivity transitions to CDI full scale deflection of 0.3nm either side centerline.  [5] Missed Approach Mode: nav to missed approach holding point, sensitivity 1nm of centerline.

RAIM: When GPS integrity doesn’t meet RAIM standards, integrity warning, signals GPS nav info may no longer be reliable (equivalent to ‘off flag’ on HSI.

Approach Restrictions: Instrument Approaches must be: [1]accomplished in accordance with FMS database using current update cycle [2]conducted in FMS Approach Mode.  RAIM must be available at FAF (INTG annunciator extinguished).  Aircraft must have other approved nav equipment.  GPS approaches can be flown without tuning, identifying, or monitoring other navaids (highly discouraged).

Preflight Actions: [1] Review GPS NOTAMS for destination and any alternates.  GPS satellite outages issued under pseudo random noise (PRN), number or satellite vehicle number (SVN).  Accessed using “KGPS” airport identifier.  Important to deselect any affected satellites using SV CONTROL function (on RAIM 2-2 page) of IEC 9002 FMS. [2] NavData NOTAMS: issued every 2 months.  Found at Jeppesen website (http://www.jeppesen.com/onlinepubs/navdatanot.phtml).  Check before flight. [3] If using GPS, aircraft is considered RNAV for flight planning purposes.  Use BE-9L/G in aircraft identification block.  [4] Check GPS ground equipment by start-up and self-test procedures as outlined in NATOPS.  Check currency of database, and predicted RAIM available status for approach plan to fly at ETA.

CRM Procedures: On deck, right-seat pilot enters flight plan into FMS.  In flight, pilot not flying (PNF) changes flight plan data.  PNF verbalizes changes.  Pilot flying (PF) verifies modification.  Both pilots review modified or previously loaded flight plan prior to execution.  Either pilot verbalize need to enter a discontinuity.  PNF will enter discontinuity and adivise PF when entry complete.  Either pilot verbalize need to close up discontinuity.  PNF receive concurrence from PF prior to closing it up.  Either pilot verbalize need to enter or edit a waypoint.  PNF enters the waypoint and executes it after receiving concurrence from PF.  PNF monitors flight progress on LEGS page.  Either pilot verbalizes if annunciator illums.  PNF looks up amplifying information.  In response to CDU APP annunciatior (APPR annunciator on instrument panel), either pilot verbalize approach mode still active, and other pilot acknowledge the call.  After missed approach, PF requests approach reselection.  PNF will acknowledge and comply, then inform PF when approach reselection is complete.  Prior to manually changing CDI sensitivity, PNF obtains concurrence from PF.  PF will verbalize disengaging FMS.  PNF will acknowledge.

Terminal Area Ops – Departure: Load SID/DP (from nav database).  If can’t be retreived, may not use RNAV to fly prior to SID/DP termination point.

Enroute Operations: Use of Predictive Integrity.  Check RAIM availability for planned approach.  If check inicates integrity may not be available, develop alternate plane for landing or proceed to alternate.

Prior to Descent: Compare approach retrieved from GPS nav database to GPS published approach on IAP.  If differences arise, the published approach chart, supplemented by NOTAMs, takes precedence.  Develop a back-up plan.  Consider the possibility of equipment failure past the FAF.  Load the STAR (if applicable).  If can’t be retreived, can’t use RNAV to fly procedure.

Terminal Area Ops – Arrival: Monitor all ground-based navaids since GPS approaches are flown point-to-point.  Bearing pointer on HSI and DME will be to next waypoint, not necessarily the field.  Be prepared to use traditional navaids.  Be wary of having head down in cockpit.

Approach Procedures: Reference NATOPS and IEC 9002M FMS Operator’s Manual.  Inbound Course: course displayed on FMS between FAF and MAP may be slightly different than printed on approach plate; should not affect approach performance.  GPS Integrity Warning Prior to FAF: don’t descent to MDA.  Instead, proceed to MAP via FAF, perform missed approach, and notify ATC.  Alternatively, may continue approach provided backup is available.  Stepdown Waypoints: nav database doesn’t provide stepdown waypoints after FAF.  In all cases, next waypoint is MAP.  Determine stepdown fixes and VDP by refering to GPS distance from MAP as depicted on approach plate.  GPS Integrity Warning After FAF: very serious, immediate action required.  Transition to back-up approach if available or proceed to MAP along FAC and execute missed approach.

Performing Missed Approach Procedures: MAP labeled with named waypoint.  Select Missed Approach Mode manually by using LEGS screen.  Clear MAP discontinuity by pressing Go Around Button on power lever within 2nm of FAF.  If Go Around Button used, EXEC key still needs to be pressed to execute missed approach.  Performing the missed approach: If initiated prior to MAP, proceed to MAP along FAC.  If missed approach includes a turn, don’t begin turn prior to MAP.  Fully review procedure before flying it!  Climb gradient: min of 200 ft/min unless higher published.

TACAN APPROACH (WITH PROCEDURE TURN):  Procedure Turns (PT): 3 acceptable methods: [1] 45/180 Technique [2] Holding Technique [3] Teardrop Technique

[1] 45/180 Technique:  Reverse course to intercept inbound course.  Complete within distance listed on profile view (normally 10nm) from the IAF.  Max speed = 250K.  When approach can be made from a published holding fix, ATC DOESN’T expect a PT approach.  Depicted by a barb on the profile view indicating direction of turn.  Before reaching IAF, slow to 150K.  Turn in shortest direction to intercept, parallel, or teardrop outbound course.  If heading within 90 deg of outbound course, use normal lead points.  Begin timing for teardrop or parallel when outbound abeam fix.  If can’t determine abeam, start timing at completion of turn.  Begin descent when outbound abeam fix.  Descend from PT altitude when established on inbound segment (when +/- 5 radials for VOR & TACAN).  Continue descent to FAF.  Configure and slow to 120K prior to the FAF (unless SSE).  The 45/180 maneuver will always be used in the T-44 for training.  From IAF to 45 turn point, time 1 minute.  From 45 turn point to 180 turn point, time 1 minute.  From 180 turn point to inbound course, back up intercept with 1 minute of timing.  When NAVAID is on the field and no FAF is listed, consider interception of final approach course as the Final Approach Point (FAP).

Procedure Turn is not flown for “SNERT” – Straight-in approach, No PT course, Established in holding, Timed approaches from a holding fix.

[2] Holding Technique:  If heading within 90 degrees of outbound PT Course, use normal lead points (includes all of teardrop entry and most of parallel entry).  If use teardrop entry, course must be within 30 degrees of PT outbound course.  If intercept the PT outbound course, maintain course for remainder of outbound leg, then turn toward the maneuvering side to reverse course.

Timing: begin timing outbound abeam procedure turn fix.  If can’t determine abeam, start timing at completion of outbound turn.  Stay within the remain within distance.  At the completion of the outbound leg, turn to intercept the procedure turn course inbound.  A 1 ½ minute outbound leg normally is sufficient (2 minutes or more can be used in instances such as an approach with no FAF or an approach with a large descent on the inbound course).

Descent: Don’t descend form fix altitude until abeam PT fix outbound.  Don’t descend from completion turn altitude until established on inbound segment.

LOW ALTITUDE PROCEDURES: Before IAF, recheck weather (ATIS), brief IAP, obtain clearance for approach, complete Approach Checklist.  Cross IAF at 150K.  Maintain last assigned altitude until on published segment of IAP.  Clearance for approach ISN’T clearance for holding, and vice-versa.   Slow to 120K when configuring and maintain until landing.  Non-precision configuration point is 3 miles prior to FAF.  (1.5 dots below glideslope for ILS).  On radar approaches, configure on dogleg prior to contact with final controller.  Landing checklist must be complete prior to FAF/Glideslope intercept, unless holding the gear for SSE.  Gear horn can’t be silenced after the FAF/Glideslope. With field in sight and in safe position to land, review landing checklist complete & slow to cross threshold at min of 110K.  Inside the FAF, minimum obstacle clearance at MDA is 200’.  Inside the FAF, at least one nav receiver must be tuned to facility providing final approach guidance.  If only one VOR operable, CAN’T be re-tuned inside FAF to identify stepdown fixes and/or MAP.  Accomplish the 6 T’s passing any ‘fix.’  Can’t use the middle marker as the sole means of identifying the MAP (if it is, then I can’t shoot the approach).  Max descent gradient for straight-in approach is 800ft/min with a GS of 120K.  Calculating VDP: HAT/300 = VDP in nm from the end of the runway (add that to the distance of the NAVAID from the end of the runway).

Arc from Radial = 0.8nm lead for 90 degree turn at 150K.  Radial from arc = 5 radials for 90 degree turn at 150K at 10DME.

TACAN NO HEADING APPROACH: (FTI 4-48) Troubleshoot heading system, check the circuit breakers and the God Box, transfer controls to co-pilot if his system still operational, secure “Big 4” (electric heater, air conditioner, windshield wiper, windshield heat) which influence mag compass, declare emergency by advising controller, squawk 7700, have copilot call out cardinal headings.

Utilize either radar vectors to final or execute a procedure turn.  Compass card is frozen but radial/bearing info from needles is still correct.  1st, determine current radial (cross reference with DME).  2nd, take radial & DME and figure out current position on an IAP or chart.  If required, make a timed turn to the VOR inbound course and fly direct to the station (using the VOR needles & the CDI).  At station passage, make a timed turn to the outbound intercept heading, rotate the course select knob to the Final Approach Course & track outbound using heading info from the copilot.  Reverse course (45/180) using time turns: 15 sec for 45 degrees & 1 minute for 180 degrees.  When CDI starts to center, commence another 15 second turn to final.  To track inbound, either visualize with radials/DME or set CDI and heading bug.  Configure at normal point.

RADIO INSTRUMENT MALFUNCTION: (FTI 4-49) (CDI failure for VOR/TACAN): Use the RMI needles.  Needle always points to station.  Head always falls & tail always rises.  Inbound intercept: put heading bug on inbound course.  Head will fall to heading bug.  When bug and head are aligned, turn to put both under upper lubber line.  Maintain by keeping pointer centered on heading bug.  Outbound intercept: put heading bug on outbound course.  Tail will rise to heading bug.  When bug and tail are aligned, turn to put both under upper lubber line.  Maintain by keeping tail centered on heading bug.

PRESSURIZATION SYSTEM & EMERGENCIES:

Procedures for an altitude warning light:[1] Descend [2] Restore cabin pressurization [3] Check pressurization and/or don O2 mask.

Loss of cabin pressurization (NATOPS 15-11):

If gradual pressurization loss:

[1] Cabin altitude – Checked (P)

[2] Pressurization controller – Checked (CP)

[3] Bleed Air – Checked (CP)

[4] Press dump test switch – TEST (CP)  (hold for 15 sec)

If test switch restores press, may be necessary to hold switch in TEST until cabin profile at 10,000’ or less.  If press is regained through test switch, pull the PRESS CONTROL CB to maintain pressurization.

If unable to restore pressurization:

[5] Oxygen mask/microphone switch (100%) – AS REQ (P,CP,OBS)

[6] Descend – AS REQ (P)

On descent when cabin alt matches press alt, ensure PRESS CONTROL CB is reset to preclude landing

pressurized.

Explosive decompression: occurs in less than 1 second, causes fog, if high alt – rush of air from lungs, dazed sensation, gas pains, hypoxia if oxygen not used immediately.

[1] Oxygen mask/ MIC switches (100%) – AS REQ

[2] Descend – AS REQ

Supplemental oxygen is required when at cabin altitudes above 10,000’.  The oxygen system capacity is 1850 (+/-50) psig for one 49 cubic foot cylinder.  Between 17,000’ to 20,000’ the diluter demand regulator should supply 100% oxygen while in the normal position. The oxygen mask should be stored in the 100% position.  The min O2 requirement for local flights is 1000psig.  For cross-country flights: 1500psig.

Smoke and fume elimination (NATOPS 15-5): Locate, isolate, and extinguish fire or source of smoke and fumes prior to initiating smoke removal procedures.  Isolate the engine as a possible source by closing the bleed air valves.  Prior to depressurizing, consider MEA and crew O2 requirements.

Immediate smoke removal:

[1] Oxygen masks/MIC switches (100%) – AS REQ

[2] Pressurization – DUMP

After depressurizing, storm windows may be opened (may draw smoke into flight station).  Consider emergency descent and immediate landing.  Nauseating fumes from desalination solution can enter cockpit via bleed air for air-conditioning/pressurization and is often detected immediately after takeoff.  Providing no secondary indications exist, closing the bleed air valves will significantly reduce the fumes.

MISSED APPROACH: (FTI 4-56) For non-precision straight-in, MAP is on final approach course no further from FAF than runway threshold.  Precision MAP is the DH (a climbing dashed line). Obstacle clearance: min of 200ft/nm unless otherwise shown on IAP.  Climbout instructions supercede printed missed approach instructions.  Delay turns until at departure end of runway and at least 400’ above field elevation.  If ATC tells to turn, supercedes departure end/400’ rule!  Practice missed approach (in VMC) at Navy Corpus requires overflying field no higher than 500’ until over departure end, then climb to assigned altitude and switch to departure.  Local area coded climbout instructions in ‘blue brains.’  Perform Missed Approach if runway environment not in sight, or unable to make safe landing, or directed to do so by controlling agency.  Execute Missed Approach if arrive at MAP or DH and instructor has not called field in sight.

# MISSED APPROACH PROCEDURES: WAVEOFF (SSE & DUAL ENGINE) (NATOPS 16-1, FTI 1-25 & 4-57)

[1] Power – AS REQ (Max allowable on operating engine)

[2] Flaps – APPR (unless already up)

[3] Gear – UP (+ rate of climb & no possibility of touchdown)

[4] Flaps – UP

[5] Props – 1900RPM (when CP calls “Gear Up”)

Insure minimum climb gradient of 200ft/nm (or min climb gradient on IAP).  Advise ATC going missed approach and request clearance.  Check min safe altitudes in charts and IAP.