T-1 Dash-1 Numbers to Know

| What N2 RPM does the starter drop out? | 35-45% |
| --- | --- |
| Range limitation for engine sync | 1.5% |
| Minimum oil pressure for engine start | 40 psi |
| How many seconds for normal engine light off | Within 10 of placing to idle |
| Max ITT for normal engine start | 550C |
| Normal charge (psi at 70F) in fire bottles | 600 psi |
| Engine fire bottle cubic inch capacity | 86 |
| Temperature range for fuel system | -40C to 50C |
| Total pressure refueling capacity (gal) | 771 |
| Total gravity refueling capacity (gal) | 739 |
| Fuel feed chamber capacity (gal) | 20 |
| What pressure in the fuel feed lines turns on boost pump automatically? | 5.0  0.5 |
| FUEL PRESS LO illuminates at what psi | 5.0  0.5 |
| F FLTR BYPASS illuminates at what psi | 2.3  0.2 |
| WING TNK OV PRESS illuminates at what psi | 3.5 |
| F XFR PRESS LO caution comes on at what psi | Below 5.0  0.5 |
| FUEL LEVEL LO illuminates at approx. how many gallons of usable fuel | 32 (200 lbs) |
| FUEL FEED illuminates at approx. how many gallons of usable fuel | 11 (70 lbs) |
| When do the sniffle valves actuate | (-0.1 to -0.3) to 5 psi |
| Battery volts and ampere-hour rating | 24 volt 40 amp/hr |
| GPU output limits | 28 – 28.5 1000-1500 amp |
| Standby battery output (volts) and time | 24 volt/30 min |
| Output of AC inverters (AC volts and hertz) | 155/26 volt 400 hertz |
| Hydraulic reservoir bleed air pressure | 15 psi |
| Capacity of hydraulic reservoir | 1.1 gal |
| Normal operating range of hydraulic system | 1350-1550 |
| Normal operating pressure of hydraulic system | 1500 |
| Hydraulic pump output (gal/min) | 0 to 3.9 |
| Hydraulic pressure relieve valve activation psi | 1650-1850 |
| HYD PUMP PRESS LO illuminates at what psi | 750 |
| HYD LEVEL LO illuminates when | 0.6 gal |
| Landing gear normal operation time | 7 sec |
| Landing gear emergency operation time | 30 sec |
| What taxi speed does anti-skid become operative | 14 kts |
| When does anti-skid become operative when landing (speed and time) | 37 kts / 2.5 sec |
| Number of emergency brake applications | 7-10 / 7-9 |
| FLAP ASYM illuminates when | Greater than 7 (main flaps) |
| Speed brake deflection in degrees | 36 |
| Speed brake extension/retraction times | 0.6 gnd / 2.6 air |
| Elevator deflection | +25 / -12 |
| Rudder deflection | 30 |
| Max nosewheel steering with rudder pedals? Differential braking? | 25 / 45 |
| Stabilizer full travel time normal operation | 14 sec |
| Stabilizer full travel time emergency operation | 45 sec |
| Roll trim deflection from neutral | 25 |
| Pitch trim deflection | +12 / -1.5 |
| Autopilot max roll angle | 27 |
| Autopilot max roll rate | 4/sec |
| Autopilot max pitch rate with pitch wheel use | 3/sec |
| Autopilot max pitch rate and G limit | 0.15 G’s |
| Autopilot max navigation capture intercept angle | 90 |
| Autopilot bank angle limit while tracking a LOC course | 15 |
| Autopilot approach mode max crab angle | 30 |
| Autopilot max bank angle when tracking a VOR, TCN, or FMS | 27 |
| What are the autopilot normal and high speeds for climb | 220/250 |
| What are the autopilot normal and high speeds for decent | 250/310 |
| How long is the standby attitude indicator accurate after power loss and to what accuracy? | 9 min / 6 |
| What temp does aux cool shut off | 20C |
| ACU maintains cabin temp of | 60F - 90F |
| Max temp and psi output from engine at TRT | 600/150 psi |
| What is normal pressure after the regulator shutoff valves in air conditioning | 30 psi |
| Cooling turbine air output temp | 39 |
| AIR COND FAIL illuminates | 400/53 psi |
| DEFON AIR OV HT illuminates | 200 F |
| CABIN AIR OV HT illuminates | 235 F |
| BL AIR DCT FAIL illuminates | 350 F/3.5 psi |
| L or R ENG switch position on the environ panel air output rate | 9-10 lbs/min |
| HIGH switch on the environ panel air output rate | 10-23 lbs/min |
| Max cabin altitude during normal operations | 125001500 |
| Cabin pressurization when throttles are advanced to T/O position on the ground | 80-120 |
| CABIN PRESS LO illuminates | 9500500 |
| CABIN PRESS HI illuminates | 9.10.1 |
| Wing anti-ice bleed air pressure | 38-44 psi |
| WG ICE TEMP LO illuminates | < 140F |
| Normal operating temp range of stabilizer anti ice | 260-280F |
| STB ANTI ICE FAIL illuminates | 310F |
| Engine anti ice max ground/air operating temp | 10C |
| ENG ICE TEMP LO illuminates | 120F |
| Rate of windshield wiper operation on a wet windshield | 10-13 cycles/10 sec |
| Max range of VOR/DME reception | 512 |
| Max range of TACAN | 800 |
| Max altimeter deviation from field elevation/ from other altimeter | 75 / 150 |
| Tilt range of wx radar ant | 14 |
| PASS OXY ON illuminates | 503 psi |
| Capacity of high pressure oxy cylinder | 77 cu Ft @ 1850 psi |
| How far open is the knife valve required to be for preflight inspection | ¼ inch |
| Battery voltage for preflight inspection | 22 volts |
| AC voltage for preflight inspection | 115/26 volts |
| Nose gear strut extension for preflight inspection | Approx. 3.5 in |
| Main gear strut extension for preflight inspection | 1.1 min |
| Ice may form on compressor blade at what RAT | 8C |
| Minimum N2 for anti-ice operation | 70% |
| Max RPM for ground operations in a congested area | 70% N2 |
| Minimum voltage for battery start | 22 volts |
| Minimum voltage for a GPU start | 28 volts |
| Max tire speed | 165 kts |
| Maximum N1 RPM for turning on anti-ice | 90% N1 |
| Minimum altitude to select 30 flaps | 300 ft AGL |
| Temperature decrease of ____ degrees will reduce oxygen pressure _____% | 100F/20% |
| How long does the Emergency Escape Breathing Device operate | 15 min minimum |
| Maximum fuel imbalance for: T/O, LND, and T&G | 100/300/300 |
| Recommended holding airpseed | 180 |
| Five point harness is required below what altitude | 10,000 |
| Single generator operating limits | 280 above 32,500 400 below 32,500 |
| Do not attempt to start second engine until generator is below | 150 amps |
| Above what altitude is flap & landing gear extension prohibited | 20,000 |
| Max airspeed for emergency gear extension | 150 |
| G limit flaps up | -1 to 3.2 |
| G limit flaps extended | -0 to 2 |
| Max ramp weight | 16,300 |
| Max takeoff weight | 16,100 |
| Max landing weight | 15,700 |
| Max zero fuel weight | 13,000 |
| Cabin floor loading limit | 100 per sq/ft |
| Max decent rate at touchdown | 600 ft/min |
| Fuel crossfeed is limited to wings level flt when less that ______ lbs in wing tank supplying fuel | 600 |
| Aircraft maximum operating altitude | 41,000 |
| Yaw damper operation required above what altitude | 28,000 |
| Do not use autopilot below what altitude | 200 AGL |
| Max brake temp do not reach the brake surface for how long | 15 min |
| Wheel thermal fuses may release at brake energy above _____ million foot/lbs. | 8.1 |
| Brake damage and failure may occur above absorption of _____ million foot/lbs | 14.8 |
| Brake efforts between _____ & _____ million ft/lbs should be regarded as hot brakes | 8.1 / 14.8 |

| Wake turbulence avoidance times behind: large, heavy, and 757, T-1 | 2 min/ 3 min/ 2.5 min/ 6000 ft |
| --- | --- |
| AOA of approximately _____ to _____ will result in optimum stall recovery | .8-.85 |
| Target penetration speed for turbulence and thunderstorm penetration | 200 kts or .58 mach (whichever is lower) |