| T-1A BOLDFACE EMERGENCY PROCEDURES/OPERATING LIMITS |  |
| --- | --- |
| 1. EMERGENCY PROCEDURES |  |
| ENGINE FIRE DURING GROUND OPERATIONS |  |
|  | THROTTLES – CUTOFF |
| ENGINE FIRE DURING FLIGHT |  |
|  | THROTTLE – IDLE IF ENGINE FIRE SWITCH LIGHT REMAINS ILLUMINATED THROTTLE – CUTOFF ENGINE FIRE SWITCH – PUSH BOTTLE ARMED SWITCH – PUSH |
| SMOKE OR FUMES IN AIRCRAFT |  |
|  | OXYGEN MASKS – DON/100% |
| PITCH TRIM/ROLL TRIM/RUDDER TRIM RUNAWAY OR FAILURE |  |
|  | TRIM INT & A/P DISENGAGE SWITCH – PUSH AND HOLD |
| LOW CABIN PRESSURE |  |
|  | OXYGEN MASKS – DON 100% |
| 19 AF FORM 1, JAN 95 REPLACES AETC FORM 807, NOV 93, WHICH IS OBSOLETE. |  |

| 2. T-1A OPERATING LIMITS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENGINE OPERATING LIMITS TABLE |  |  |  |  |  |  |
| THROTTLE SETTING | TIME LIMIT (MIN) | MAX ITT | % N2 | %N1 | OIL PRESSURE (PSI) | OIL TEMPERATURE (C) |
| TAKEOFF | 5 min | 700 | 96 | 104 | 60-83 | 10-121 |
| MAX CONTINUOUS | Cont | 680 | 96 | 104 | 60-83 | 10-121 |
| IDLE | Cont | 580 | 52-54 |  | 40 (min) | -40-121 |
| STARTING |  | 550-600 (4 sec) |  |  | 83 – 95 for 90 sec | -40 (min) |
|  |  | 600-700 (2 sec) |  |  |  |  |
|  |  | 700-720 (1 sec) |  |  |  |  |
| TRANSIENT |  | 720 (2 sec) | 96 | 106 | 95 for 90 sec | 135 for 15 min (max) |
| Normal oil pressure is 60 to 83 psi at N2 > 60%. Under cold starting procedures, oil pressure between 83 to 95 psi is limited to 90 seconds. Normal hydraulic pressure 1350 to 1550 psi. Normal nitrogen pressure 1350 to 1650 psi. Normal oxygen pressure 1600 to 1850 psi. Minimum battery voltage for battery start is 22 volts DC. Maximum generator load prior to starting second engine is 150 amps. Motoring of the starter is limited to: First, 30 seconds then 5 minutes cooling. Second, 30 seconds then 15 minutes cooling. Third, 30 seconds then 60 minutes cooling. Maximum limiting airspeeds: Extend flaps 10 degrees 200 KIAS Extend flaps > 10 degrees 170 KIAS Flaps extended > 10 degrees 165 KIAS Landing Lights 200 KIAS Wipers 200 KIAS Aircraft is limited to 330 KIAS or .78 MACH, whichever is less Minimum/maximum fuel temperature -40/50 C Emergency/minimum fuel Dual 300/500 Team 300/700 Maximum fuel imbalance: Takeoff 100 lbs. Flight 300 lbs. Landing 300 lbs. Maximum tailwind 10 knots. Maximum crosswind: (Dry) 25 knots (Wet) 15 knots (Icy) 10 knots |  |  |  |  |  |  |
| 19 AF FORM 1, JAN 95 (REVERSE) |  |  |  |  |  |  |